3GPP TSG-RAN2 Meeting #121	R2-23xxxxx

Athens, Greece, 27 February – 3 March 2023

Title:	DRAFT LS on the use of PEI during an emergency PDU session

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	Ericsson

To:	3GPP SA WG2

Cc:	3GPP CT WG1

Contact Person:

#### Name:	Martin van der Zee

E-mail Address:	martin.van.der.zee@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 discussed the use of PEI during an emergency PDU session, and RAN2 observed that disabling UE-ID based PEI for CM-IDLE paging (Paging on NG) and CM-CONNECTED paging (RAN paging on Xn/F1) during an emergency PDU session is not supported:

- The UE-ID based PEI capability (pei-SubgroupingSupportBandList-r17) is signalled in the PAGING message (UERadioPagingInformation) either from AMF or from the last serving gNB. Based on the UE capability and gNB’s configuration (PEI-Config-r17) UE-ID based PEI is used even when the UE has an emergency PDU session.

- The latency introduced by UE-ID based PEI (similar as for CN-assigned PEI) depends on the configured time offset between PEI and the following Paging Occasion (PO) e.g. 50 ms. The RAN2 specifications do not have any special treatment for PEI during emergency call considering the latency difference is negligible comparing to the paging cycle length.

This means that when the UE has an emergency PDU session the CN-assigned PEI (PEIPS Assistance Information) is not used during an emergency PDU session, but UE-ID based is used during an emergency PDU session.

2. Actions:

To SA2:

ACTION: 	RAN2 kindly asks SA2 to take this information into account and make any corrections if deemed necessary (in Rel-17 or a later release).

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #121bis-e	17 - 26 April 2023		E-meeting

TSG RAN2 Meeting #122		22 - 26 May 2023		Republic of Korea