3GPP TSG-RAN WG2 Meeting #121bis electronic	R2-230wxyz

Online, April 17-26, 2023

Agenda Item:	7.9.3

Source:	NEC (Email Discussion Rapporteur)

Title:      	Summary of [AT121bis-e][432]Candidate solutions for lossless delivery

Document for:	Discussion, Decision

# Introduction

This document is to provide a summary of the email discussion [AT121bis-e][432]Candidate solutions for lossless delivery:

- [AT121bis-e][432][Relay] Candidate solutions for lossless delivery (NEC)

Scope: Evaluate candidate solutions for lossless delivery (DL/UL) in U2N service continuity.  Intention is to capture solutions for down-selection next meeting.

Intended outcome: Report to CB session

Deadline: Monday 2023-04-24 2359 UTC

## Contact information

| Company | Name (Email) |
| --- | --- |

# Uplink lossless data delivery for path switch

## Background

In case of indirect-to-direct or indirect-to-indirect inter-gNB path switch for UE-to-Network relay, it is assumed that the gNB holding the PDCP entity for the radio bearers of the Remote UE changes after path switch. Then this scenario is like the inter-gNB handover for normal UEs as in legacy handover procedure, where PDCP is re-established.

In legacy handover, for RLC AM based radio bearer, if the target gNB receives the receiving status of UL PDCP in SN Status Transfer, the target gNB may use it in a PDCP Status Report sent to the UE. This will help the UE to determine if a PDCP packets should be retransmitted to the target gNB after handover.

As specified by PDCP specification (i.e.,TS38.323), the current UL PDCP retransmission determines the boundary with reference to the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers.

During direct-to-indirect or indirect-to-indirect inter-gNB path switch, from Remote UE point of view, at PDCP layer, it may skip the packet that has already been successfully transmitted (i.e. acknowledged at PC5 RLC by Relay UE at the first hop) during its decision on the packet boundary for retransmission. In addition, the PDCP entity of Remote UE may discard the packet that has already been successfully transmitted (i.e. acknowledged at PC5 RLC by Relay UE at the first hop) when the discard timer expires. This means that during this type of path switch, even though the target gNB receives the receiving status of UL PDCP in SN Status Transfer message and use it to send the accurate PDCP status report to the Remote UE, the Remote UE may not be able to do retransmission for the missing UL packets (i.e. acknowledged at PC5 RLC by Relay UE at the first hop, but did not reach the gNB at the second hop).

## Candidate solutions description for UL

### Solution- U1: Relay UE delays its RLC feedback to Remote UE

Relay UE can maintain the transmission status between the received PC5 RLC packets and the outgoing Uu RLC packets. When providing RLC status report to Remote UE, the Relay UE only provides the positive feedback to Remote UE on the PC5 RLC packets, of which the corresponding Uu RLC packets have been successfully transmitted to source gNB via Uu RLC (which means acknowledgements have been received for these packets over Uu from source gNB).

Figure 1: Relay UE delays its ACK to Remote UE

As in legacy handling, the Remote UE will not indicate its successful transmission of such packets (ACKed at PC5 RLC, not ACKed at Uu RLC) to PDCP layer, since the positive acknowledgement for these packets is postponed by Relay UE.

As shown in Figure 1, the RLC packet K+3 and K+4 will not be acknowledged by Relay UE to Remote UE since they are not acknowledged by source gNB to the Relay UE.

Evaluation

This solution is transparent to the Remote UE and the gNB but will require changes at the Relay UE. However, the Remote UE may retransmit the unacknowledged packets, which were actually received by Relay UE.

### Solution- U2: Remote UE’s PDCP retransmission based on remaining packets in the buffer

Upon PDCP re-establishment during inter-gNB path switch, the Remote UE transmits/retransmits all PDUs that are in the transmit buffer, even though some packets have already been ACKed by lower layers.

Evaluation

Since the Remote UE can retransmit all of the remaining packets within its buffer, it may result in some redundant retransmissions (as the PDCP packets may have already been received at the target gNB.

### Solution- U3: Remote UE’s PDCP retransmission based on DL PDCP Status Report from target gNB

Alternatively, the Remote UE can determine the PDCP SDUs for retransmission to the target gNB following the PDCP Status Report sent from target gNB to the Remote UE after path switch. Remote UE can use this PDCP Status Report to determine the boundary for PDCP SDU retransmission.

Figure 2: Enhancing Remote UE’s PDCP retransmission

For example, as shown in Figure-2, if we apply the legacy PDCP retransmission handling (i.e., based on the lower layer confirmation) during PDCP re-establishment for inter-gNB path switch, at Remote UE, the first non-confirmed PDCP SDU is N+6, which means N, N+1, N+2, N+3, N+4 and N+5 were confirmed by lower layer due to its successful transmission at PC5 (i.e., at the first hop). And then legacy handling will retransmit packet N+6 and the packets that follow it. However, in this solution, assuming that target gNB sends a PDCP SR to Remote UE indicating that N+1, N+3, N+4 and N+6 are missing, this solution will allow the Remote UE to retransmit N+1, N+3, N+4, N+6 and the packets that follow N+6.

This option can reduce the data loss during path switch since more PDCP SDUs can be retransmitted from Remote UE to the target gNB at PDCP layer. This will ensure that there will be no UL packet loss upon path switch from indirect to indirect/direct, as long as the packet has not been discarded already due to the expiry of the discard timer.

Evaluation

This solution can prevent unnecessary retransmission and present minimum specification change.

### Solution-U4: Enhancing RLC status report to Remote UE

As a option, additional indication can be added by Relay UE within the RLC status report when providing that report to Remote UE for the RLC packets. The additional information can be the status of the acknowledgement at the second hop (i.e., Uu). When receiving the RLC status report including this additional information, Remote UE’s RLC entity calculates the corresponding PDCP SDU packet based on the RLC status report and feedback to PDCP entity to indicate the transmission status of the PDCP packets.

In this case, PDCP entity is fully aware of the transmission status of the PDCP SDUs at both first hop and second hop. Remote UE can decide the retransmission boundary for the PDCP SDUs based on both transmission status as received from its RLC layer and the PDCP Status Report he may receive from the target gNB. With this option, Remote UE’s PDCP entity will potentially retransmit missing PDCP packets at the second hop, if the PDCP packet was not discarded due to expiration of PDCP discard timer.

Evaluation

This solution requires the RLC specification change to enhance the RLC status report.

## Dicussion

### Question 1: Do companies agree that the decription and evaluation of solution-U1 is accurate for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 2: Do companies agree that solution-U1 is a valid solution for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 3: Do companies agree that the decription and evaluation of solution-U2 is accurate for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 4: Do companies agree that solution-U2 is a valid solution for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 5: Do companies agree that the decription and evaluation of solution-U3 is accurate for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 6: Do companies agree that solution-U3 is a valid solution for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 7: Do companies agree that the decription and evaluation of solution-U4 is accurate for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 8: Do companies agree that solution-U4 is a valid solution for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 9: Do companies see any additional solution(s) for Uplink lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 10: Do companies agree to take solution-U1/U2/U3/U4 as the candidate solutions for Uplink lossless data delivery for path switch for downselection next meeting?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

TBD: Rapporteur summary:

TBD: Proposal-X:

# Downlink lossless data delivery for path switch

## Background

In legacy handover, for RLC AM based radio bearer, from source gNB perspective, the unacknowledged PDCP SDUs are forwarded to target gNB during handover. In addition, the PDCP Status Report (by UE report) helps target gNB to skip the PDCP SDUs that are received by the UE but source gNB has not received the acknowledgement from the UE.

If we assume the symmetric operation of PDCP entity be adopted by gNB in practical implementation, logically the similar issue, as discussed for UL data loss, should be applicable to DL also. The only thing different is that the sender of the data for retransmission is changed from source gNB to target gNB. In addition, target gNB determines the retransmission boundary for PDCP packets PDCP Status Report reported by the UE and the packets forwarded from source gNB.

For the packets that was acknowledged at Uu RLC by Relay UE at the first hop, but did not reach the Remote UE at the second hop (i.e. PC5), the source gNB may discard them when the discard timers expire, then source gNB has no chance to forward these packets to target gNB for retransmission, which will lead to data loss at downlink data transmission.

Following the legacy spec, for i2d/i2i case, the DL data has not been acknowledged by the Relay RLC may be forwarded, but the data acknowledged by Relay UE but lost in the source PC5 link will not be forwarded to the target gNB, therefore it cannot be re-transmitted from the target gNB to the UE, no matter the PDCP status report is configured to be sent by the UE or not.

In intra-gNB path switch (Rel-17 scenario), the network may be able to configure a long enough PDCP discard timer to hold the concerned PDCP packets, or use other private mechanism to keep the packets at the gNB. Then the gNB can perform DL packet retransmission when it is not acknowledged by PDCP status report from the UE later on.

For the inter-gNB scenario (Rel-18 scenario), following the same example as described above, it may require the target gNB to fetch the missing PDCP packets from source gNB based on the UE PDCP status report after path switch, which requires the source gNB to keep the PDCP packets after the completion of path switch.  This is an unusual handling and may be extremely difficult within the multi-vendor deployment scenarios.

This is why network implementation (i.e., Rel-17 mechanism) cannot handle Rel-18 scenario.

## Candidate solutions description for DL

### Solution-D1: Relay UE delays its RLC feedback to source gNB

Relay UE can maintain the transmission status between the received Uu RLC packets and the outgoing PC5 RLC packets. When providing RLC status report to source gNB, the Relay UE only provides the positive feedback to source gNB on the Uu RLC packets, of which the corresponding PC5 RLC packets have been successfully transmitted to Remote UE via PC5 RLC, which means acknowledgements have been received for these packets over PC5 from Remote UE. Base station’s PDCP/RLC entity operation is not specified by 3GPP, but we assume the symmetric operation of PDCP/RLC entity corresponding to UE side is adopted by Base Station. As in legacy symmetric RLC operation, the source gNB’ RLC does not indicate its successful transmission of such packets (ACKed at Uu RLC, not ACKed at PC5 RLC) to its PDCP layer, since the positive acknowledgement for these packets is postponed by Relay UE.

Evaluation

This solution is transparent to the Remote UE and the gNB but will require changes at the Relay UE. However, the source gNB may retransmit the unacknowledged packets, which were actually received by Relay UE.

### Solution-D2: Relay UE indicates the packet transmission status to source gNB

As described by R2-2302859, Relay UE indicates the packet transmission status to source gNB, in order for source gNB to better determine which packet(s) should be forwarded to the target gNB for retransmission during path switch.

There are following two options for such indication:

Option 1: a simple indication that there is received data in the Relay UE from the source gNB, but not yet delivered to the Remote UE successfully. Based on this indication, the source gNB can forward all the buffered PDCP PDUs (acknowledged or non-acknowledged from the lower layers) to the target gNB.

Option 2: the indication includes further information, e.g., the number of TBs that is received from the gNB, but not delivered to the Remote UE successfully yet, or the list of RLC SNs or the earliest RLC SN that the Relay UE received from the gNB, but not delivered to the Remote UE successfully yet. Based on the indication, the source gNB can identify which data has been acknowledged from the lower layers but still not delivered to the Remote UE successfully so that those data should be forwarded to the target gNB.

Evaluation

This solution requires changes on RLC or MAC specification.

### Solution-D3: A new PDCP status report sent from Remote UE to the source gNB

As described by R2-2302859, the source gNB triggers the Remote UE to send a PDCP status report to the source gNB before the source gNB performs SN status transfer to the target gNB. The source gNB can then forward the buffered data to the target gNB, and the target gNB can retransmit PDCP Data PDUs to the Remote UE as required.

The PDCP status report can be triggered by the source gNB at one of the following timelines:

- Upon receiving the path switch command

- An explicit trigger before path switching command

- Measurement reporting event triggers status report

This solution requires to specify a new trigger for PDCP status report before handover for PDCP specification (TS38.323).

Evaluation

This solution can only work if the source gNB can receive an accurate PDCP status report before the SN status transfer, and assumes the the source gNB can send the required data to the target gNB during path switch.

### Solution-D4: Legacy Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

As proposed by some companies in the contributions, target gNB relies on the legacy PDCP status report sent from the Remote UE after path switch. The target gNB requests the source gNB to forward the DL packets that have not been acknowledged by Remote UE to it.

The data forwarding mechanism should be enhanced for the inter-gNB path switch, to allow source gNB forward all the PDCP SDUs in the buffer to the target gNB, and then, the target gNB re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch.

Evaluation

This solution basically is a legacy solution (following the legacy handling for inter-gNB handover.) with some assumed gNB implementation.

The feasibility of this solution depends if source gNB (PDCP sublayer) can buffer (i.e., will not discard) the DL data even though the delivery of the data may be acknowledged by its lower layer (i.e., RLC). In practice, this may require the source gNB to buffer a lot of data.

### Solution-D5: Proactive Data forwarding from source gNB to target gNB

Following the same principle of the solution-D4, this solution allow the source gNB to forward the data to the target gNB without receiving the request from target gNB, on top of the current mechanism. This means a new supplementary inter-gNB data forwarding is supported from source gNB target gNB. In practice, this may occur after the normal data forwarding or before it.

Evaluation

This solution will have Xn interface impact managed by RAN3.

The feasibility of this solution depends on if source gNB (PDCP sublayer) can buffer (i.e., will not discard) the DL data even though the delivery of the data may be acknowledged by its lower layer (i.e., RLC). In practice, this may require the source gNB to buffer a lot of data and lots of data needs to be forwarded to target gNB, which leads to unnecessary data forwarding, since this data forwarding is not based on the target gNB request.

## Discussion

### Question 11: Do companies agree that the decription and evaluation of solution-D1 is accurate for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 12: Do companies agree that solution- D1 is a valid solution for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 13: Do companies agree that the decription and evaluation of solution-D2 is accurate for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 14: Do companies agree that solution-D2 is a valid solution for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 15: Do companies agree that the decription and evaluation of solution-D3 is accurate for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 16: Do companies agree that solution-D3 is a valid solution for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 17: Do companies agree that the decription and evaluation of solution-D4 is accurate for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 18: Do companies agree that solution-D4 is a valid solution for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 19: Do companies agree that the decription and evaluation of solution-D5 is accurate for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 20: Do companies agree that solution-D5 is a valid solution for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 21: Do companies see any additional solution(s) for DL lossless data delivery for path switch?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

### Question 22: Do companies agree to take solution-D1/D2/D3/D4/D5 as the candidate solutions for DL lossless data delivery for path switch for downselection next meeting?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

TBD Rapporteur summary:

TBD: Proposal

# Conclusion and Proposal

We have the following proposals:

TBD

# Reference

- R2-2302493	Support of Lossless Path Switching

- R2-2302602	Considerations on Service Continuity Enhancements for L2 U2N Relay

- R2-2302859	Discussion on lossless data delivery during inter-gNB path switching

- R2-2302860	Discussion on service continuity issues for Inter-gNB path switching of L2 U2N relay

- R2-2302869	Discussion on lossless path switching and measurement events

- R2-2302903	Discussion on Inter-gNB Service Continuity

- R2-2302923	Lossless path switching from indirect to indirect/direct

- R2-2302971	Discussion on Service Continuity Enhancements

- R2-2302995	Path switching procedure for the service continuity enhancement

- R2-2303006	Further discussion on service continuity for SL relay

- R2-2303089	Service continuity enhancements for UE sidelink relay

- R2-2303110	Discussion on lossless data forwarding for inter-gNB service continuity	,

- R2-2303117	Discussion on service continuity enhancement

- R2-2303223	Service continuity for Inter-gNB path switching

- R2-2303341	Remaining issues on service continuity enhancement for L2 U2N relay

- R2-2303389	Discussion on Service continuity enhancement of L2 U2N relay

- R2-2303507	Scenarios and solution on lossless delivery during path switch from indirect path to target path

- R2-2303546	Discussion on service continuity	CMCC

- R2-2303558	Discussion on Service Continuity

- R2-2303564	Service continuity enhancements support for L2 U2N relay

- R2-2303609	CP and UP aspects of inter-gNB path switching

- R2-2304075	remaining issues for U2N path switching with lossless delivery

- R2-2304124	Lossless data delivery in the inter-gNB cases