3GPP TSG-RAN2 Meeting #121bis-e	R2-230xxxx

eMeeting, 17 – 26 April 2023

Agenda Item:	6.2.2 CP corrections

Source: 	Ericsson

Title:  	[AT121bis-e][601][MBS-R17] CP issues (Ericsson)

Document for:	Discussion and Decision

Introduction

This report provides a summary of the following offline discussion:

[AT121bis-e][601][MBS-R17] CP issues (Ericsson)

Scope: Review Tdocs/CRs submitted to 6.2.2, identify agreeable proposals and CRs for approval.

Outcome:

- Phase 1: Summary with proposals

- Phase 2: Updated summary and proposals, if needed, (updated) CRs

- Phase 3: CRs ready for approval

Deadline:

- Phase 1: Deadline for comments: W1 Thursday 0800 UTC

- Phase 2: Deadline for comments: W2 Tuesday 0500 UTC (report available for CB session, if needed)

- Phase 3: Agreeable CRs available EOM

The deadline for providing comments to phase 1 is Thursday 20th April 08:00 UTC.

Contact information

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Martin van der Zee | martin.van.der.zee@ericsson.com |
| Qualcomm | Umesh Phuyal | uphuyal@qti.qualcomm.com |
| CATT | Rui Zhou | zhourui@catt.cn |
| ZTE | Tao QI | qi.tao3@zte.com.cn |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Samsung | Anil Agiwal | anilag@samsung.com |
| Huawei, HiSilicon | Xubin | xubin10@huawei.com |
| Nokia | Jarkko Koskela | jarkko.t.koskela@nokia.com |
| Sharp | Fangying Xiao | Fangying.xiao@cn.sharp-world.com |
| MediaTek | Xiaonan Zhang | xiaonan.zhang@mediatek.com |

Phase 1

SPS related

### R2-2303919	Corrections on MBS SPS configuration		ASUSTeK			CR 38.331

Concerning the RAN2 questions about SPS configuration for unicast and multicast RAN1 replied (R2-2302406):

Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP.

Reply to Q1: Yes, from RAN1’s perspective, the similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network can only use SPS-ConfigToAddModList-r16 to configure SPS PDSCH for unicast in this case.

It is proposed to capture in the field description of sps-Config:

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList. Network does not configure SPS in one BWP using this field and sps-ConfigMulticastToAddModList-r17 simultaneously.

NOTE: the 1st change in R2-2303966 (see below) is the same as the change proposed in R2-2303919.

#### Q1: Do companies agree with the proposed change in R2-2303919?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Use italic for sps-ConfigMulticastToAddModList-r17 |
| Qualcomm | Yes | Agree with Ericsson’s comments. |
| CATT | Yes | Need to align with the RAN1 reply |
| ZTE | Yes | OK to record RAN1 feedback into 331. |
| vivo | Yes | Agree with Ericsson. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia | Maybe | No strong view. Change seems OK as such but not critical – Ericsson update is needed if agreed. |
| Sharp | Yes |  |
| MediaTek | Yes | To align with RAN1 LS |

### R2-2303966	Miscellabeous RRC corrections for MBS			Huawei, CBN		CR 38.331

2nd change

The field description of harq-FeedbackEnablerMulticast when the IE is absent is misaligned with TS 38.213, according to the RAN1's CR of R1-2212972:

When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions.

That means, if HARQ is disabled for some G-RNTIs or G-CS-RNTIs (by not configuring harq-FeedbackEnablerMulticast) and enabled for other G-RNTIs or G-CS-RNTIs and semi-static HARQ ACK codebook is used (configured per cell group), the UE should still report HARQ feedback for all G-RNTIs or G-CS-RNTIs to make sure the HARQ codebook size is aligned between UE and gNB.

Besides, in TS38.213, Clause 9.1.2, the following is specified:

If a Type-1 HARQ-ACK codebook would not include any HARQ-ACK information for transport blocks with enabled HARQ-ACK information, the UE does not provide the Type-1 HARQ-ACK codebook and does not transmit a corresponding PUCCH.

This is another case that the UE doesn’t provide HARQ feedback when the HARQ feedback is disabled. This should also be added to the field description. For other cases, the UE should provide HARQ feedback even if the HARQ feedback is disabled:

harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI as specified in TS 38.213 [13]. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent and pdsch-HARQ-ACK-Codebook is set to dynamic, the UE does not provide HARQ feedback for MBS multicast (see TS 38.213 [13], clause 18). When the field is absent and pdsch-HARQ-ACK-Codebook is set to semi-static, the UE does not provide HARQ feedback for MBS multicast if the Type-1 HARQ-ACK codebook would not include any HARQ-ACK information for transport blocks with enabled HARQ-ACK information (see TS 38.213 [13], clause 9.1.2).

#### Q2: Do companies agree with 2nd change in R2-2303966?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | We recognize that not everything that is specified in RAN1 is captured completely in RAN2, but that is ok. The cases when the PHY layer needs to add some bits are specified in RAN1 specs. Further, the behavior in case of absence as proposed in the last sentence in the CR is still under discussion in RAN1. So, instead of modifying RAN2 spec every time RAN1 changes something, as concluded in last meeting, we should just refer to RAN1 specifications. If the main argument is that even in case of this field being absent there are cases where HARQ feedback may need to be provided as per RAN1 spec, (and ASN.1 “Need S” needs us to capture something about absence), one option is to update the existing statement as follows, so that we don’t need to come back and fix it again when RAN1 makes more progress. When the field is absent, the UE does not provide HARQ feedback for MBS multicast (see behavior is specified in TS 38.213 [13], clause 18.) |
| CATT | comments | The change is needed, but we agree with QCOM’s view that we can simplify the description by referring to RAN1 spec as much as possible. |
| ZTE | comments | but agree with QC that duplicating RAN1 spec in 331 might not be necessary. maybe we can follow the normal practice, e.g., as in last RAN2 meeting, simply "referring to 38.213 [13] clause 9.1.2 and 18". |
| vivo | No | The current text with a reference to PHY spec is clear and sufficient. |
| Samsung | No | Reference to TS 38.213 is enough. There is no need to list all the detailed conditions in field description. |
| Huawei, HiSilicon | Yes | Proponent. We can also accept QC’s suggestion. “clause 18” is not needed in QC’s wording as this may refer to more than one clause: When the field is absent, the UE does not provide HARQ feedback for MBS multicast (see behavior is specified in TS 38.213 [13], clause 18.) |
| Nokia | Maybe with changes | As such we are not sure if we should try to capture all these details in RRC – it might be easier to just have reference to 38.213? |
| Sharp |  | We prefer the wording from QC. |
| MediaTek | No | Agree with Qualcomm’s view |

3rd change

CORESET#0 cannot be configured in SIB1:

locationAndBandwidthBroadcast Indicates starting PRB and the number of PRBs of CFR used for MCCH and MTCH reception. Value sameAsSib1ConfiguredLocationAndBW means the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1. Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP configured in SIB1 and CORESET#0. If the field is absent, the CFR for broadcast has the same location and size as CORESET#0.

#### Q3: Do companies agree with 3rd change in R2-2303966?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia | Yes | Editorial – do we need to explicitly mention SIB1 even. Could we just remove “configured in SIB1”? |
| Sharp | Yes |  |
| MediaTek | Yes |  |

### R2-2302590	Correction to PDSCH Aggregation of MBS SPS		vivo				CR 38.331

In 38.214 it says:

When receiving PDSCH scheduled by DCI format 4_1 or 4_2 for multicast reception in PDCCH with CRC scrambled by G-CS-RNTI, or PDSCH without corresponding PDCCH transmission using associated SPS-Config and activated by the DCI format 4_1 or 4_2 in PDCCH with CRC scrambled by G-CS-RNTI, the same symbol allocation is applied across the pdsch-AggregationFactor, in associated SPS-Config if configured, or 1 otherwise, consecutive slots.

In other words, when pdsch-AggregationFactor is not configured in SPS-Config, then only 1 slot is scheduled for multicast SPS PTM transmission, regardless of pdsch-AggregationFactor configured in pdsch-Config. However, the current field description of pdsch-AggregationFactor in SPS-Config mentioned that when the field is absent, the UE applies PDSCH aggregation factor of pdsch-Config. The

pdsch-AggregationFactor Number of repetitions for SPS PDSCH (see TS 38.214 [19], clause 5.1.2.1). When the field is absent, the UE applies pdsch-AggregationFactor in pdsch-config which is not used for MBS multicast data or the value 1 for MBS multicast data.

#### Q4: Do companies agree with the proposed change in R2-2302590?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Perhaps the following wording reads easier?: When the field is absent, the UE applies the value 1 for MBS multicast data and the pdsch-AggregationFactor in pdsch-config for other data. |
| Qualcomm | Intent ok, see comments | Rewording is needed, e.g. the following: (we should avoid using ‘other data’ and ‘MBS multicast data’.) When the field is absent, except for MBS multicast the UE applies pdsch-AggregationFactor in pdsch-config, and for MBS multicast the UE applies value 1. |
| CATT | Yes for the intention | The wording from Ericsson is clearer |
| ZTE | Yes and | slightly prefer Ericsson's revised version which is more concise. |
| vivo | Yes | We are fine with Ericsson’s revision. |
| Samsung | Yes | TP from Ericsson seems better. |
| Huawei, HiSilicon | Yes for the intention | On top of Ericsson’ wording, we can change “data” to “transmission”. |
| Nokia | Maybe with changes | Similar wording to what Ericsson proposes looks more correct. The other option could be adding a reference to TS23.214? |
| MediaTek | Yes | Prefer Qualcomm’s correction |

SPNP related

### R2-2302522	Remaining issues on Supporting MBS in SNPN		CATT, CBN		discussion

1st change

In RAN2#121 the following agreements were reached:

No explicit NID signaling is added in Uu. No explicit NID signaling is added in inter-node message in rel-17.

Based on above agreement, only plmn-Index can be used if the TMGI is to be included in MII and it is for a broadcast service on a SNPN:

plmn-Index PLMN index or NPN index according to the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1. If this field is included in the MRB-ToAddMod-r17, the UE translates the plmn-Index into the PLMN Identity or SNPN Identity based on the configuration in SIB1 (which is the SIB1 of the target cell in case of handover). Only plmn-Index(i.e., UE does not use explicitValue) can be used if the corresponding TMGI is to be included in MII and the service belongs to a SNPN.

serviceId Uniquely identifies the identity of an MBS service within a PLMN. The field contains octet 3- 5 of the IE Temporary Mobile Group Identity (TMGI) as defined in TS 24.008 [38]. The first octet contains the third octet of the TMGI, the second octet contains the fourth octet of the TMGI and so on.

#### Q5: Do companies agree with the 1st change proposed in R2-2302522?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comment | We agree to clarify that plmn-Index must be used for SNPNs, but this applies to all messages over Uu interface (Paging, MBSBroadcastConfiguration, mrb-ToAddModList and MII), i.e. we suggest the following simplified wording: The explicitValue is not used for MBS service(s) of an SNPN. |
| Qualcomm | Yes, see comment | Intent is ok, but proposed change should be reworded to e.g. “The explicitValue is not used if the corresponding TMGI is to be included in MII and the service belongs to a SNPN.” |
| CATT | Yes(proponent) | OK with Ericsson’ suggestion about applying it to all messages over Uu |
| ZTE | Yes and | again, Ericsson's suggestion is a more general and concise description. |
| vivo | Yes | Fine with Ericsson’s revision. |
| Samsung | Yes | Agree with Ericsson’s suggestion. |
| Huawei, HiSilicon | See comments | We think the intention is technically correct and agree with Ericsson that some other messages (except for mrb-ToAddModList) should also use plmn-index. For MBS multicast configuration (mrb-ToAddModList), the explicitValue can be used as the UE knows its serving NID. So the proposed wording from Ericsson is not 100% accurate. |
| Nokia | No | It is not possible to include explicit SNPN identity in the MII. There does not seem really any possibility that UE could try to do this as there is no ASN.1 supporting this kind of behaviour. Ericsson revision could be acceptable. |
| MediaTek | Yes | Agree with the intention and the words suggested by Ericsson and Qualcomm |

2nd change

In RAN2#121, it is agreed that,

RAN2 specs do not preclude MBS broadcast reception on non-serving SNPNs in Rel-17. This may require update to PLMN index field description in SIB1 (discussed together with PLMN ID indication changes). No explicit NID signaling is added in Uu. No explicit NID signaling is added in inter-node message in rel-17.

Based on the agreement above, it is possble UE receives MBS broadcast on non-serving SNPNs, so it can report plmn index of non-serving SNPNs in MII message. For PLMN case, it is agreed the PLMN index is replaced by PLMN ID in inter-node message during handover. But for SNPN, it is not possible to replace plmn-index with SNPN ID (i.e., PLMN+NID) in inter-node message as the asn.1 structure does not support it. For TMGI belongs to the serving SNPN, the plmn-index can replaced by PLMN ID and the NID part (i.e. ,the serving NID) can be included in the legacy HO Request message,so the target cell can still format a complete SNPN ID.But for non-serving SNPN,the NID part can not be transferred in inter-node message during handover,So there should be a restriction that the PLMN IDs of these non-serving SNPNs are also not transferred in inter-node message during handover, or it will cause ambiguity in target node.

Proposal 2: PLMN IDs of non-serving SNPNs are not transferred in MII message contained in inter-node message during handover.

AS-Context field descriptions

mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID, if needed. For plmn-Index pointing to a non-serving SNPN, the corresponding PLMN ID is not transferred in MII message contained in inter-node message during handover.

#### Q6: Do companies agree with the 2nd change proposed in R2-2302522?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comments | The wording is not explicit whether the plmn-Index is sent or not, i.e. more clear to say that such TMGI is removed: A TMGI for which the plmn-Index points to a non-serving SNPN is removed from the NR MBSInterestIndication message. @QC: - Our understanding is that “if needed” indicates the case when the Broadcast PLMN Identity Info List NR IE in the Served Cell Information NR IE is included in the XN SETUP REQUEST message and the plmn-Index can be used. Perhaps this should be clarified. - Our understanding is that only the serving NID is supported in the NPN Mobility Information, i.e. non-serving SNPNs cannot be signalled. |
| Qualcomm | No, see comments | First, I thought the first (existing) sentence was already intended to be changed to “may be replaced” (instead of ‘is replaced’) as discussed in the last meeting. Somehow this one was missed to be aligned or was out of scope, I am not sure now. Then, in some cases networks may be able to handle sending even the ‘plmn-index’. How does simply ‘not transferring’ plmn-Index is done and how does it solve the issue? Note: It is not possible to just ‘not include’ neither plmn-index nor explicitValue based on the ASN.1 as plmn-Id-r17 is mandatory inside TMGI-r17. TMGI-r17 ::= SEQUENCE { plmn-Id-r17 CHOICE { plmn-Index INTEGER (1..maxPLMN), explicitValue PLMN-Identity }, serviceId-r17 OCTET STRING (SIZE (3)) } |
| CATT | Yes(proponent) | Agree with Ericsson that neither the plmn-index nor the corresponding PLMN ID are to be transferred in MII message contained in inter-node message during handover. |
| ZTE | OK with the intention |  |
| vivo | Comments | Before going into the correction, we need to first clarify whether MII reporting is not supported for the broadcast within non-serving SNPN? |
| Samsung | Yes |  |
| Huawei, HiSilicon | See comments | NW may have different behaviours in different cases: - If there is Xn interface established, the target cell will know the PLMN list and NID list of source cell. In this case, the source can transmit the corresponding plmn-index of the non-serving SNPN to the target. However, it seems not useful anyway even the target can get this information as inter-SNPN HO is not supported. - If there is no Xn interface established, the source can just drop this TMGI from the MII message to avoid confusion. In this sense, we think this can be left to gNB implementation. |
| Nokia | No | There is no explicit identity in the MII to be transferred. So how could one provide it even if one wants to? |
| MediaTek | No | It depends on network’s behavior. No need to specify something which is “not transferred”. |

### R2-2303552	Misc correction to TS 38.331 on NR MBS			ZTE, Sanechips		CR 38.331

1st change

For each session it is indicated via a bitmap which neighbour cells support or do not support the session:

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17           MBS-SessionInfoList-r17                      OPTIONAL,  -- Need R

mbs-SessionId-r17             TMGI-r17,

mtch-NeighbourCell-r17        BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL,  -- Need S

…

mbs-NeighbourCellList-r17         MBS-NeighbourCellList-r17                    OPTIONAL,  -- Need S

MBS-NeighbourCellList-r17 ::= SEQUENCE (SIZE (0..maxNeighCellMBS-r17)) OF MBS-NeighbourCell-r17

…

The field description for mtch-neighbourCell is not complete and even wrong, i.e. the following three use cases are not clearly captured:

- if the mbs-NeighbourCellList is absent, then mtch-NeighbourCell shall be absent as well, and UE is not aware of info in neighbour cell;

- if the mbs-NeighbourCellList is empty, then mtch-NeighbourCell shall be absent as well, and UE considers the service is not available in any neighbour cell;

- if a non-empty mbs-NeighbourCellList is configured and mtch-neighbourCell is absent, UE is not aware of the info in neighbour cell;

mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. This field shall be absent if the mbs-NeighbourCellList is absent, in such case, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. This field shall be absent if the mbs-NeighbourCellList is empty, in such case the related service are not provided in any neighbouring cell. If a non-empty mbs-NeighbourCellList is configured and mtch-neighbourCell is absent, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field.

#### Q7: Do companies agree with the 1st proposed change in R2-2303552?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comments | In our understanding the UE assumes the same for a service when the IE is absent and mbs-NeighbourCellList is absent or non-empty. For the case when the IE is absent and mbs-NeighbourCellList is empty we propose to use the same wording as in the field description of mbs-NeighbourCellList to avoid any confusion: mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. The field is absent when mbs-NeighbourCellList is absent or empty. If this field is absent and mbs-NeighbourCellList is absent or non-empty, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. If this field is absent and mbs-NeighbourCellList is empty, then the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. |
| Qualcomm | Yes, with comments | Agree with Ericsson’s comments. To align further with the field description of mbs-NeighboourCellList, following rewording is proposed (taking Ericsson’s version as baseline). mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. The field is absent when mbs-NeighbourCellList is absent or an empty mbs-NeighbourCellList is signalled. If this field is absent, and when mbs-NeighbourCellList is absent or a non-empty mbs-NeighbourCellList is signalled, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. If this field is absent and an empty mbs-NeighbourCellList is signalled, the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. (for reference) mbs-NeighbourCellList List of neighbour cells providing one or more MBS broadcast services via broadcast MRB that are provided by the current cell. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. When a non-empty mbs-NeighbourCellList is signalled, the current serving cell does not provide information about MBS broadcast services of a neighbour cell that is not included in mbs-NeighbourCellList, i.e., the UE cannot determine the presence or absence of an MBS service of a neighbour cell that is absent. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS broadcast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field. |
| CATT | Yes for a,b No for c | c) is not a valid case in our view. In our view,If mbs-NeighbourCellList is non-empty, mtch-neighbourCell should be present. |
| ZTE | Yes (as proponent) | agree with Ericsson's revision, i.e., to align the wording. agree with Qualcomm comment to make it even more clearer. |
| vivo | No strong view for a,b; No for c | Agree with CATT. The network configuration will avoid this case (no benefit foreseen). |
| Samsung | Yes, with comments | Prefer TP from Ericsson with one change. Delete “The field is absent when mbs-NeighbourCellList is absent or empty.”. The field can be absent irrespective of whether mbs-NeighbourCellList is absent or not, empty or non empty. |
| Huawei, HiSilicon | Yes but | QC’s wording is also fine with us. |
| Nokia | No | The mtch-neighbourCell field description changes proposed are repeating what is already clear from the existing field descriptions for this field and the mbs-NeighbourCellList field. |
| MediaTek | Yes with comment | Agree with the description for a and b For scenario c, we think it means the session is not available in any neighbour cell; I.e., if a non-empty mbs-NeighbourCellList is configured and mtch-neighbourCell is absent, UE considers the service is not available in any neighbour cell; |

2nd change

plmn-Index shall be clarified for broadcast as well since we have updated it for multicast in RAN2#121. Other than alignment with multicast, such clarification is needed to avoid wrong MII report from UE side:

plmn-Index PLMN index or NPN index according to the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1. If this field is included in the MRB-ToAddMod-r17, the UE translates the plmn-Index into the PLMN Identity or SNPN Identity based on the configuration in SIB1 (which is the SIB1 of the target cell in case of handover). If this field is included in the MBS-SessionInfoList, the UE translates the plmn-Index into the PLMN Identity or SNPN Identity based on the configuration in SIB1. If this field is included in the mbs-ServiceList in MBSInterestIndication message, the UE translates the PLMN Identity or SNPN Identity back to plmn-Index based on the configuration in SIB1; the source gNB decodes the MBSInterestIndication, translates the plmn-index to explicit PLMN ID and replaces the plmn-index with the explicit PLMN ID when sending MBSInterestIndication to target gNB in case of handover.

#### Q8: Do companies agree with the 2nd proposed change in R2-2303552?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No, see comments | We do not think these clarifications are needed, i.e. for multicast this was a special case, because the UE is required to store the PLMN/SNPN identity for the handover case. But for broadcast there is no requirement for the UE to store the PLMN/SNPN identity. In R2-2302522 the plmn-Index with SNPNs is clarified. The replacement of plmn-Index with PLMN ID is already captured for the AS-Context in HandoverPreparationInformation message: mbsInterestIndication Includes the information last reported by the UE in the NR MBSInterestIndication message, where the plmn-Index (if included by the UE in tmgi) is replaced by the PLMN ID, if needed. |
| Qualcomm | No | Similar comment as Ericsson: the gNB part is already captured in specifications, the UE part (i.e. UE translates back to plmn-Index based on the configuration in SIB1) is obvious when UE wants to include plmn-Index. |
| CATT | No | Agree with Ericsson that these clarifications are not necessary. The reason for such translation for multicast is to support delta config during HO, but that is not the case for broadcast. So it seems no need to have such clarification for broadcast. |
| ZTE | Yes (as proponent) | the principle we had agreed in last meeting is, UE locally always store the explicit value, i.e., the index thing is only used for signaling overhead reduction (and in SNPN case it is life-saving.. though). if not, there will be issues as what is broadcast on air in SIB1 may change. it seems there are already different understandings based on above comments. if majority think this is something obvious, especially from UE vendor or chipset vendor, we are OK. |
| vivo | Comments | We are fine with the intention of the first change: If this field is included in the MBS-SessionInfoList, the UE translates the plmn-Index into the PLMN Identity or SNPN Identity based on the configuration in SIB1. If this field is included in the mbs-ServiceList in MBSInterestIndication message, the UE translates the PLMN Identity or SNPN Identity back to plmn-Index based on the configuration in SIB1; This is used for indicating UP resource establishment to the upper layer for a Session marked by TMGI. The second is not needed as per Ericsson’s comment. |
| Samsung | No | Agree with Ericsson. |
| Huawei, HiSilicon | No | Similar view with companies above that this is not needed. |
| Nokia | No | So we are talking about broadcast – what could be wrong UE behaviour here? There is no way UE would interpret plmn-Index based on some other cell SIB1. So we don’t really see need for this. = we agree with ericsson comment above |
| MediaTek | No | Agree with Ericsson. |

Miscellaneous

### R2-2302523	Corrections to TS 38.331					CATT, CBN		CR 38.331

1st change

The search space for MCCH, i.e. searchSpaceMCCH-r17, is provided in SIB1, but this is not covered in the general description of 5.9.1.1 for the configuration information required by UE to receive MCCH:

5.9.1.1	General

UE receiving or interested to receive MBS broadcast service(s) applies MBS broadcast procedures described in this clause as well as the MBS Interest Indication procedure as specified in clause 5.9.4.

MBS broadcast configuration information is provided on MCCH logical channel. MCCH carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSBroadcastConfiguration message may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. The configuration information required by the UE to receive MCCH is provided in SIB1 and SIB20. Additionally, System Information provides also an information related to service continuity of MBS broadcast in SIB21.

2nd change

A cell may provide SIB20 but it does not always broadcasting SIB20 as on demand manner is supported for SIB20. In the current description of 5.9.2.3, UE decides to acuqire MCCH based on whether SIB20 is broadcasting in the target cell, but this is not correct since UE should check whether SIB20 is provided in the scheduling information of SIB1:

5.9.2.3	MCCH information acquisition by the UE

An MBS capable UE interested to receive or receiving an MBS broadcast service shall:

1>	if the procedure is triggered by an MCCH information change notification:

2>	start acquiring the MBSBroadcastConfiguration message on MCCH in the concerned cell from the slot in which the change notification was received;

1>	if the UE enters a cell providing SIB20; or

1>	if the UE receives sCellSIB20:

2>	acquire the MBSBroadcastConfiguration message on MCCH in the concerned cell at the next repetition period.

3rd change

UE should firstly establish the SDAP entity and then receive the DL-SCH for broadcast reception. But in 5.9.3.3, the descrpition of establishing SDAP entity is after the description of receiving DL-SCH, which is not correct:

5.9.3.3	Broadcast MRB establishment

Upon a broadcast MRB establishment, the UE shall:

1>	if an SDAP entity with the received mbs-SessionId does not exist:

2>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1.

2>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers.

1>	establish a PDCP entity and an RLC entity in accordance with MRB-InfoBroadcast for this broadcast MRB included in the MBSBroadcastConfiguration message and the configuration specified in 9.1.1.7;

1>	configure the MAC layer in accordance with the mtch-SchedulingInfo (if included);

1>	configure the physical layer in accordance with the mbs-SessionInfoList, searchSpaceMTCH, and pdsch-ConfigMTCH, applicable for the broadcast MRB;

1>	receive DL-SCH on the cell where the MBSBroadcastConfiguration message was received for the established broadcast MRB using g-RNTI and mtch-SchedulingInfo (if included) in this message for this MBS broadcast service;

#### Q9: Do companies agree with the changes proposed in R2-2302523?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes to all 3 |  |
| Qualcomm | See comment | On the 3rd change, the intent seems ok, however the text should be move only immediately above the “1> receive DL-SCH…” instead of moving all the way to the top. That is because, otherwise the reference to MBSBroadcastConfiguration (which was originally in the first bullet 1>) now moves to the second place, i.e. after the SDAP bullet). |
| CATT | Yes(proponent) | On the 3rd change, the suggested change is OK to us. |
| ZTE | ok with all. |  |
| vivo | Okay only for 1/3. | For 2, the UE can only acquire the broadcast configuration if SIB20 is provided. Nothing is wrong. We fail to see the motivation for this change. For 3, agree with Qualcomm. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes with comments | On the 3rd change, agree with QC’s suggestion. |
| Nokia | Yes to all | regarding 2nd change – word “providing”? Is that clear that it includes the SIB1 scheduling SIB20? We could write bit more long version like in 38.304 if not seen clear like this i.e. SIB1 scheduling information contains SIB20 |
| Sharp | Yes |  |
| MediaTek | Yes to all 3 |  |

### R2-2302823	CP Corrections for MBS					Samsung			CR 38.331

According to 5.3.2.3:

If UE is in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList:

- if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers

  - UE initiates the RRC connection resumption procedure

The highlighted text only considers the scenario when paging record(s) are included in paging message. The scenario that there are no paging records in paging message is missing. In this scenario UE behaviour should be same as the case paging records are included but UE identity is not included in any of these paging record.

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList, if any, included in the Paging message:

2>	if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers; or

2> if PagingRecord is not included in the Paging message:

3>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below:

4>	if the UE is configured by upper layers with Access Identity 1:

5>	set resumeCause to mps-PriorityAccess;

4>	else if the UE is configured by upper layers with Access Identity 2:

5>	set resumeCause to mcs-PriorityAccess;

4>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

5>	set resumeCause to highPriorityAccess;

4>	else:

5>	set resumeCause to mt-Access;

2>	else:

3>	forward the TMGI(s) to the upper layers;

#### Q10: Do companies agree with the proposed changes in R2-2302823?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comments | It is not clear that if any, refers to one or more paging record(s). We propose to say: 1> if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList, and PagingRecordlist is included in the Paging message: End add “List” here: 2> if PagingRecordList is not included in the Paging message: |
| Qualcomm | Intent ok, see comments | The new condition “2> if PagingRecord is not included in the Paging message:” can be the first condition. Then the existing “if included in the Paging message" becomes redundant and can be removed as shown below. Also agree to Ericsson’s comment that “List” is missing in the PagingRecorList. 2> if PagingRecordList is not included in the Paging message, or 2> if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers: 3> initiate the RRC connection resumption procedure… |
| CATT | Yes | On the ambiguity of “if any” mentioned by Ericsson,we understand the intention is to say if pagingGroupList is present in PAGING message,so suggest a rewording as below, 1> if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList, if pagingGroupList is present: |
| ZTE | OK |  |
| vivo | No | The current text with “if included in the Paging message” has covered the meaning. Nothing is wrong. |
| Samsung | Yes | Regarding comments from Ericsson: - The change ‘If any’ is not related to paging records. pagingGroupList may not be included in the paging message. ‘if any’ in the text ‘if any, included in the Paging message’ refers to pagingGroupList. Ok for the second change. PagingRecord PagingRecordList |
| Huawei, HiSilicon | See comments | The 1st change is not needed. There should be no misunderstanding even without it. On the 2nd change, it looks fine to us. QC’s wording is also OK. |
| Nokia | yes with intention | QC version looks rather OK |
| Sharp |  | We prefer the wording from QC. |
| MediaTek | Yes | Agree with the rewording by Ericsson and QC |

### R2-2303031	Clarificaition on Key Refresh in MBS			vivo				CR 38.331

As both (de)ciphering and integrity protection are not supported for the PDCP associated with a multicast MRB, as a result, key refresh is not needed for either multicast MRB and broadcast MRB (i.e. key refresh is only for SRB or DRB). However, such kind of clarification is missing in the current RRC spec:

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security (for SRBs and DRBs) and re-establishment of RLC and PDCP triggered by explicit indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators.

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security (for SRBs and DRBs) and re-establishment of RLC and PDCP triggered by explicit indicators;

-	for DAPS bearer: establishment of RLC for the target PCell, refresh of security and reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: refresh of security and establishment of RLC and PDCP for the target PCell;

#### Q11: Do companies agree with the proposed changes in R2-2303031?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Qualcomm | No | While intent is correct, the CR doesn't seem essential since this will be clear from other parts of the specifications. |
| CATT | No | Not essential. It is already clear in 38.323 security(ciphering, integrity protection) is not applied to MRBs |
| ZTE | Maybe not | if AS security does not apply to MRB, then the original wording "refresh of security" does not apply to MRB. |
| vivo | Yes (propoent) |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | No | Same view with QC and CATT. |
| Nokia | Yes | But editorial anyway as clear in 38.323. |
| Sharp | No | We understand that it was already mentioned in TS38.323. |
| MediaTek | Yes | OK to clarify, or we can say “except MRBs”? |

### R2-2303619	Corrections for MBS with eDRX and MICO mode	Ericsson			CR 38.304

When the UE joins an MBS multicast session when configured by upper layers with eDRX or MICO mode there can be inter-operability issues. When the UE enters RRC_IDLE the UE may not be reachable for group paging when the session is activated.

The UE can receive MBS broadcast, when the UE is configured by upper layers with eDRX or MICO mode without inter-operability problems, i.e. this can be left to UE implementation.

Proposed changes in 38.304:

4.1	Overview

<TEXT OMITTED>

When the UE is in RRC_IDLE state, upper layers may deactivate AS layer when MICO mode is activated as specified in TS 24.501 [14]. When MICO mode is activated, the AS configuration (e.g. priorities provided by dedicated signalling) is kept and all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while MICO mode is activated it is up to the UE implementation whether it performs the corresponding action immediately or the latest when MICO mode is deactivated. When MICO mode is deactivated, the UE shall perform all idle mode tasks.

NOTE:	It is up to UE implementation to receive MBS broadcast when MICO mode is activated.

The UE shall not join a multicast session, as specified in TS 24.501 [14], when the UE is configured with MICO mode by upper layers. The UE shall not request MICO mode, as specified in TS 24.501 [14], when the UE has joined a multicast session.

<TEXT OMITTED>

7.4	Paging in extended DRX

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN. The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1. If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE.

NOTE:	It is up to UE implementation to receive MBS broadcast when the UE operates in eDRX for CN or RAN paging.

The UE shall not join a multicast session, as specified in TS 24.501 [14], when the UE is configured by upper layers with an extended DRX (eDRX) cycle TeDRX, CN. The UE shall not request eDRX cycle TeDRX, CN, as specified in TS 24.501 [14], when the UE has joined a multicast session.

#### Q12: Do companies agree with the proposed changes in R2-2303619?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | This needs to be clarified for the multicast case, because this could otherwise lead to inter-operability issues. The network cannot avoid that the UE joins a multicast session (SMF) when the UE is configured with eDRX/MICO mode (AMF), because this is handled by different CN nodes. Reception of MBS broadcast with eDRX/MICO mode can be left to UE implementation. |
| Qualcomm | Ok |  |
| CATT | See comments | For broadcast, no change is needed, it can be up to UE implementation. For multicast, no need to add the text, we are OK to clarify the UE behaviour in the NOTE, NOTE: If the UE is interested to receive a multicast session, UE can choose to not request MICO/eDRX via legacy NAS signaling |
| ZTE | comments | the broadcast part, e.g., - for section 4.1, "It is up to UE implementation to receive MBS broadcast when MICO mode is activated.", and - for section 7.4, "It is up to UE implementation to receive MBS broadcast when the UE operates in eDRX for CN or RAN paging." are not needed, as broadcast reception is a DL only behaviour, and it does not affect MICO mode or eDRX for any paging. |
| vivo | comments | We are fine with the proposed NOTE regarding broadcast reception. For the other part, we propose to have some limitations to NW implementation, e.g. the network shall not release the UE to IDLE/INACTIVE with joining a activated multicst session and configured with eDRX. |
| Samsung | Yes |  |
| Huawei, HiSilicon | See comments | We have similar understanding with CATT and think both broadcast and multicast can be left to UE implementation without spec change. |
| Nokia | No | This can be left to NW implementation – NW may prevent not sending UE to MICO/eDRX if it joins multicast. In addition, in Rel-18, SA2 had studied and concluded a solution that targets optimizing both scenarios. No need for bringing any restrictions here CATT proposal could be acceptable. |
| Sharp | Yes |  |
| MediaTek | Yes | Agree with the intention |

Editorials

### R2-2303127	General MBS CR to 38.331				Nokia				CR 38.331

Various editorial corrections to the 38.331:

- Lots of “e.g.” and “i.e.” are missing comma after them.

- Message text style not using italics

#### Q13: Do companies agree with the proposed change in R2-2303127?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comments | In section 5.3.2.2 it seems that the original text was accidently modified, i.e. already indicated italic Paging. And the spelling in 5.9.2.2 is incorrect. |
| Qualcomm | No | Absolutely non-essential edits. This is correctly a Cat D, but if authors are so keen, they can raise this during CR implementation directly to MCC. In addition, as Ericsson pointed out correctly also, this CR adds more editorial errors (in some cases where the text was already correct)! |
| CATT | Yes | OK to correct it even it is editorial |
| ZTE | no strong view | but share the same view with QC that this can be directly suggested during CR implementation phase. |
| vivo | No strong view |  |
| Samsung | Yes | This purely editorial change should be merged to other CRs. |
| Huawei, HiSilicon | Yes but | Better to be included in a mega CR (improvement needed as indicated by Ericssion). |
| Nokia | Yes (proponent) |  |
| Sharp | Yes |  |
| MediaTek | Yes |  |

### R2-2304170	Editorial modification to TS 38.331 on NR MBS		MediaTek 			CR 38.331

The ENUMERATED value for mcch-ModificationPeriod-r17 in SIB20 has an editorial error:

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, rf2048, rf4096, rf8192, rf16384, rf32768, rf65536}

#### Q14: Do companies agree with the proposed change in R2-2304170?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comment | The used styles are not correct. |
| Qualcomm | No | Purely editorial Cat D. Could be done by MCC directly during CR implementation or included in RRC rapp CR. No MBS-specific CR is needed. |
| CATT | Yes | OK to correct it even it is editorial |
| ZTE |  | same as above. |
| vivo | Yes |  |
| Samsung | Yes | This purely editorial change should be merged to other CRs. |
| Huawei, HiSilicon | Yes but | Better to be included in a mega CR. |
| Nokia | Yes | agree with Ericsson |
| Sharp | Yes |  |
| MediaTek | Yes |  |

For discussion

### R2-2303967	Discussion on the remainning MBS issues			Huawei, HiSilicon		discussion

MII reporting during SDT procedure in RRC_INACTIVE

Currently in Rel-17, MII reporting is used to inform the network that the UE is receiving or is interested to receive MBS broadcast service(s) in RRC_CONNECTED state. After receiving the MII, the network will be able to schedule the unicast and broadcast service(s) properly according to UE capability. For example, if the FDM transmission of unicast PDSCH and broadcast GC-PDSCH within a slot is not supported by the UE, the intra-slot FDM scheduling can be avoided, and inter-slot TDM scheduling may be used.

On another hand, SDT has been introduced in Rel-17 for power saving purpose. A UE is allowed to perform transmission of small data/signalling while remaining in RRC_INACTIVE state. During SDT procedure, the network can schedule subsequent DL transmissions with dynamic DL assignments. However, the MII report is restricted to RRC_CONNECTED only in the current specification. Hence, the network may not be aware of the MBS services the UE receives during the SDT procedure. This may result in data loss if there is over scheduling exceeding UE capability when MBS broadcast reception and SDT are performed simultaneously. To avoid data loss, MII reporting should be allowed during SDT procedure in RRC_INACTIVE state. TPs are provided on 38.331 and 38.300.

Proposal 1: Allow MII reporting during SDT procedure in RRC_INACTIVE.

#### Q15: Do companies agree with proposal 1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This is an optimization and not something critical to fix in Rel-17. |
| CATT | No | Such optimization is not critical as the co-existence of broadcast reception and SDT should be a rare case. |
| ZTE | No | the purpose of MII is for better scheduling of UE in RRC_CONNECTED. |
| vivo | No | The NW anyway cannot guarantee service continuity for this INACTIVE UE. No benefit is foreseen. |
| Samsung | No |  |
| Huawei, HiSilicon | Yes | Proponent. When we specifying MII reporting, SDT is not considered. But later on, RAN2 discussed and agreed SDT can co-exist with MBS. In this case, considering the benefit, it is not logical to restrict that the MII can only be reported in RRC_CONNECTED. |
| Nokia | No | Agree with Ericsson |
| Sharp | No | It is an optimization. |
| MediaTek | No | This may introduce more detail discussion, which are not pursued in this stage (for Rel-17) |

EHC and MBS multicast

In RAN2#112-e meeting and RAN2#116-e meeting, it was agreed that ROHC and EHC are supported for MBS multicast MRB:

Agreement - RoHC (at least U-mode) can be configured for NR MBS bearers. This is applicable for Mcast, assume this is applicable also to broadcast. - EHC is supported for MRB for cases when feedback path is available (UL RLC) and it is expected that no further optimizations are needed.

However, in current 38.306, the description on corresponding capabilities are missing, i.e. ehc-r16 and jointEHC-ROHC-Config-r16. To make the applicability of these features clear, we propose to add the missing description on multicast MRB to the specification. A TP for TS 38.306 is provided in Annex 3:

4.2.4	PDCP Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF |
| --- | --- | --- | --- |
| ehc-r16 Indicates that the UE supports Ethernet header compression and decompression using EHC protocol, as specified in TS 38.323 [16]. The UE indicating this capability and indicating support for at least one ROHC profile, shall support simultaneous configuration of EHC and ROHC on different DRBs/multicast MRBs. | UE | No | No |
| jointEHC-ROHC-Config-r16 Indicates whether the UE supports simultaneous configuration of EHC and ROHC protocols for the same DRB/multicast MRB. | UE | No | No |

Proposal 2: Adopt the 38.306 TP in the Annex3 to specify that the ehc-r16 and jointEHC-ROHC-Config-r16 are applicable for multicast MRBs.

#### Q16: Do companies agree with proposal 2 and the proposed corrections?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes, with comment | A normal UE does not support MRB, i.e. we wonder if we should explicitly write: ehc-r16 Indicates that the UE supports Ethernet header compression and decompression using EHC protocol, as specified in TS 38.323 [16]. The UE indicating this capability and indicating support for at least one ROHC profile, shall support simultaneous configuration of EHC and ROHC on different DRBs.The UE indicating this capability and indicating support for at least one ROHC profile and indicating support of dynamicMulticastPCell-r17 shall support simultaneous configuration of EHC and ROHC on different DRBs/multicast MRBs. jointEHC-ROHC-Config-r16 Indicates whether the UE supports simultaneous configuration of EHC and ROHC protocols for the same DRB and for the same multicast MRB when the UE indicates support of dynamicMulticastPCell-r17. But perhaps the proposed wording is simpler and clear enough, i.e. no strong view. |
| CATT | Yes |  |
| ZTE | Yes |  |
| vivo | Yes | Fine with Ericsson’s revision. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia | Yes | We are ok with Ericsson update. |
| Sharp | Yes |  |
| MediaTek | Yes |  |

PLMN ID in TMGI and PLMN IDs in SIB1

According to the current TS 38.331, maximum of 1024 MBS broadcast sessions can be transmitted via broadcast MRB. For each MBS broadcast session, the PLMN ID of the session is indicated within the TMGI. In our view, the PLMN ID indicated in the TMGI for MBS broadcast sessions should be one among the PLMN ID list indicated in SIB1 (i.e.plmn-IdentityInfoList within cellAccessRelatedInfo ). To avoid ambiguity, we would like RAN2 to confirm this as a common understanding.

CellAccessRelatedInfo information element CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityInfoList PLMN-IdentityInfoList, cellReservedForOtherUse ENUMERATED {true} OPTIONAL, -- Need R ..., [[ cellReservedForFutureUse-r16 ENUMERATED {true} OPTIONAL, -- Need R npn-IdentityInfoList-r16 NPN-IdentityInfoList-r16 OPTIONAL -- Need R ]], [[ snpn-AccessInfoList-r17 SEQUENCE (SIZE (1..maxNPN-r16)) OF SNPN-AccessInfo-r17 OPTIONAL -- Need R ]] } TMGI information element TMGI-r17 ::= SEQUENCE { plmn-Id-r17 CHOICE { plmn-Index INTEGER (1..maxPLMN), explicitValue PLMN-Identity }, serviceId-r17 OCTET STRING (SIZE (3)) }

Proposal 3: RAN2 confirms that the PLMN ID indicated in the TMGI for MBS broadcast sessions is among the PLMN ID list indicated in SIB1 (i.e. plmn-IdentityInfoList).

#### Q17: Do companies confirm the understanding in proposal 3?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No, see comments | Strictly speaking this only holds when the plmn-Index is used. The UE sends the MII content based on the USD/SIB21 info, and we are not sure to which extend the USD content is synced with SIB1 configuration. We also assume that the gNB initiates group paging for TMGIs not included in SIB1. Perhaps the TMGIs in SIB21 and MCCH include only PLMN IDs included in SIB1, but this restriction has not been specified. |
| Qualcomm | No | Agree with Ericsson’s comments. Also, somewhat relevant but not exactly: RAN2 previously discussed and replied to RAN3 that “From RRC point of view there is no restriction that the TMGIs for the broadcast services that UE is interested to receive or is receiving should contain PLMN ID broadcasted in SIB1”. |
| CATT | No | Such limitation is not aligned with previous agreement, as mentioned by QCOM. |
| ZTE | No | Agree with Ericsson’s comments. |
| vivo | No | NW implementation with PLMN ID can avoid this issue. Nothing is wrong. |
| Samsung | No | We do not see a motivation for this restriction. |
| Huawei, HiSilicon | Yes | Proponent. In real deployment, the possibility of supporting more PLMNs than broadcast in SIB1 is rather low. In that case, assuming so many PLMNs will add complexity during implementation. |
| Nokia | No | We do not see motivation to have such a limitation specified explicitly. This can be up to NW implementation |
| Sharp | No | Agree with Ericsson’s view. |
| MediaTek | No | Agree with Ericsson and Qualcomm |

MBS broadcast reception on SCell and plmn-Index on MCCH

For broadcast reception on Scell, network sends SIB20 of Scell via RRC dedicated signalling to UE, and UE receives MCCH message of Scell. However, UE will not obtain SIB1 of Scell. That is to say, UE will not know the relationship between PLMN index and PLMN identity and the relationship between NPN index and NPN identity of Scell. If there are MBS service(s) indicated by plmn-Index field in MCCH message of Scell, UE cannot identify TMGI or TMGI+NID of the MBS service(s) included in MCCH message of Scell and cannot determine MTCH configuration of MBS service(s) of interest because the matching of the TMGI or TMGI+NID cannot be performed.

In order to enable UE to receive broadcast on Scell, we propose that network also sends the relationship between PLMN index and PLMN identity and the relationship between NPN index and NPN identity of Scell (e.g., SIB1 of Scell) when sending the SIB20 of Scell via RRC dedicated signalling to the UE.

Proposal 4: Network also sends the relationship between PLMN index and PLMN identity and the relationship between NPN index and NPN identity of Scell (e.g., SIB1 of Scell) when sending the SIB20 of Scell via RRC dedicated signalling to the UE.

#### Q18: Do companies agree with proposal 4, i.e. do companies think that a correction is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Not sure about the solution though: the network could send sCellSIB1 when plmn-Index is used in MCCH. But not all SIB1 info is needed and another IE could be introduced as well. Furthermore Q18 is related to Q19. |
| Qualcomm | See comment | The problem seems genuine but not sure about solution though. It seems we should’ve added explicit NID in TMGI anyway as that could have been straightforward for so many workarounds. |
| CATT | Yes with comments | But it seems the correction is a NBC change(i.e., add sCellSIB1 in SCellConfig).not sure if it can be done at this late phase |
| ZTE | comment | same concern as QC, we have done so much for a non NBC change. still, the unexpected are everywhere ^^. but if we need a solution, P4 may be the way to go. |
| vivo | Comments | We are fine with the change if it is the majority view. Alternatively, we can clarify that this mentioned case cannot be supported in Rel-17. |
| Samsung | Yes | Scell-Config may need to include the relationship |
| Huawei, HiSilicon | Yes | Proponent. Without the mapping between plmn-index and explicit ID, the UE wouldn’t know the exact TMGI broadcast on the SCells. And basically, the plmn-index doesn’t work at all. |
| Nokia | No | Regarding PLMN MBS reception one can always use explicit signaling. Then regarding NPN and index usage – same could be avoided by just adding explicit signaling of NPN in TMGI and we have no issues whatsoever. So making this proposed ASN.1 NBC change is not good idea but if we do something then we add explicit identity in the TMGI for all cases. |
| MediaTek | No | We should clarify that this case is not support in Rel-17 at this stage |

MBS broadcast reception on SCell and broadcast CFR and PDSCH configuration of MCCH

The network can configure the broadcast CFR by indicating that the broadcast CFR has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1. And the network can configure the PDSCH configuration of MCCH by indicating that PDSCH configuration of MCCH is the same as PDSCH configuration provided in initialDownlinkBWP in SIB1.

However, for broadcast reception on Scell, UE will not obtain SIB1 of Scell. That is to say, UE will not know the location and size for initial BWP configured in SIB1 of Scell and PDSCH configuration provided in initialDownlinkBWP in SIB1. In this case, according to the current specification, UE cannot determine the location and size of the broadcast CFR of Scell and the PDSCH configuration of MCCH of Scell, and cannot receive MCCH message of Scell.

In addition, it is worth noting that network will send the location and size for initial BWP and PDSCH configuration provided in initialDownlinkBWP via RRC dedicated signalling (i.e., DownlinkConfigCommon) to the UE.

In order to enable UE to receive broadcast on Scell, we propose RAN2 to consider the following solutions:

Proposal 5: For broadcast reception on Scell, RAN2 to consider the following solutions:

  - Solution 1: if proposal 4 is agreed (i.e., network also sends SIB1 of Scell when sending the SIB20 of Scell via RRC dedicated signalling to the UE.), UE determines the broadcast CFR of Scell based on the location and size for initial BWP configured in SIB1 of Scell, and UE determines the PDSCH configuration of MCCH of Scell based on PDSCH configuration provided in initialDownlinkBWP in SIB1 of Scell.

  - Solution 2: UE determines the broadcast CFR of Scell based on the location and size for initial BWP configured in DownlinkConfigCommon of Scell, and UE determines the PDSCH configuration of MCCH of Scell based on PDSCH configuration provided in initialDownlinkBWP in DownlinkConfigCommon of Scell.

  - Solution x: TBD.

#### Q19: Do companies agree that a correction is needed? Do companies have a preference for a solution?

| Company | Correction needed? | Preferred solution | Comments |
| --- | --- | --- | --- |
| Ericsson | Yes | TBD |  |
| Qualcomm | No |  | Current spec is not broken. E.g. for the case of SCell, NW can include optional fields locationAndBandwidthBroadcast-r17 (set to explicit locationAndBandwidth) and pdsch-configMCCH-r17. Solution 1 (adding the whole SIB1 of sCell always in dedicated) is overkill. Solution 2 is not needed as current spec already supports NW to always include optional value and always include explicit value. So, these changes are not needed. |
| CATT | Yes | TBD |  |
| ZTE | maybe no |  | if as QC suggested, current spec already works. |
| vivo | Comments |  | Same comments as Q18. |
| Samsung | No |  | CFR-ConfigMCCH-MTCH can configure location and bandwidth of CFR. No additional solution seems not needed. |
| Huawei, HiSilicon | Yes | Either is OK as long as the issue is solved | Regarding QC’s comment, we think the current spec cannot support Case A and Case C for CFR. Note that according to RRC spec: “Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP and CORESET#0 configured in SIB1.” |
| Nokia | No |  | UE already has all the relevant information from DownlinkConfigCommon. No need to do anything. |
| MediaTek | No |  | Agree with QC that the current spec can work in this case. |

Proposal 6: RAN2 to delete the unnecessary start condition of drx-HARQ-RTT-TimerDL (i.e., if the first HARQ-ACK reporting mode (i.e. ack-nack) is configured).

### NOTE: Proposal 6 is treated in offline #602:

[AT121bis-e][602][MBS-R17] Stage-2 and UP issues (Nokia)

Phase 1 summary and proposals

TBD

References

- R2-2303919	Corrections on MBS SPS configuration			ASUSTeK			CR 38.331

- R2-2303966	Miscellabeous RRC corrections for MBS			Huawei, CBN		CR 38.331

- R2-2302590	Correction to PDSCH Aggregation of MBS SPS		vivo				CR 38.331

- R2-2302522	Remaining issues on Supporting MBS in SNPN		CATT, CBN		discussion

- R2-2303552	Misc correction to TS 38.331 on NR MBS			ZTE, Sanechips		CR 38.331

- R2-2302523	Corrections to TS 38.331					CATT, CBN		CR 38.331

- R2-2302823	CP Corrections for MBS					Samsung			CR 38.331

- R2-2303031	Clarificaition on Key Refresh in MBS			vivo				CR 38.331

- R2-2303619	Corrections for MBS with eDRX and MICO mode		Ericsson			CR 38.304

- R2-2303127	General MBS CR to 38.331					Nokia				CR 38.331

- R2-2304170	Editorial modification to TS 38.331 on NR MBS		MediaTek 			CR 38.331

- R2-2303967	Discussion on the remainning MBS issues			Huawei, HiSilicon		discussion