3GPP TSG-RAN WG2 Meeting #122e	R2-2304933

# Incheon, Korea, 22 - 26 May, 2023

Agenda Item:    7.11.2.1

Source:         Xiaomi

Title:         	Consideration on the control plane issues for multicast reception in RRC_INACTIVE

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 achieved the following agreements on the control plane issues for the multicast reception in the RRC_INACTIVE. [1]

# Service continuity scenarios in RRC_INACTIVE

- Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

- When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

- UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

# Frequency prioritization & NCL

- Frequency prioritization may be provided to the UE for cell reselection for multicast reception in RRC_INACTIVE, detailed mechanism on how to identify the frequency info (e.g., SAI, USD, or frequency info directly provided by network) is FFS.

- No need to define a mechanism other than the frequency prioritization, i.e., per cell based prioritization in cell re-selection, to help UE choose the right cell to camp on.

- The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

# Notification on session state change or data availability

- A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

- Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

- For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

- FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

- No additional enhancement (with regard to enhancements made for ‘deactivation/temp no data’) is needed specifically for enabling UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon session release.

- Legacy group paging (i.e., Rel-17 group paging) can be used to resume UE to RRC_CONNECTED state.

- Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption.

- UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

- When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

In this contribution, we aim to provide our insights on some unresolved matters.

Discussion

  - Service continuity scenarios in RRC_INACTIVE

In the previous meeting, RAN2 discussed on whether to provide the PTM configuration for neighbour cells to enhance the service continuality for multicast reception in RRC_INACTIVE, and agreed to preclude the inter-gNB case as it requires complex design on the network interface from RAN3 perspective. As for the case for intra-gNB cells, it is still FFS whether the network can provide PTM configuration for intra-gNB cell. The agreements are as follows: [2]

=> Serving cell will not provide the PTM configuration of neighbour cells from other gNBs.

=> FFS whether the network can provide PTM configuration for intra-gNB cells.

Obviously, supporting the PTM configuration provision for the intra-gNB case is beneficial for service continuity during mobility. However, considering that there is a lot of PHY and MAC configurations for the multicast reception in RRC_INACTIVE, providing the explicit PTM configurations for the intra-gNB cells seems not flexible and may cause significant signalling overhead. As an alternative, the serving cell can indicate only the neighbour cell list which the PTM configuration of the serving cell can be applied for, instead of providing the explicit PTM configurations for all intra-gNB neighbour cells.

Proposal 1: Serving cell can provide the neighbour cell information (NCL) to indicate the PTM configuration applicability for intra-gNB neighbour cells to support the service continuity during mobility.

  - Frequency prioritization mechanism

In last meeting, RAN2 agreed to have frequency prioritization mechanism to enable UE receiving multicast in RRC_INACTIVE to reselect to a multicast cell as much as possible. As for the mechanism details, some companies proposed to have the FSAI-based frequency prioritization mechanism similar to R17 broadcast. However, different from broadcast service, the multicast reception in RRC_INACTIVE is supported for the congestion scenario and there is no pre-planned area for multicast in RRC_INACTIVE. As such, it is not feasible to use the FSAI-based frequency mechanism for R18 multicast in RRC_INACTIVE.

Instead of this, the network can provide UE with TMGI-Frequency pair(s) for the frequency prioritization of R18 multicast in RRC_INACTIVE. For UE receiving or interested to receive MBS multicast service(s) in RRC_INACTIVE, it may use the TMGI-Frequency pair to prioritize the frequency(s) providing the multicast service in RRC_INACTIVE which the UE is receiving or interested in. Furthermore, as assumed in LTE SC-PTM and NR MBS broadcast, it can be up to UE implementation on how/whether to prioritize an MBS multicast frequency.

Proposal 2: NG-RAN can provide UE with TMGI-Frequency pair(s) for frequency prioritization of R18 multicast in RRC_INACTIVE.

Proposal 3: It’s up to UE implementation on how/whether to prioritize an MBS multicast frequency.

  - Notification on session state change or data availability

PTM configuration update

In the previous meeting, following agreements were achieved for PTM configuration delivery [3].

1. When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases).

2. MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

When the PTM configuration changes, it is necessary to define a PTM configuration change notification to enable the UE to be aware of such change during RRC_INACTIVE without the need for constant MCCH monitoring. The following options can be considered for this:

Option 1: Enhanced group paging. UE upon reception obtains the new PTM configuration via acquiring the MCCH message or resuming the RRC connection.

Option 2: MCCH change notification

If there is always MCCH scheduling in a cell supporting the multicast reception in RRC_INACTIVE, both options work. However, considering the security and privacy concerns, the PTM configuration for some multicast services may not be provided in MCCH in some cases, i.e. no MCCH scheduling in the cell and the MCCH configuration is not available for UE. In this case, the PTM configuration is only provided in dedicated signaling and update notified via group paging.

Therefore, to provide a common solution, we propose to use group paging for PTM configuration change notification. Upon receiving such notification, if the MCCH configuration of serving cell is available to UE, it can stay in RRC_INACTIVE and acquire the MCCH message to obtain the new PTM configuration, otherwise, it resumes the RRC connection to obtain that.

Proposal 4: Group paging is enhanced to indicate the PTM configuration update, and UE upon reception obtains the new PTM configuration via acquiring the MCCH message in RRC_INACTIVE (if the MCCH configuration is available for UE) or resuming the RRC connection.

Multicast session activation

In last meeting, RAN2 agreed to use the enhanced group paging for the multicast session activation notification, but the details of this are FFS.

In legacy, a group notification mechanism is used to inform UEs in RRC IDLE/INACTIVE state when the multicast session is activated by the CN or when the network has multicast session data to deliver. Upon receiving the group paging, all UEs joined to the multicast session shall reconnect to the network.

In R18, for UEs configured with PTM configuration for multicast reception in RRC_INACTIVE, they can stay in RRC_INACTIVE and receive the multicast session when it is activated. However, in some cases, the network may reconsider the UE's RRC state for multicast reception based on the current cell load. In this case, to provide flexible network control, we can consider to have group paging enhanced to indicate the RRC state in which the UE receives the multicast session upon session activation/data transmission resumption.

Proposal 5: Group paging is enhanced to indicate the RRC state in which the UE is expected to receive the multicast session upon session activation or data transmission resumption.

In previous meeting, SA2 sent an LS to RAN2 in R2-2301934 (S2-2303407), which indicated that [4]

-		SA2 agrees that the MBS assistance information for the MBS session sent to NG-RAN consists of an indication that the UE is preferred to be kept in connected when receiving the related MBS session data.

In last meeting, we discussed about the handling of special UE and had the following agreement:

- A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

For the mechanism of session activation notification and RRC state transition, we had the following agreements:

- Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

- UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

- When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

Based on the agreements above, a specific MBS multicast UE can be switched to RRC_CONNECTED via enhanced group paging or UE-specific paging upon session activation. As we understand it, the specific MBS multicast UE can refer to a “special UE”, so there is no additional enhancement required for handling the special UE in this case.

Proposal 6: Network enables “special UE” to resume to RRC_CONNECTED upon session activation via enhanced group paging or UE-specific paging, i.e. no additional enhancement is needed.

Multicast session deactivation

In last meeting, following agreements were achieved for the multicast session deactivation.

- For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

- FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

For the option of enhanced MCCH, it requires the UE to continuously monitor the MCCH during RRC_INACTIVE. Since the PTM configuration for multicast reception may not be updated frequently, this will result in additional UE power consumption for constant monitoring. However, for the enhanced group paging option, there is no extra power consumption issue as the UE needs to monitor it for session activation notifications anyway.

Besides, the option of enhanced MCCH requires mandatory multicast MCCH scheduling. However, in some cases, the PTM configuration may only be provided in dedicated signaling, and there may be no MCCH scheduling in the cell due to security concerns or other reasons

Based on the above, we prefer to use the enhanced group paging for the notification of session deactivation/temporary no data.

Proposal 7: RAN2 agrees to use the enhanced group paging with session state indication to enable UE to stay in RRC_INACTIVE and stop monitoring G-RNTI upon session deactivation/temporary no data.

Conclusions

According to the analysis given above, we have the following observations and proposals:

# Service continuity scenarios in RRC_INACTIVE

Proposal 1: Serving cell can provide the neighbour cell information (NCL) to indicate the PTM configuration applicability for intra-gNB neighbour cells to support the service continuity during mobility.

# Frequency prioritization mechanism

Proposal 2: NG-RAN can provide UE with TMGI-Frequency pair(s) for frequency prioritization of R18 multicast in RRC_INACTIVE.

Proposal 3: It’s up to UE implementation on how/whether to prioritize an MBS multicast frequency.

Notification on session state change or data availability

Proposal 4: Group paging is enhanced to indicate the PTM configuration update, and UE upon reception obtains the new PTM configuration via acquiring the MCCH message in RRC_INACTIVE (if the MCCH configuration is available for UE) or resuming the RRC connection.

Proposal 5: Group paging is enhanced to indicate the RRC state in which the UE is expected to receive the multicast session upon session activation or data transmission resumption.

Proposal 6: Network enables a “special UE” to resume to RRC_CONNECTED upon session activation via enhanced group paging or UE-specific paging, i.e. no additional enhancement is needed.

Proposal 7: RAN2 agrees to use the enhanced group paging with session state indication to enable UE to stay in RRC_INACTIVE and stop monitoring G-RNTI upon session deactivation/temporary no data.

References

- DRAFT_ R2-2304207 R2_121bis MBS session notes (Dawid)_2023-04-26_EOM_rm

- R2-22xxxx Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece.

- R2-22xxxx Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France.

- R2-2301934 LS on the open issues related to RAN WGs in 5MBS_Ph2.