3GPP TSG-RAN WG2 Meeting #122                                              	R2-2304972

Incheon, Korea, 22nd– 26th May, 2023

Source:	CATT

Title:	RAN2 Impact of 5GS network timing synchronization status and reporting

Agenda Item:	7.23.2

Document for:	Discussion and Decision

Introduction

A New WID on Further URLLC Enhancement [1] has been approved, which is used to support the SA2 URLLC enhancements. One of the objectives is about the 5GS network timing synchronization status and reporting. And 2 bullets in the objective have RAN2 impact:

b.	gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3]

Note 1: 	Details of the 5G clock quality information will be decided by RAN3.

c.	UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2]

In last meeting, most of the agreements achieved are about objective 1c. And for objective 1b, it is only agreed that the 5G Clock Quality information is carried in dedicated RRC message of DLInformationTransfer:

Agreements - RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message - Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain - 5G Clock Quality information is carried in DLInformationTransfer message. - FFS if update of event ID is informed to UE by SI modification procedure - For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS. - The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive

In this contribution, we discuss and clarify the content and the delivery method of the 5GS network timing synchronization in RRC_CONNECTED and continue to study the RRC state change mechanism from RRC_IDLE/RRC_INACTIVE.

Discussion

  - The Clock quality information sent to UE in RRC_CONNECTED

In WID it is said that the “5G Clock quality information” will be transmitted by the gNB to the UE in RRC_CONNECTED state. Details of the 5G clock quality information will be decided by RAN3, and the gNB will send this information based on the “clock quality reporting control information” from the core network and the gNB capability.

From the agreed CR of TS23.501 [2] in last SA2#156e meeting, the “clock quality reporting control information” contains the field of “Clock quality detail level” which indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication.

5.27.1.12 Support for network timing synchronization status monitoring [Omitted part] For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, and AF may include CQRCI in its request. CQRCI contains the following fields: - Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. Clock quality metrics refers to one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1 with the exception of PTP clockClass. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon degradation or enable it again upon recovery).

So in the air-interface, the gNB could send one type of the clock quality information to the UE via DLInformationTransfer message. It comprises one of the following values: clock quality metrics or acceptable/not acceptable indication.

For acceptable/not acceptable indication, only 1 bit is needed to indicate whether the clock quality is “acceptable” or “not acceptable” for the UE.

For clock quality metrics, the parameters refer to one or more of the following information: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state. There is a table in the agreed CR of TS23.501 [2] to illustrate what these parameters mean:

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional) Information Name; Description. Synchronization state; Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1).. Clock quality. >> Traceable to GNSS; Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. >> Traceable to UTC; Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. >> Frequency stability; Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2).. >> PTP clockClass; The attribute is defined in Table 5.28.3.1-2 and available under the assumption that the nodes is synchronized using (g)PTP.. >> Clock Accuracy; Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). Parent time source; Describes the primary source the node is currently using, represented by the values "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other".. NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process..

Proposal 1:  Define the Clock quality detail level using CHOICE structure to indicate “acceptable/not acceptable indication” or “clock quality metrics” to UE in air-interface.

Proposal 2:  For “acceptable/not acceptable indication”, 1 bit flag should be defined. And for “clock quality metrics”, the following parameters should be defined using SEQUENCE structure:

- Synchronization state: Value of “Locked”, “Holdover”, or “Freerun”;

- Clock quality, including “Clock accuracy”, “traceability to UTC”,  “traceability to GNSS”, and “Frequency stability”;

- Parent time source: Values of “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand_set”, “other”.

  - RRC state change mechanism from RRC_IDLE/RRC_INACTIVE

The reference report ID in SIB9 may be changed, e.g. the reference report ID in SIB9 may be present in T1 and it may be absent in T2, as highlighted with yellow below. And as the text highlighted with green, either event needs to notify UEs so that interested UEs can know there is new clock quality information available. In addition, the value of reference report ID in SIB9 may be updated, e.g. the reference report ID 1 to the reference report ID 2. Since the change of clock quality information will not occur frequently, to keep the mechanism simple, the existing SI modification mechanism can be reused to notify UEs about the update of reference report ID in SIB9.

5.27.1.12 Support for network timing synchronization status monitoring [Omitted part] gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e., all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB9: - When the network timing synchronization status exceeds any of the pre-configured thresholds, the gNB includes in SIB9 a reference report ID. When the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB stops broadcasting the reference report ID in SIB9. Either event serves as a notification for the UEs reading the SIB9 that there is new TSS information available. - If supported, the UE in the RRC_INACTIVE or RRC_IDLE state compares the reference report ID in SIB9 (or lack of reference report ID) with its locally stored reference report ID to determine whether it has the lastest available clock quality information already or it needs to transit to the RRC_CONNECTED state to retrieve it. A reference report ID consists of a scope of the TSS and an Event ID. The Event ID is an integer indicating that the clock quality information change has been detected by a gNB. A scope of the TSS supports providing TSS information for a group of cells within a single gNB. - If the UE is instructed by AMF (via the Registration procedure, the UE Configuration Update procedure, or via DL NAS TRANSPORT message) to reconnect to the network in the case when the UE determines that the reference report ID has changed, the UE in the RRC_INACTIVE or RRC_IDLE state, if supported by the UE, reconnects to the network. RAN may delay or prioritize UE’s transition to the RRC_CONNECTED state using the UAC framework [28], i.e., UEs are not expected to transition to the RRC_CONNECTED state immediately after determining that the clock quality information has changed and receiving instructions from the AMF. After the UE has reconnected to the network, the gNB uses unicast RRC signaling to provision the clock quality information to the UEs.

Proposal 3: When the reference report ID in SIB9 is changed, including absence to presence, presence to absence, and the update of the value of the reference report ID, existing SI update mechanism can be reused to notify UEs.

In last meeting it is FFS whether the change of RAN timing synchronization status should be informed by the AS layer to NAS layer. SA2 gave the principle as the description highlighted with blue above. The UE should be instructed by AMF to reconnect to the network from either RRC_INACTIVE or RRC_IDLE state, using the NAS Registration or UE Configuration Update procedure. Therefore after comparing and determining the change of reference report ID, the UE AS should inform NAS about the change. Then the NAS layer of the UE triggers corresponding procedures as mentioned above.

Proposal 4: When the UE AS finds there is a change of reference report ID, UE AS should inform UE NAS about the change of RAN timing synchronization status, for both inactive UE and idle UE.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1:  Define the Clock quality detail level using CHOICE structure to indicate “acceptable/not acceptable indication” or “clock quality metrics” to UE in air-interface.

Proposal 2:  For “acceptable/not acceptable indication”, 1 bit flag should be defined. And for “clock quality metrics”, the following parameters should be defined using SEQUENCE structure:

- Synchronization state: Value of “Locked”, “Holdover”, or “Freerun”;

- Clock quality, including “Clock accuracy”, “traceability to UTC”,  “traceability to GNSS”, and “Frequency stability”;

- Parent time source: Values of “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand_set”, “other”.

Proposal 3: When the reference report ID in SIB9 is changed, including absence to presence, presence to absence, and the update of the value of the reference report ID, existing SI update mechanism can be reused to notify UEs.

Proposal 4: When the UE AS finds there is a change of reference report ID, UE AS should inform UE NAS about the change of RAN timing synchronization status, for both inactive UE and idle UE.

Reference

- RP-230754 New WID on NR Timing Resiliency and URLLC enhancements Nokia, Nokia Shanghai Bell

- S2-2306224 Addressing ENs on timing synchronization status reporting