3GPP TSG-RAN WG2 Meeting #122                                   R2-2305402

Incheon, Korea, May 22-26, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Remaining issues for side control information

Agenda item:		7.1.2

Document for: 	Discussion and decision

- Introduction

In this contribution, we discuss the remaining issues in MAC running CR[1],

- Discussion

In the NCR MAC CE parameter list from RAN1, the following FFSs were listed for NCR downlink backhaul link beam indication MAC CE:

FFS: - A/D: This field indicates whether to activate or deactivate indicated TCI state. The field is set to 1 to indicate activation, otherwise it indicates deactivation.

FFS:  The BWP where tci-States is configured.

After RAN1 April meeting, RAN1 has reached the following agreements [2], Per RAN1 agreements, A/D field is not needed, and  RAN1 agreed that the TCI state/SRI for DL/UL beam indication of backhaul link are associated with the active BWP of C-link.

Agreement For the backhaul link beam indication - The TCI state for the DL beam indication of backhaul link is selected from the TCI state list in active DL BWP of C-link. - The TCI state for the UL beam indication of backhaul link is selected from the UL TCI state list in active UL BWP of C-link. - NOTE: For the case of joint TCI state, how to indicate/determine TCI state for the UL beam of backhaul link is subject to RAN2. - The SRI for the UL beam indication of backhaul link is associated with active UL BWP of C-link. Conclusion The explicit backhaul link beam indication is updated by MAC CE indication related to the active BWP. Conclusion No A/D field is included in MAC CE for the backhaul link beam indication.

The conclusion on A/D field does not impact MAC CR, but the association with activate BWP should be captured in MAC spec, so we suggest to update MAC spec to reflect the RAN1 agreement.

Proposal 1: For NCR DL/UL backhaul link beam indication MAC CEs, to clarify the indicated TCI state and SRI are associated with active DL/UL BWP.

In current MAC running CR, 6.1.3.z, NCR Downlink Backhaul Link Beam Indication MAC CE, the field Downlonk TCI state ID is described as:

This field is used to indicate the downlink beam to be used for backhaul link transmission. It contains TCI-StateId (comprising all 7 bits), as specified in TS 38.331 [5], of a TCI State,The length of the field is 7 bits;

However, it is unclear to which TCI state list the “downlink TCI state ID” is referring to. Depends on RRC configuration, there are two cases. If R15 framework is used, the TCI-stateID should refer to the tci-StatesToAddModList field configured in the PDSCH-Config in active BWP. If R17 framework is used, the TCI-stateID should refer to the dl-OrJointTCI-StateToAddModList field configured in the PDSCH-Config in active DL BWP.

In RAN#121bis-e meeting, companies have discussed whether unified TCI state can be considered for backhaul link beam indication MAC CE. In the end, RAN2 agreed that the TCI state configured via unified TCI state can be included in Uplink Backhaul link beam indication MAC CE, used as uplink beam indication, and the description of Uplink MAC CE is updated accordingly. However, without modification to Downlink MAC CE, it is confusing whether the TCI state configured via unified TCI state can also be included in Downlink Backhaul link beam indication MAC CE.

To avoid confusion, we suggest to add description to clarify which TCI-state list the field downlink TCI state ID is referring to, same as for Uplink backhaul beam indication MAC CE.

Proposal 2: For NCR DL backhaul link beam indication MAC CE, to clarify the “downlink TCI state ID” field in NCR Downlink Backhaul Link Beam Indication MAC CE refers to the TCI-state ID configured by tci-StatesToAddModList or dl-OrJointTCI-StateToAddModList.

For proposal and proposal 2, the corresponding draft TP is provided in Annex A.

- Conclusion

In this contribution, proposals and observations are:

Proposal 1: For NCR DL/UL backhaul link beam indication MAC CEs, to clarify the indicated TCI state and SRI are associated with active DL/UL BWP.

Proposal 2: For NCR DL backhaul link beam indication MAC CE, to clarify the “downlink TCI state ID” field refers to the TCI-state ID configured by tci-StatesToAddModList or dl-OrJointTCI-StateToAddModList.

- Reference

[1] R2-230441 MAC running CR

[2] RAN1#112bis chairman notes

- Annex A

NEXT CHANGE

- 6.1.3	MAC Control Elements (CEs)

6.1.3.z	NCR Downlink Backhaul Link Beam Indication MAC CE

The NCR Downlink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.y-1):

-	Downlink TCI state ID: This field is used to indicate the downlink beam to be used for backhaul link transmission. It contains TCI-StateId (comprising all 7 bits), as specified in TS 38.331 [5], of a TCI State configured by tci-StatesToAddModList in the PDSCH-Config in the active DL BWP or by dl-OrJointTCI-StateToAddModList in the PDSCH-Config in the active DL BWP. The length of the field is 7 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.y-1: NCR Downlink Backhaul Beam Indication MAC CE

NEXT CHANGE

#### 6.1.3.y	NCR Uplink Backhaul Link Beam Indication MAC CE

The NCR Uplink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.z-1):

-	Uplink TCI state ID or SRI: This field is used to indicate the uplink beam to be used for backhaul link transmission. If the dl-OrJoint-TCIStateList is configured in the PDSCH-config in the active DL BWP as specified in TS 38.331 [5] and unifiedTCI-StateType is configured with a value joint, this field contains TCI-StateId (comprising all 7 bits) of a Joint TCI State, which is used as the uplink beam indication for backhaul link transmission. If the ul-TCI-StateList is configured in the active UL BWP as specified in TS 38.331 [5], this field contains TCI-UL-State-Id (contained in the 6 rightmost bits) of an UL TCI State, which is used as the uplink beam indication for backhaul link transmission, with the 1 remaining bit set to zero. Otherwise, this field contains an SRI (contained in the 4 rightmost bits) configured in the active UL BWP which is used as the uplink beam indication for backhaul link transmission, with the 3 remaining bits set to zero;

-	R: Reserved bit, set to 0.

Figure 6.1.3.z-1: NCR Uplink Backhaul Beam Indication MAC CE