3GPP TSG-RAN WG2 #122	R2-2305535
Incheon, Korea, May 2023

Agenda Item:	7.5.4.3

Source:	OPPO

Title:	Discussion on configured grant enhancement for XR

Document for:	Discussion, Decision

Introduction

This paper will discuss configured grant enhancement for XR.

Discussion

Retransmission-less CG

In the RAN2#121bis meeting, RAN2 achieved the following agreements on retransmission-less CG.

- There is support to adopt NTN solution for the retransmission-less CG.

- If adopted, RAN2 aims to only consider option1 or option 2:

- Option1: Adapt the NTN solution by disabling the HARQ RTT timer per CG configuration for CG.

- Option2: Adapt the NTN solution by disabling the HARQ RTT timer per HARQ processes for both CG and DG.

- FFS how the solution ensures consistent HARQ operation.

Option 1 restricts the functionality of retransmission-less to specific CG(s) and implies the same operation to all associated HARQ processes of the CG. Option 2 is more like what NTN supports, i.e. the functionality of retransmission-less is for specific HARQ process(s) no matter if it is associated with CG or DG. We understand Option 1 is suitable to what we face here since typically the gNB uses CG to transmit the periodic traffic (e.g. pose/control information). Besides, using the existing LCH mapping restriction (e.g. allowedCG-List) is sufficient to link the pose/control information to the specific CG that enables retransmission-less.

If the initial transmission on a HARQ process of a CG configuration fails, the gNB would schedule a dynamic grant to re-transmit the MAC PDU of the corresponding HARQ process of the CG. We do not see the need to have distinguished handling for the initial transmission and the retransmission of the same MAC PDU, thus, DG for the CG retransmission would also apply the functionality of retransmission-less. In other words, the UE disables the HARQ RTT timer for the uplink grant which is scheduled for retransmission for the MAC entity's CS-RNTI and associated with the HARQ process of a retransmission-less CG.

RAN2 considers adapting the NTN solution by disabling the HARQ RTT timer per CG configuration.

RAN2 considers reusing the existing LCH mapping restriction parameters to indicate LCH associated with pose/control traffic to be mapped into which CG.

RAN2 considers disabling the HARQ RTT timer for an uplink grant which is scheduled for retransmission for MAC entity's CS-RNTI and associated with the HARQ process of a retransmission-less CG.

Multiple CG PUSCH transmission occasions in a CG period

R18 XR supports multiple CG PUSCH occasions in a CG period, which in our view would be used to resolve the issues below.

- Issue 1: Efficient transmission of the XR traffic with large size.

- Issue 2: Non-integer period. Figure 1 shows an example, i.e. 60fps for UL video. In this case, the configured periodicity is 50ms instead of 50/3ms as well as each period distributes 3 PUSCH occasions.

Figure 1: Multiple CG PUSCH occasions in a 50ms-period (60fps)

On the design of the PUSCH occasion, RAN1 is the expert and already achieves good progress on determining TDRA/FDRA table, thus, RAN2 can wait for further RAN1 conclusions. On the HPI determination, the existing HPI calculation formulas below in our view do not suit the XR requirement, i.e. using the existing formula results in the same HPI applied to all CG PUSCH occasions of a CG period and then MAC PDU associated with the same HPI would be flushed unexpectedly.

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

Similarly, the HPI collision also exists in the adjacent CG periods and needs to be resolved (see Figure 2, assuming the HPI of the first PUSCH in a period is determined based on the existing HPI calculation formula and the HPIs of other PUSCH have offset to the HPI of the first PUSCH).

Figure 2: Example of the HPI collision

In RAN1#112bis, RAN1 has some discussion on the HPI determination for XR for which RAN1 achieves the conclusions and leaves the remaining issues.

Agreement

From RAN1 perspective, for determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - FFS whether in formulas above X is outside or inside floor operation, i.e.

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

  - (Working Assumption) The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by Y with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

    - FFS whether X=1 or X= the number of configured PUSCHs in the CG period

    - FFS whether Y =1 or a value larger than 1, e.g. Y=2.

      - FFS: If Y>1, Y is determined based on RRC

    - FFS whether Offset 1= 0 or can be a non-zero value.

      - FFS: If offset1 is non-zero, how offset1 is determined (i.e., based on RRC)

    - FFS whether Offset 2= 0 or can be a non-zero value.

      - FFS: If offset2 is non-zero, how offset2 is determined (i.e., based on RRC or dynamically)

- Note1: The equations will be updated accordingly when FFSs are clarified, e.g., if X=1, remove X; if Y=1, remove Y; if non-zero offset1 or Offset 2 is not supported, remove offset 1 or Offset 2.

- Note2: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

On the remaining issues, we would like to share our views which may help to speed the RAN1’s progress.

- X is the number of configured PUSCHs in the CG period, in order to avoid the HPI collision in the adjacent CG periods.

- The HPI of other CG PUSCH occasion(s) in a period is an offset to the HPI of the first CG PUSCH occasion, in order to avoid the HPI collision in the adjacent PUSCH occasions of a CG period. In other words, the HPI of the remaining configured/valid PUSCHs in a CG period is determined by incrementing the HPI of the preceding PUSCH in the period by Y, where Y equals 1 or is configured by RRC.

- Remove offset1 and offset2. We do not see the necessity of offset1/2, i.e. it is not helpful for the HPI collision in the adjacent CG periods.

- X could be outside the floor operation, in order to avoid the rational number issue.

For the case of multiple CG PUSCH transmission occasions in each CG period, RAN2 considers the following to determine the HPI of each occasion.

- The HPI for the first configured/valid PUSCH in a period is determined by applying the following formula, where X is the number of configured PUSCHs in the CG period.

- HARQ Process ID = [X*floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes; or

- HARQ Process ID = [X*floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

- The HPI of the remaining configured/valid CG PUSCHs in the CG period is determined by incrementing the HPI of the preceding PUSCH in the period by Y, where Y equals 1 or is configured by RRC.

Unused CG occasion indication in UCI

R18 XR agrees to use UCI to dynamically indicate the unused CG PUSCH occasion(s). This kind of UCI is now called UTO-UCI. Based on RAN1’s agreements in RAN1#112bis, UTO-UCI is included in every CG PUSCH and the bitmap in UTO-UCI provides the indication of whether the TO is “unused”.

Agreement

- Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112)

  - FFS details

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Agreement

The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

- FFS: Details including time duration/range

Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

On how to set the value of the bitmap in UTO-UCI, the factors below in our view would be considered.

- The amount of available data buffered at the UE side

This is the main factor for the determination of whether a specific CG is “unused”. Typically, the UE would know the data status based on data volume information associated with the corresponding logical channel. Once the UE knows no available data remains, the UE can set the subsequent CG(s) as unused to save the resources.

- The collision between CG and other information

There exists CG colliding with other PUCCH/PUSCH. From the flexibility perspective, the CG would be set as unused once it is deprioritized, in order to optimize the blind decoding on the resources collided.

As some collision involving CG happens in the PHY layer and should be kept transparent to the MAC layer, we prefer the PHY layer to determine whether a CG occasion is unused as well as the decision in the PHY layer should be transparent to the MAC layer. Besides, if uplink skipping is enabled and no data is available for a CG PUSCH, the MAC follows the existing rule, i.e. the MAC does not generate a MAC PDU if there is no HARQ-ACK/CSI to be multiplexed in the CG PUSCH.

Rely on the physical layer to determine whether a CG occasion is unused or not.

If uplink skipping is enabled and no data is available for a CG PUSCH, the MAC does not generate a MAC PDU if there is no HARQ-ACK/CSI to be multiplexed in the CG PUSCH.

Conclusion

We have the following proposals:

Proposal 1	RAN2 considers adapting the NTN solution by disabling the HARQ RTT timer per CG configuration.

Proposal 2	RAN2 considers reusing the existing LCH mapping restriction parameters to indicate LCH associated with pose/control traffic to be mapped into which CG.

Proposal 3	RAN2 considers disabling the HARQ RTT timer for an uplink grant which is scheduled for retransmission for MAC entity's CS-RNTI and associated with the HARQ process of a retransmission-less CG.

Proposal 4	For the case of multiple CG PUSCH transmission occasions in each CG period, RAN2 considers the following to determine the HPI of each occasion.

•	The HPI for the first configured/valid PUSCH in a period is determined by applying the following formula, where X is the number of configured PUSCHs in the CG period.

-	HARQ Process ID = [X*floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes; or

-	HARQ Process ID = [X*floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

•	The HPI of the remaining configured/valid CG PUSCHs in the CG period is determined by incrementing the HPI of the preceding PUSCH in the period by Y, where Y equals 1 or is configured by RRC.

Proposal 5	Rely on the physical layer to determine whether a CG occasion is unused or not.

Proposal 6	If uplink skipping is enabled and no data is available for a CG PUSCH, the MAC does not generate a MAC PDU if there is no HARQ-ACK/CSI to be multiplexed in the CG PUSCH.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)