3GPP TSG-RAN WG2 Meeting #122	R2-2305889

Incheon, KR, May 22 – 26, 2023

Agenda item:	7.24.2

Source: 	Qualcomm Incorporated

Title: 	Support of Local Cartesian Coordinates in LPP

Document for:		Discussion

# 1.	Discussion

Support for Local Cartesian coordinates were introduced in 3GPP Rel-17 (e.g., TS 23.273 [1], TS 23.032 [2]). The geographical coordinates of a TRP may be provided to an LMF in global coordinates (WGS84) or in local Cartesian coordinates (x/y/z) (e.g., [3]). However, LPP [4] currently supports global coordinates for the TRP locations and UE location estimates only, as discussed in [5].

At RAN2#121bis-e the following working assumption was made [6]:

Agreement:

WA: Support for local Cartesian coordinates for DL-TDOA and DL-AoD positioning is added to LPP.

An email discussion was organized at RAN2#121bis-e, and individual company comments/concerns are documented in [7]. The comments/concerns raised did not identify any technical obstacles for introducing local coordinates also in LPP. The details/encoding are already available from other 3GPP specifications. However, comments/concerns were raised "on the need" or "on use case". Specifically:

Some companies believe that global TRP coordinates would either always be available or can always be obtained from TRP local coordinates. However, local coordinates where introduced to support scenarios where global coordinates can not be surveyed with great accuracy or can not be surveyed at all (e.g., underground stations, indoor locations such as airports, shopping malls, factories, etc.). In such scenarios an application may also not be interested in global coordinates. E.g., it may be sufficient to know where all tools in a factory hall are located relative to some selected origin, instead of knowing each tools geodetic latitude/longitude/altitude, etc.

Some companies also believe that global coordinates can always be derived from local Cartesian coordinates. However, this is generally not possible. Global coordinates in 3GPP are defined in TS 23.032 in a WGS-84 reference frame. Local Cartesian coordinates in 3GPP are defined in TS 23.032 whose origin is expressed by a reference point. The origin may have known WGS-84 coordinates, but not necessarily. Local coordinates can have an arbitrary origin. Positioning would then happen in this local coordinate system. The local coordinates can be converted into global coordinates only when the local origin has a known translation (3D direction) and rotation to a known geodetic ellipsoid point.

Given the fact that local Cartesian coordinates exist in 3GPP specifications, TRP/ARP coordinates may be available in local Cartesian coordinates. Therefore, the proposal is considered as a 'Small Technical Enhancement and Improvement (TEI)' to enable DL-AoD/TDOA also for UE-based mode in scenarios where TRP coordinates are available in local coordinates, or for user cases where global coordinates are not of interest or are not needed.

It is up to UE capability and network to make use of local coordinates. However, we can not see a reason why local coordinates can not be supported also in LPP, given the fact that they were already introduced and are supported in 3GPP specifications.

Proposal 1:	Confirm the 'Working Assumption' from RAN2#121bis-e:
		Support for local Cartesian coordinates for DL-TDOA and DL-AoD positioning is added to LPP.

The draft CR has been updated according to the comments provided in [7]. Local Cartesian Corodinates are introduced according to NRPPa [3], 23.273 [1], and 23.032 [2] as follows:

-	The assistance data IE NR-TRP-LocationInfo allows providing the ARPs via a new IE RelativeCartesianLocation-r18.

-	The IE RelativeCartesianLocation-r18 is defined analogous to IE RelativeLocation (same value/distance ranges) and in agreement with TS 23.032 [2]/TS 29.572 [8].

-	For the IE ReferencePoint, a new CHOICE localOrigin-v18xy is introduced analogous to NRPPa with a CoordinateID-r18 defined in TS 23.032 [2]/TS 29.572 [8].

-	To report the UE location, the two additional GAD shapes Local2dPointWithUncertaintyEllipse and Local3dPointWithUncertaintyEllipsoid are introduced [2].

-	Capabilities are added for DL-AoD and DL-TDOA (analogous to GNSS).

Proposal 2:	Agree the CR in R2-2305891 for "Support of Local Cartesian Coordinates in LPP".

# 2.	Summary

In this contribution, we further discussed the support of local Cartesian coordinates also in LPP. The existence of local Cartesian coordinates in 3GPP specifications clearly means that a network may use local coordinates for the TRP/ARP and/or UE locations. Therefore, local coordinates should not be excluded from LPP. Since the details/encodings are already available from 3GPP specifications ([1],[2],[3],[8]), the required changes are isolated to LPP and are considered as aligning LPP with these mentioned 3GPP specifications.

Proposal 1:	Confirm the 'Working Assumption' from RAN2#121bis-e:
		Support for local Cartesian coordinates for DL-TDOA and DL-AoD positioning is added to LPP.

Proposal 2:	Agree the CR in R2-2305891 for "Support of Local Cartesian Coordinates in LPP".

# References

[1]	3GPP TS 23.273: "Location Services (LCS); Stage 2".

[2]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[3]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[4]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[5]	R2-2303698, "Support of Local Cartesian Coordinates in LPP", Qualcomm Incorporated.

[6]	R2-2304204, "Report from session on positioning and sidelink relay", Session Chair (MediaTek), RAN2 #121bis-e.

[7]	R2-2304289, "Summary of [AT121bis-e][414][POS] Local Cartesian Coordinates", Qualcomm Incorporated.

[8]	3GPP TS 29.572: "Location Management Services; Stage 3".

[9]	R2-2305891, "Support of Local Cartesian Coordinates in LPP [PosLocalCoords]", CR#0447 to 37.355, Qualcomm Incorporated.