3GPP TSG-RAN WG2 Meeting #122	R2-2306365

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.10.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Interference direction for TDM Assistance Information for IDC

WID/SID:	NR_IDC_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution reviews the TDM assistance information that was specified for LTE in order to assess what could also be used for NR in Rel-18.

# 2		Discussion on TDM solution

The root problem of In-Device Co-existence, IDC, is outlined in 3GPP TR 36.816, where the main figure illustrating IDC is copied in and shown in Figure 1.

Figure 1 From: 3GPP TR 36.816 V11.2.0 (2011-12)

Figure 2 From: 3GPP TR 36.816 V11.2.0 (2011-12). 3GPP/NR/LTE transmitter disturb the ISM transceiver in the same manner.

3GPP modems in UE’s may not know exact timing and frequency of non 3GPP RATs like BT and Wi-Fi as the two systems are likely to be implemented in separate hardware without direct communications between them. Coordination between the two systems is therefore coordinated at high level in an application processor without any means to provide exact real time coordination and interference mitigation. Without such coordination, the Signal to Interference Ratio, SIR, will change by a lot from slot to slot in an unpredictable manner making it difficult to ensure good quality of service and optimum MCS (modulation and coding scheme). Resulting low MCS will cause longer periods of active transmit time which in turn increase the likelihood of interference.  The problem is described in:  3GPP TR 36.816 V11.2.0 (2011-12)

Even if exact real time coordination between Wi-Fi and 3GPP modems were implemented, there would still be a coordination problem since timing and coordination is likely controlled by the Wi-Fi RFP for most use cases (except maybe when the UE is used as RFP/WiFi sharing).

The current DRX based solution may only help solve the problems when the UE 3GPP receiver is victim due to Wi-Fi or BT transmissions and may not solve the problem of Wi-Fi and BT being the victim due to UE 3GPP modem transmitter. One aspect to help here is the indication of interference direction: “3GPP receiver is victim” or “ISM modem is victim”.

As a result, Listen Before Talk (LBT) requirements for Wi-FI may be disrupted in case the ACLR leakage from the 3GPP transmitter is strong enough to trig the Wi-Fi transmitter to not start transmission leading to degradation of Wi-Fi quality of service.

In summary, because time synchronisation is not known between WiFi and 3GPP modems, IDC avoidance may not optimal with the given solutions outlined in specifications today. Awareness of UE capabilities of obtaining such timing synchronisation and utilization of same will improve network resource utilisation and avoid IDC impact on signal quality.

In current signaling one can only indicate one TDM pattern that are common to all affected frequencies, this is bit problematic as it seems likely that only part of frequencies would be having same TDM patterns affecting the frequency.

If the 3GPP system detects interference caused by applications or services (e.g. WiFi/BT) that use the ISM band may the 3GPP system not in all cases be aware of the source of the interference, which is illustrated in figure 3. It can be due to internal (In device) use of the ISM band or external use between devices not related to the 3GPP devices at all; like use of WiFi between a laptop and a WiFi access point at home or at work. Or use of BT between a PC and a headset. RF isolation between devices used in close vicinity to each other is in many use cases, more or less, the same as the achievable RF isolation between a 3GPP modem and a WiFI/BT modem integrated into the same device (smartphone).

Figure 3: Illustration of multiple interference sources, both device internal and device external.

It is crucial to exactly know the root cause of the interference and the type of aggressor that is the root cause of the detected interference, as the mitigation techniques that can be applied to solve co-existence problems between WiFi/BT and 3GPP systems is very different depending on whether the aggressor is external or internal to the UE.

So we think it would be beneficial to get more detailed information about what is the source of interference i.e. whether it is such that UE can determine accurate timing of interference (e.g. UE has ability to share timing information between ISM / 3GPP) or whether interference is something UE cannot know timing (e.g. external BT transmitter). This way it is easier for NW to know whether DRX solution will actually be able to solve potential issues without excessive safety margings when providing DRX configuration. Especially as the external problems are very challenging to be solved by DRX solution it would be good if UE could indicate this to the NW so that NW immediately knows that it is better to try to solve the issue with FDM solution if feasible.

Proposal 1: Interference direction should be able to indicate separately whether the interference is coming in such way that UE can derive accurate timing information (e.g. internal vs. external interference) of the interference

As UE may have concurrently multiple sources of interference it would be also helpful if one could indicate per interference source interference direction i.e. multiple TDM patterns

Proposal 2: Enable indicating multiple TDM patterns to the NW (with the interference direction information)

In Device coexistence related parameters that can be exchanged between a 3GPP modem (UE) and the 3GPP network also need to be available on the AT interface within the UE. This enables other modems (WiFi, BT or any other non 3GPP system) to reallocate resources in frequency and time domain or change RF front end tuning or configuration such that coexistence problems can be mitigated or minimized. Above, it is assumed there are separate systems communicating, WiFi system, 3GPP system and AP system. This may all be SW components within the same HW system and not separate HW systems.

Proposal 3: Inform CT1 with a LS on the need to enhance AT commands to enable IDC information transfer between application processor and different radios of the UE

# 4	Conclusion

Proposal 1: Interference direction should be able to indicate separately whether the interference is coming in such way that UE can derive accurate timing information (e.g. internal vs. external interference) of the interference

Proposal 2: Enable indicating multiple TDM patterns to the NW (with the interference direction information)

Proposal 3: Inform CT1 with a LS on the need to enhance AT commands to enable IDC information transfer between application processor and different radios of the UE