3GPP TSG-RAN WG2 Meeting #123                             	R2-2307758

Toulouse, France, August 21–25, 2023

Agenda item:	7.11.2.2

Source:	Samsung

Title:	UP Aspects for Multicast Reception in RRC_INACTIVE

Document for:	Discussion & Decision

Introduction

This document discusses the following UP issues for multicast reception in RRC_INACTIVE:

- SPS support

- Multicast DRX operation

- L2 operation during state transition

- L2 operation during cell reselection

- Issues related to RAN1 LS R1-2306243.

Discussion

MAC operation for RRC_INACTIVE

SPS

In RAN2#121bis-e meeting, RAN2 could not reach the consensus on SPS support due to the diverged company views. The main argument was whether SPS in RRC_INACTIVE is beneficial and whether SPS in RRC_INACTIVE is feasible.

We see the benefit is clear that the PDCCH transmission for every single downlink transmission is not necessary. It not only saves the PDCCH resource but also reduces the risk of PDCCH misdetection. Once a multicast SPS is activated, the UE does not need to worry about missing PDCCH at all. For periodic traffic like streaming service and real-time service, resource scheduling based on SPS will be useful. Moreover, SPS does not affect DRX Active Time, so using SPS is beneficial from UE power saving perspective.

During the discussion in RAN2#121bis-e meeting, there was a concern on the risk of missing PDCCH. However, this missing PDCCH problem is more severe for dynamic grant based on G-RNTI. In dynamic scheduling, missing PDCCH could happen at every single grant allocation. In contrast, SPS does not have a PDCCH issue after the activation. Activation does not occur so frequently, so gNB can send the activation message multiple times to minimize the missing PDCCH probability. Thus, we can say that SPS is a safer way to deliver multicast data in RRC_INACTIVE than dynamic grant, so we propose to support multicast SPS in RRC_INACTIVE.

Proposal 1: Multicast SPS in RRC_INACTIVE is supported.

If the multicast SPS is supported in RRC_INACTIVE, how to activate it could be the next issue. We see that it should be the same as RRC_CONNECTED by G-CS-RNTI except no HARQ feedback for activation/deactivation of SPS. We see that gNB will try to minimize missing PDCCH indicating activation/deactivation, so it is not a problem.

Proposal 2: Multicast SPS in RRC_INACTIVE is activated and deactivated by G-CS-RNTI, same as RRC_CONNECTED without HARQ feedback.

DRX

In RAN2#122bis-e meeting, RAN2 agreed that multicast DRX defined for RRC_CONNECTED will be used also for RRC_INACTIVE. Since HARQ feedback is not supported at all in RRC_INACTIVE, drx-RetransmissionTimerDLPTM will not be started and the UE may not stay awake to receive the retransmission.

However, drx-InactivityTimerPTM is started when the UE receives a new transmission. If the time duration of drx-InactivityTimerPTM is sufficiently long, the UE will have sufficient time to receive the retransmission. This can be handled and should be considered by a sensible NW implementation.

Proposal 3: In RRC_INACTIVE, drx-HARQ-RTT-TimerDLPTM and drx-RetransmissionTimerDLPTM are not used.

L2 operation during state transition

The next issue is whether to support HARQ continuation for MBS multicast reception during state transition. Since there is no uplink transmission in RRC_INACTIVE, all multicast MRBs configured in RRC_INACTIVE much be UM MRB whose lossless operation is not guaranteed. When the UE enters RRC_INACTIVE, MAC reset with DL HARQ buffer flushing is performed. Considering MRBs are a UM bearer and RRC Release does not frequently happen. We do not need any special handling for multicast MRB at state transition.

Moreover, the HARQ continuation may not be always feasible. In case that the multicast CFR for RRC_CONNECTED and that for RRC_INACTIVE are different, HARQ continuation is not possible, since the  same packet transmitted via the same HARQ process and the same NDI are too strong NW restriction. Thus, we can have a single procedure

Proposal 4: HARQ continuation for multicast reception during RRC state transition is not supported. DL soft buffer is flushed (no specification impact).

Similarly, MRB continuation is possible at the state transition only if all MRB configurations does not change. For the case that MRB configuration is change at the state transition, PDCP re-establishment is necessary anyway. Thus, it is ok to suspend MRB and re-establishment when the UE enters RRC_INACTIVE.

Proposal 5: Multicast MRB is suspended and immediately re-established when the UE enters RRC_INACTIVE.

L2 operation and PDCP synchronization during mobility

In RAN2#121bis-e meeting, RAN2 discussed the case that the source and the target cells have PDCP synchronization. The issue here is whether the MRB needs to be re-established at cell reselection in this case. Even if the PDCP SN is synchronized between cells, exact scheduling time can be different for those cells. If the scheduling time difference is longer than t-reordering timer duration which should be set small in RRC_INACTIVE, PDCP synchronization does not guarantee the delivery. It means that there is no need to keep the PDCP.

Similar to the state transition, RAN2 should first consider the case that PDCP is not synchronized. In this case, MRB should be re-established and MAC entity should be reset for flushing DL soft buffer. Then, it would be better to have a common procedure irrespective of PDCP synchronization.

Proposal 6. At cell reselection with service continuity for multicast (i.e. same multicast service is provided in the target cell), MRB is re-established and DL soft buffer is flushed.

Discussion on RAN1 LS R1-2306243

RAN1 LS R1-2306243 replied that some RAN2 assumptions made in RAN2#121bis-e are confirmed. Although some issues are still under discussion in RAN1, we can confirm the current RAN1 conclusion, so we propose:

Proposal 7. RAN2 confirms DCI 4_0 is used for multicast MCCH in RRC_INACTIVE.

Proposal 8. RAN2 confirms DCI_4_1 is used for multicast MTCH in RRC_INACTIVE.

Proposal 9. RAN2 confirms Slot-level repetition is supported for RRC_INACTIVE.

Proposal 10. RAN2 confirms separate CSSes for multicast MCCH and multicast MTCH can be configured.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: Multicast SPS in RRC_INACTIVE is supported.

Proposal 2: Multicast SPS in RRC_INACTIVE is activated and deactivated by G-CS-RNTI, same as RRC_CONNECTED without HARQ feedback.

Proposal 3: In RRC_INACTIVE, drx-HARQ-RTT-TimerDLPTM and drx-RetransmissionTimerDLPTM are not used.

Proposal 4: HARQ continuation for multicast reception during RRC state transition is not supported. DL soft buffer is flushed (no specification impact).

Proposal 5: Multicast MRB is suspended and immediately re-established when the UE enters RRC_INACTIVE.

Proposal 6. At cell reselection with service continuity for multicast (i.e. same multicast service is provided in the target cell), MRB is re-established and DL soft buffer is flushed.

Proposal 7. RAN2 confirms DCI 4_0 is used for multicast MCCH in RRC_INACTIVE.

Proposal 8. RAN2 confirms DCI_4_1 is used for multicast MTCH in RRC_INACTIVE.

Proposal 9. RAN2 confirms Slot-level repetition is supported for RRC_INACTIVE.

Proposal 10. RAN2 confirms separate CSSes for multicast MCCH and multicast MTCH can be configured.