3GPP TSG-RAN WG2 Meeting #123	R2-2308045

Toulouse, France, 21 - 25 Aug, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RAN1 and RAN3 led NES techniques

Agenda Item:	7.3.6

Document for:	Discussion and decision

Introduction

In RAN1#113 meeting, the techniques in spatial and power domains for NES was discussed and the following agreements were reached [1]:

Agreement For N>=1 CSI reporting corresponding to N out of L sub-configurations in one reportConfig where each sub-configuration corresponding to an SD adaptation pattern or/[and] a powerControlOffset value, - For A-CSI and SP-CSI on PUSCH report, support DCI-based triggering - For A-CSI-RS, CPU and CSI-RS resource/port counting depend on N indicated sub-configurations - FFS: How to do the counting - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations - For SP-CSI on PUCCH report, support MAC-CE-based triggering - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations Note: UE complexity reduction is not precluded - For DCI-based triggering, - Alt 1: A triggering state corresponding to N sub-configurations is indicated via the existing CSI request field in DCI. Different triggering states could represent different subsets of L sub-configurations. - The DCI is UE specific (in this case, legacy DCI format applies) - For MAC-CE based triggering - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE) - Only one MAC CE is used for this triggering

In RAN3 #119b meeting, the paging enhancement was discussed, with the following agreements [2]:

Enhancements on restricting paging in a limited area: It’s up to gNB’s implementation to decide to which UEs should apply the paging enhancement technique. The paging enhancement technique is applicable for UEs in RRC inactive state. Introduce the recommended SSB beam list in the F1AP paging message. Introduce a list of last few served SSB beam s/recommended SSB beam list as paging assistance information to the gNB-CU in UE CONTEXT RELEASE COMPLETE message over F1AP. Send a LS to RAN2 asking feedback containing the agreements for RRC inactive UE, and to SA2 asking feedback for RRC idle UE. The above agreements can be revisited based on reply from RAN2.

Also, RAN3 sent a LS R3-232084 [3] with the following question to RAN2:

According to above agreements, paging for UEs in RRC INACTIVE state may be restricted to a limited number of beams within a cell. RAN3 would respectfully like to ask RAN2 the following question: Q1: Whether RAN3’s agreements have any impact on, or could be impacted by, TS 38. 304 clause 7 Paging, e.g., related to the following paragraph. TS 38.304 clause 7.1: “In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

In SA2#157 meeting, SA2 the replied LS S2-2307984 [4] to RAN3 with the following answers:

Answer: SA2 does not see any showstoppers from CN perspective for the enhancements to restrict paging in a limited area for UEs in RRC IDLE. SA2 requests RAN3 to inform SA2 when the related CRs are agreed in order to proceed with alignment CRs in specifications under SA2 control. SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures. and ask the following questions: Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation? Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

In this paper, we discuss the signalling design of the MAC-CE for spatial/power domains enhancements and paging enhancement in a limited area.

Discussion

- 2.1 CSI report

In previous meetings, RAN1 has discussed the NES enhancements for spatial domain and power domain. The intention is to let the UE measure and report CSI corresponding to different sub-configurations. Each sub-configuration contains different parameters, including spatial domain adapatation pattern parameters and/or power offsets.

In RAN1#113 meeting, RAN1 agreed to support the CSI reporting of N out of L sub-configurations in one reportConfig. In particular, for SP-CSI report on PUCCH, MAC-CE based triggering has been agreed. The L sub-configurations will be provided to the UE via dedicated RRC signalling, and the details are pending on the RAN1 parameter list to be approved. On top of that, RAN2 needs to determine the signalling design of the MAC CE for selecting the N sub-configurations, as indicated in RAN1 agreement:

- For MAC-CE based triggering - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE) - Only one MAC CE is used for this triggering

In TS 38.321 clause 6.1.3.16 [5], the SP CSI reporting on PUCCH Activation/Deactivation MAC CE is defined. It has a fixed size of 16 bits with 5 bits reserved:

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Whether the reserved 5 bits can be utilized depends on the total number of sub-configurations. RAN1 has agreed in RAN1 #112bis-e that “The maximum value of N and L are subject to UE capability”, meaning that there won’t be too many sub-configurations. Also, supporting more sub-configurations will increase UE complexity.

As long as L does not exceed 5, a bitmap of 5 bits can indicate the selected sub-configurations. If the total number of sub-configurations exceeds 5, an alternative is to use RRC signalling to configure the possible combinations of sub-configurations and use the 5 reserved bits to indicate the index of the combination, in this way, at most 2^5 combinations can be supported. On this basis, we think there is no need to introduce a new MAC CE.

Proposal 1: The reserved bits in SP CSI reporting on PUCCH Activation/Deactivation MAC CE are used to activate/deactivate sub-configurations.

- 2.2 Paging enhancement

The paging enhancement technique is to page a UE in only part of SSB beams within a cell, instead of in all SSB beams within a cell. This technique is mainly applicable to stationary UEs. The legacy paging assumes that the paging message is repeated in all SSB beams within a cell, as follows:

TS 38.304 clause 7.1:

“In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

TS 38.304 clause 7.2.1

The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframes or OFDM symbols) where PEI can be sent (TS 38.213 [4]). In multi-beam operations, the UE assumes that the same PEI is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the PEI is up to UE implementation.”

Restricting paging to a limited number of SSB beams may lead to paging failure, i.e. the UE may miss the paging message if it only monitors on other SSB beams. SA2 also worries that how to solve such problem with existing procedure.

However, in our understanding, such failure can be avoided by network implementation. A reasonable paging staggery of NW could be like the following:

- NW pages the UE in a limited number of SSB beams within a cell.

- If the UE does not respond to the paging in a pre-set time, NW will page the UE in all SSB beams within the cell.

This is similar with the sub-area paging in current spec, as described in TS 23.502 as follows:

#### 4.2.3.3 Network Triggered Service Request < Irrelevant text skipped> Paging strategies may include: - paging retransmission scheme (e.g. how frequently the paging is repeated or with what time interval); - determining whether to send the Paging message to the (R)AN nodes during certain AMF high load conditions; - whether to apply sub-area based paging (e.g. first page in the last known cell-id or TA and retransmission in all registered TAs).

By implementing such paging staggery, the UE behaviour can keep unchanged and there is no RAN2 impact. Thereby the paging enhancement technique could also apply to legacy UEs.

Proposal 2: There is no RAN2 impact of paging enhancement. The paging failure could be avoided by proper NW implementation.

The draft reply LS to RAN3 is provided in the appendix.

Conclusion

In this paper, we discussed the CSI reporting signal design in spatial and power domains for NES, paging enhancement in a limited area and made the following proposals:

Proposal 1: The reserved bits in SP CSI reporting on PUCCH Activation/Deactivation MAC CE are used to activate/deactivate sub-configurations.

Proposal 2: There is no RAN2 impact of paging enhancement. The paging failure could be avoided by proper NW implementation.

References

- R1-2306352, Report of RAN1#113 meeting, ETSI MCC

- R3-232502, Report of 3GPP TSG RAN3 meeting #119-bis-e, ETSI MCC

- R3-232084/R2-2304627, LS on the enhancements to restricting paging in a limited area

- S2-2307984, Reply LS on the enhancements to restricting paging in a limited area

- TS 38.321 v17.5.0

- Annex. [Draft] Reply LS

Title:	[Draft] Reply LS on the enhancements to restricting paging in a limited area

Response to:		R3-232084/R2-2304627

Release:	Rel-18

Work Item:	Netw_Energy_NR-Core

Source:	Huawei [will be RAN2]

To:	RAN3

Cc:	SA2

Contact Person:

#### Name:	Lili Zheng

E-mail Address:	zhenglili4@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

RAN2 would like to thank RAN3 for their LS on the enhancements to restricting paging in a limited area.

RAN2 discussed Q1 and would like to provide the following answer.

Q1: Whether RAN3’s agreements have any impact on, or could be impacted by, TS 38. 304 clause 7 Paging, e.g., related to the following paragraph.

TS 38.304 clause 7.1:

“In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

RAN2's Answer:

No RAN2 impact is identified. The UE behavior does not need to be changed, and the paging failure can be avoided by proper NW implementation.

# 2. Actions:

To RAN3

ACTION: 	RAN2 kindly asks RAN3 to take above information into account.

# 3. Date of Next RAN2 Meetings:

3GPP TSG RAN WG2#123-bis	09 - 13 October, 2023		Xiamen, China

3GPP TSG RAN WG2#124	13 – 17 November, 2023		Chicago, United States