3GPP TSG-RAN WG2 Meeting #123	R2-2308301

Toulouse, France, August 21-25, 2023

Agenda Item:	7.3.2

Source:	CMCC

Title:	Discussion on Cell DTX/DRX

Document for:	Discussion

Introduction

In RAN2#122 meeting, the following agreements were made for cell DTX/DRX [1]:

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

There are still some open issues of cell DTX/DRX design, in this contribution, we provide our further considerations on CHO including target MCG and candidate SCGs.

Discussion

Alignment of cell DTX/DRX and UE C-DRX

To accelerate the discussion of cell DTX/DRX design, there’s a post email discussion, and one controversial issue is whether to support the alignment of cell DTX/DRX and UE C-DRX or up to NW implementation.

From our point of view, without the alignment of cell DTX/DRX and UE C-DRX, the power saving gain and the UE experience will be reduced.

For example, as shown in Figure 1, different UEs have different or same DRX cycles cycle but different start offsets, then these UEs will be scheduled dispersedly in time. When cell DTX/DRX are configured, the active time of some UEs will fall out of on-duration time of gNB, then such UEs will not be scheduled, unless a longer on-duration timer of cell DTX/DRX is configured, while it may cause the power saving gain of cell DTX/DRX to be limited.

Figure 1

Besides, if it is left to NW implementation, it’s complex for the NW to provide a proper configuration considering all UEs.

Observation 1: without alignment of cell DTX/DRX and UE C-DRX the power saving gain and the UE experience will be reduced, and the complexity for the NW to provide a proper configuration considering all the UEs is increased.

From our point of view, the alignment of cell DTX/DRX and UE C-DRX is to ensure that UE may have more opportunities for PDCCH monitoring when the cell DTX/DRX is activated, but not limit the UE’s on-duration/active time all in cell’s DTX/DRX active time. The alignment may provide more flexibility for the network scheduling.

Proposal 1: It’s proposed to introduce the alignment of cell DTX/DRX and UE C-DRX to provide more opportunities for UE’s PDCCH monitoring.

Since both RAN1 and RAN2 agreed to use the common L1 signalling for the cell DTX/DRX activation/deactivation [2][3], the alignment is only performed when UE receives the common L1 signalling. As shown in Figure 2, when gNB decides to start cell DTX/DRX, it sends common L1 signalling to inform UE about the activation of cell DTX/DRX, and along with the dynamic activation, all UEs will update the DRX parameters such as DRX cycle, drx-startoffset, or drx-slotoffset to a common start offset, common DRX cycle, common slot offset value configured by the network, or the common parameters share same value with cell DTX/DRX. As shown in Figure 2, all UE use the same offset and cycle as cell DTX/DRX, and the on-duration timer can share the same value or different values.

By this dynamic way, gNB can turn on/off the cell DTX/DRX flexibly, and also align the active time of gNB and UEs, therefore, the power saving gain can be increased, and it can react to dynamic UE traffic varying.

Figure 2 Dynamic alignment of UE DRX with cell DTX/DRX

Proposal 2: Both cell DTX/DRX configuration and UE C-DRX are provided to UE in RRC signaling, the UE performs the configuration alignment if it receives the L1 activation signaling, else the UE will use the C-DRX configuration as legacy.

Based on RAN1’s LS, the signalling is used to indicated several cells’ DTX/DRX activation/deactivation, but it is not clear whether it’s separate indication for cell DTX and cell DRX or share the same L1 indication. Since it is analysed in the SI phase that the power consumption of downlink is much higher than uplink, therefore, cell DTX may bring more power saving gains than cell DRX [4]. Therefore, we think independent DTX and simultaneous DTX/DRX can be considered. And for the case both DTX/DRX are configured simultaneous, there’s no need to activate DTX and DRX separately, the shared indication can be used, which can reduce the DCI payload. Whether separate indication or shared indication is used can be configured by RRC.

What’s more, in case cell DTX and cell DRX are configured simultaneously, though it’s possible to configure cell DTX and cell DRX with different values for the same parameter, such as periodicity, start slot/offset and on duration, it will bring more scheduling complexity to the network, and reduce the power saving gain. Therefore, we prefer to configure cell DTX and cell DRX with the same values once they are configured simultaneously.

Proposal 3a: Cell DTX can be configured independent of cell DRX.

Proposal 3b: Cell DTX and cell DRX can be configured together, and the same parameter values are shared.

Proposal 4: The L1 activation/deactivation of cell DTX and cell DRX can be separate one or shared the same one, which can be configured by RRC.

Besides, another issue for basic cell DTX/DRX design is whether to support multiple cell DRX/DTX configurations. The motivation of supporting multiple cell DTX/DRX configurations is for the adaptation of the dynamic traffic changes, for a better power saving gain. To support multiple cell DRX/DTX configurations, multiple configurations may be provided to the UE, but from UE point of view, only one cell DRX/DTX configuration is running at a time. Since we have agreed to use L1 activation/deactivation signalling for a dynamic activation of cell DTX/DRX, the requirements to support multiple cell DTX/DRX configurations may be depend on whether the traffic mode in a NES cell may change frequently. If not, single cell DTX/DRX configuration with L1 activation/deactivation is enough.

Proposal 5: Single cell DTX/DRX configuration with L1s activation/deactivation can be used as baseline, whether to support multiple cell DTX/DRX configurations may depend on the discussion of the NES cell’s traffic mode.

SR transmission

Following are the agreements related to SR transmission:

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

In last meeting, the discussion of SR transmission comes with the discussion of emergency call supporting, and no explicit agreements was made for SR transmission.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

sr-ProhibitTimer, sr-TransMax and SR_COUNTER are configured per SR configuration [5]. And we a SR is triggered, sr-ProhibitTimer is started, and during the sr-ProhibitTimer running, there’s no new SR transmission. Once SR_COUNTER= sr-TransMax, UE will initiate a Random Access procedure

4> if SR_COUNTER < sr-TransMax: 5> instruct the physical layer to signal the SR on one valid PUCCH resource for SR; 5> if LBT failure indication is not received from lower layers: 6> increment SR_COUNTER by 1; 6> start the sr-ProhibitTimer. 5> else if lbt-FailureRecoveryConfig is not configured: 6> increment SR_COUNTER by 1. 4> else: 5> notify RRC to release PUCCH for all Serving Cells; 5> notify RRC to release SRS for all Serving Cells; 5> clear any configured downlink assignments and uplink grants; 5> clear any PUSCH resources for semi-persistent CSI reporting; 5> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

Therefore, the related timer will not be started. But, there’re two cases need further discussion:

Case 1: one SR_COUNTER reaches sr-TransMax-1 at/near the end of the cell’s active period and the UE does not receive the gNB’s scheduling until sr-ProhibitTimer rues out, due to the cell enters non-active period. Based on the current spec design, UE may initiate RACH procedure during the non-active period when the sr-ProhibitTimer runs out, and based on the current agreements, RACH procedure is not affected by cell DTX/DRX. However, the gNB may scheduling the UE in the next active period, therefore, the RACH should be avoided.

Case 2: UE’s sr-ProhibitTimer is overlapping with active period and non-active period, for example, the SR is transmitted near the end of the active period, and the sr-ProhibitTimer runs out in the non-active period, and SR_COUNTER is creased, which may also cause the unnecessary RACH procedure like in Case 1. In our view, for this case, the sr-ProhibitTime can be suspended and re-start when the cell enters active-peirod.

Figure 3

Proposal 6: For SR transmission, the following two cases can be further discussed:

Case 1: one SR_COUNTER reaches sr-TransMax-1 at/near the end of the cell’s active period and the UE does not receive the gNB’s scheduling due to the cell enters non-active period.

Case 2: UE’s sr-ProhibitTimer is overlapping with active period and non-active period.

Conclusions

In this contribution, we analyse the open issues of ell DTX/DRX. Following is our observations and proposals.

Observations:

Observation 1: without alignment of cell DTX/DRX and UE C-DRX the power saving gain and the UE experience will be reduced, and the complexity for the NW to provide a proper configuration considering all the UEs is increased.

Proposals:

Proposal 1: It’s proposed to introduce the alignment of cell DTX/DRX and UE C-DRX to provide more opportunities for UE’s PDCCH monitoring.

Proposal 2: Both cell DTX/DRX configuration and UE C-DRX are provided to UE in RRC signaling, the UE performs the configuration alignment if it receives the L1 activation signaling, else the UE will use the C-DRX configuration as legacy.

Proposal 3a: Cell DTX can be configured independent of cell DRX.

Proposal 3b: Cell DTX and cell DRX can be configured together, and the same parameter values are shared.

Proposal 4: The L1 activation/deactivation of cell DTX and cell DRX can be separate one or shared the same one, which can be configured by RRC.

Proposal 5: Single cell DTX/DRX configuration with L1s activation/deactivation can be used as baseline, whether to support multiple cell DTX/DRX configurations may depend on the discussion of the NES cell’s traffic mode.

Proposal 6: For SR transmission, the following two cases can be further discussed:

Case 1: one SR_COUNTER reaches sr-TransMax-1 at/near the end of the cell’s active period and the UE does not receive the gNB’s scheduling due to the cell enters non-active period.

Case 2: UE’s sr-ProhibitTimer is overlapping with active period and non-active period.

References

- R2_122_meeing_report;

- R1-2306246 Reply LS on Cell DTX/DRX activation/deactivation

- R2_121bis_meeing_report;

- 3GPP TR 38.864 Study on network energy savings for NR;

- 3GPP TS 38.321 Medium Access Control (MAC) protocol specification.