3GPP TSG-RAN WG2 Meeting #123	R2-2308552

# Toulouse, France, May 21st – 25th, 2023                             resubmission of R2-2305917

Agenda Item:	7.11.2.1 Control plane

Source: 	Ericsson

Title:  	MBS multicast and UE power saving

Document for:	Discussion and Decision

# Introduction

In this contribution MBS multicast and UE power saving (eDRX and MICO mode) is discussed further.

# Background

DRX and eDRX in RRC_INACTIVE and RAN group paging

The gNB configures the DRX or eDRX cycle the UE uses in RRC_INACTIVE to monitor RAN paging, which includes both unicast and group paging:

- gNB triggers unicast paging when there is new unicast data

- gNB triggers group paging when there is new multicast data or the session is activated

For RAN paging the gNB uses the DRX or eDRX cycle configured in the RRCRelease message (SuspendConfig):

- Rel-16: DRX cycle 		≤ 2.56 sec

- Rel-17: short eDRX cycle 	≤ 10.24 sec

- Rel-18: long eDRX cycle	≤ 1024 Hyper Frames (10 sec):

For group paging the gNB needs to consider the different DRX cycles configured for the group members, which can be UE specific or RRC state dependent:

- The gNB uses the DRX cycle used in RRC_IDLE to perform CN group paging.

- The gNB uses the DRX cycle used in RRC_INACTIVE to perform RAN group paging.

- The DRX cycle configured in RRC_IDLE and RRC_INACTIVE can be different.

- The DRX cycle in RRC_IDLE is UE specific when the UE specific DRX via NAS is configured.

- The DRX cycle in RRC_INACTIVE is UE specific.

- The UE in RRC_INACTIVE may receive both RAN and CN (group) paging.

The gNB does not use the eDRX when configured in the UE for RRC_IDLE, because that would introduce too much latency. The gNB is also not informed if a group member has eDRX configured in RRC_IDLE (MULTICAST GROUP PAGING message does not include that information), i.e. the gNB cannot use eDRX for CN group paging.

The gNB can only configure an eDRX in RRC_INACTIVE, when the UE has been configured with an eDRX in RRC_IDLE. The UE can request an eDRX cycle length and PTW size, and the AMF then configures a negotiated eDRX cycle/PTW in the UE. Only a UE that is delay tolerant will request and may be configured with a long eDRX cycle.

RAN group paging and eDRX is discussed further in chapter 3.

# Discussion

## Rel-17: MBS and UE power saving

Concerning eDRX and MICO mode in RRC_IDLE it was discussed in RAN2#121bis-e that:

- If the UE is configured with eDRX/MICO mode, it is up to UE implementation to decide to receive MBS broadcast (no specification impact)

- If the UE is configured with eDRX/MICO mode and the UE joins a multicast session then the UE may miss group paging when the session is activated:

  - The gNB can keep the UE in RRC_CONNECTED when the UE has joined a multicast session and is configured with eDRX/MICO mode, until the UE leaves the session or the session is released (no specification impact).

Observation 1: In case the UE prefers UE power saving over MBS multicast reception then the UE can decide to not join a multicast session when the UE is configured with eDRX/MICO in RRC_IDLE.

## Rel-18: MBS and UE power saving

Short description

SA2 introduced enhancements for MBS and UE power saving in Rel-18.

An example of the use case and the steps involved:

- RedCap UEs are configured with eDRX or MICO mode in RRC_IDLE for power saving

- RedCap UEs use MBS broadcast or multicast at pre-configured times e.g. for software download

- Via service announcement “scheduled activation time(s)” are configured in the UE

- At the scheduled activation time the UE wakes- up from deep sleep and receives MBS data

- Due to data-inactivity the UE returns to eDRX/MICO mode in RRC_IDLE

- eDRX and MICO mode remain configured in the UE during the steps above

For further information see 23.247:

- 6.16	Support of MBS data reception for UEs using power saving functions

- 7.3.8	Broadcast MBS procedures for UEs using power saving functions

- 7.2.10	Multicast MBS procedures for UEs using power saving functions

### About the start time, stop time and scheduled activation time

In Rel-17 the UE is always configured with a start and stop for a broadcast session. Optionally the UE can also be configured with a start and stop time for a multicast session (when start time is not configured the UE assumes “now”):

MBS broadcast:

- Start time: data transmissions start

- Stop time: data transmissions stop

MBS multicast:

- Start time:

  - the session has been created and the UE may join (without being rejected)

  - the session may be activated at the start time or later

  - data transmissions start after the session has been activated

- Stop time:

  - data transmissions stop

  - the session may be activated/deactivated multiple times between start and stop times

  - after a session has been deactivated the data transmissions stop until the session is activated again

  - data transmissions may stop (temporarily) without session deactivation

In Rel-18 a series of scheduled activation times can be signalled via service announcement for both broadcast and multicast sessions:

Scheduled activation time:

- MBS broadcast:

  - data transmissions start

- MBS multicast:

  - the session is activated at the scheduled activation time

  - data transmissions start

### MBS broadcast and power saving

When the UE is in RRC_IDLE with eDRX or MICO mode activated, then the UE can decide to wake-up at the scheduled activation time, similar as in Rel-17 using the start time:

Observation 2: Similar as with MBS broadcast and eDRX/MICO mode in Rel-17 there is no RAN2 specification impact for Rel-18 MBS broadcast and UE power saving.

### MBS multicast and power saving

When the UE is in RRC_IDLE with eDRX or MICO mode activated:

- The UE wakes-up at the scheduled activation time:

  - When the UE already joined the session: the UE starts monitoring paging in RRC_IDLE

  - When the UE did not join the session already: the UE requests to join the session

The CN triggers group paging at the scheduled activation time when there are UEs in CM-IDLE that have already joined. The gNB does not use eDRX for CN group paging, and this feature is transparent to the gNB.

But the UE requirements to monitor paging when the UE wakes-up at the scheduled activation time should be captured in 38.304. For example it needs to be clarified that the UE configured with eDRX or MICO mode monitors paging as if eDRX or MICO mode was not configured:

Proposal 1: Clarify in 38.304 that when the UE is configured with eDRX or MICO mode, the UE monitors paging at the scheduled activation time as if eDRX or MICO mode was not configured.

These paging requirements do not need to be captured in CT1 specifications TS 24.501, as was proposed in CT1 [1]. The join procedure is specified in TS 24.501. CT1 may consider clarifying the join behavior when a series of scheduled activation times is configured. For example the UE joins and leaves the session based on the UE’s interest, i.e. the UE does not leave when it is interested to receive the session at the next scheduled activation time.

SA2 noted in 23.247 that it is left to UE implementation for how long the UE monitors group paging:

-	At the possible scheduled activation times, an RRC_IDLE UE that already joined the multicast MBS session needs to listen for paging requests and if paged by the network follow the existing procedures in clause 7.2.5.2. How long the device listens to paging is left up to device implementation.

NOTE 3:	How long the UE need to listen to paging is left up to UE implementation.

The scheduled activation time is a date and time in UTC format, e.g. (YYYY-MM-DD) and time (hh:mm:ss) [2]. The granularity is in seconds. Furthermore the SNTP time synchronization requirement for an MBS UE is +\- 1 second [3]:

Observation 3: In case the UE only monitors a single paging occasion the UE may miss the group paging at the scheduled activation time due to the granularity of the scheduled activation time and the clock accuracy in the upper layers.

Similar as with eDRX and MICO mode there should be a certain period during which the UE is reachable:

Proposal 2: The UE monitors paging at the scheduled activation time for the duration of the configured CN PTW (when configured with eDRX) or Active Time (when configured with MICO mode).

RAN2 needs to discuss further what the UE should do when the Active Time is not configured with MICO mode.

NOTE: SA2 decided to use group paging for UEs that already joined before the scheduled activation time. But it is beneficial for both UE (power saving) and NW (paging resources) if the paging phase is skipped, and the UE transits to RRC_CONNECTED at the scheduled activation time (similar as the UE that still has to join). Perhaps SA2 preferred to re-use the Rel-17 procedure, and perhaps the SMF is not aware of the scheduled activation time (to skip paging).

### eDRX in RRC_INACTIVE and RAN group paging

The gNB may release a UE to RRC_INACTIVE when there is data-inactivity for an active session or the session is deactivated. It is the responsibility to trigger RAN group paging when there is new multicast data or the session is activated again. In case of session activation there could also be UEs in CM-IDLE causing the CN to trigger CN group paging, which the UEs in RRC_INACTIVE may also receive. But the gNB cannot rely on that.

When the UE is configured with a short or long eDRX in RRC_INACTIVE, then the UE wakes up to monitor paging according to the configured eDRX cycle e.g. up to every 10,24 sec (Rel-17) or every 3 h (Rel-18). For how long downlink multicast data is and can be buffered in the gNB depends on the gNB implementation. When eDRX in RRC_INACTIVE is configured it may take too long time for the UE to wake-up to monitor paging again and multicast data may be lost. When long eDRX is configured, the gNB may request the CN to buffer the data.

In general when a UE is released to RRC_INACTIVE due to data-inactivity this is assumed to be temporary, i.e. the UE may have to return soon:

Observation 4: The gNB does not configure a short or long eDRX for RRC_INACTIVE when the UE has joined a multicast session.

The gNB is not aware that the UE may be configured with a scheduled activation time and wake-up when the session is activated. This implies that the gNB cannot configure eDRX in RRC_INACTIVE when the session is deactivated. And scheduled activation times do not apply to data-inactivity detected in RAN:

Observation 5: MBS multicast and UE power saving with eDRX in RRC_INACTIVE is not supported.

SA2 assumed that the UE may be configured with a long eDRX in RRC_INACTIVE and when the UE wakes-up at the scheduled activation time that RAN group paging can be used, i.e. the gNB uses DRX to perform RAN group paging. SA2 removed the requirement to not configure eDRX in RRC_INACTIVE when the UE has joined a multicast session [4].

### RACH congestion at the scheduled activation time

When a large group of UEs is requested to transition to connected mode at the scheduled activation time, then this may cause congestion and delay the time until all UEs have entered RRC_CONNECTED. Congestion may also occur during group paging, but in such case the accesses are spread over a complete DRX cycle.

When the UE wants to join at the scheduled activation time then NAS triggers a Join request, i.e. the UE wakes-up from RRC_IDLE and initiates initiates access by sending a preamble in the next available PRACH occasion (38.321). The next available PRACH occasion depends on the PRACH configuration in SIB1 (e.g. PRACH configuration index referring to lookup Tables in 38.211).

Congestion is avoided when an access window is used during which the UEs randomly initiate access. access are spread out. For example an access window TAccessWindow (seconds) is configured in additional to the activation time(s) TActivationTime in the service announcement:

Instead of initiating PRACH selection immediately, the UE initiates PRACH selection in a random frame TAccessWindow later.

Alternatively the CN can configure different activation times in the UEs and try to spread the UEs out that way. But this spreading can only be implemented with a second granularity which includes many PRACH occasions over which the UEs could be distributed to avoid congestion.

Proposal 3: RAN2 to discuss if there is a need to distribute the random access attempts of the UEs at the scheduled activation time.

The UE may join the session before or at the scheduled activation time (see 23.247 section 7.2.10):

-	At the possible start time, an RRC_IDLE or CM-CONNECTED with RRC_INACTIVE UE needs to send a request to transition to RRC_CONNECTED state and join the MBS multicast session (if not done before).

The gNB is not aware of the scheduled activation time and the gNB may release the UE just before the scheduled activation time due to data-inactivity.

And in case the accesses of a large group is distributed over a window in seconds, then there is also the possibility that the UE is released, just before the scheduled activation time starts. The configuration of the RRC inactivity timer is up to gNB implementation and for example may range between 2-10 sec.

To avoid this problem it is proposed to add the remaining time until the scheduled activation time in MSG5. This enables the gNB to avoid releasing the UE just before the scheduled activation time due to inactivity:

Proposal 4: When the UE transitions to RRC_CONNECTED triggered by a scheduled activation time (TActivationTime) the UE includes the remaining time until the scheduled activation time (TActivationTime) in MSG5.

# Summary

RAN2 is kindly asked to discuss MBS and UE power saving:

## Observations

Observation 1: In case the UE prefers UE power saving over MBS multicast reception then the UE can decide to not join a multicast session when the UE is configured with eDRX/MICO in RRC_IDLE.

Observation 2: Similar as with MBS broadcast and eDRX/MICO mode in Rel-17 there is no RAN2 specification impact for Rel-18 MBS broadcast and UE power saving.

Observation 3: In case the UE only monitors a single paging occasion the UE may miss the group paging at the scheduled activation time due to the granularity of the scheduled activation time and the clock accuracy in the upper layers.

Observation 4: The gNB does not configure a short or long eDRX for RRC_INACTIVE when the UE has joined a multicast session.

Observation 5: MBS multicast and UE power saving with eDRX in RRC_INACTIVE is not supported.

## Proposals

Proposal 1: Clarify in 38.304 that when the UE is configured with eDRX or MICO mode, the UE monitors paging at the scheduled activation time as if eDRX or MICO mode was not configured.

Proposal 2: The UE monitors paging at the scheduled activation time for the duration of the configured CN PTW (when configured with eDRX) or Active Time (when configured with MICO mode).

Proposal 3: RAN2 to discuss if there is a need to distribute the random access attempts of the UEs at the scheduled activation time.

Proposal 4: When the UE transitions to RRC_CONNECTED triggered by a scheduled activation time (TActivationTime) the UE includes the remaining time until the scheduled activation time (TActivationTime) in MSG5.

# References

- C1-232896, Supporting multicast MBS session for UE that uses eDRX, Nokia, CR 24.501, CT1#141e

- 26.517, 5G Multicast-Broadcast User Services; Protocols and Formats, SA4, Rel-17

- 26.346, Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs, SA4, Rel-17

- S2-2305464, Resolving open issues for Multicast MBS procedures for UEs using power saving functions, Nokia, CR 23.247, SA2#156-e

The gNB only configures a short eDRX up to 10,24 in RRC_INACTIVE when the UE is also configured with a long eDRX in RRC_IDLE

The gNB configures a long eDRX in RRC_INACTIVE that is shorter or equal to the long eDRX configured in RRC_IDLE. RAN3 is discussing to inform the CN when the UE is configured with (long) eDRX in RRC_INACTIVE.

The UE shall select randomly from the consecutive PRACH occasions within a subframe/PRACH slot, as defined in section 8.1 in 38.213.