3GPP TSG-RAN WG2 Meeting#123                                	R2-230xxxx

Toulouse, France, Aug 21-25, 2023

Agenda item:	7.19.3

Source: 	Nordic Semiconductor ASA

Title: 	Access restrictions for eRedCap

Document for:	Discussion and Approval

# Introduction

Agreements in RAN2#121bis for further reduced UE complexity in FR1:

- SIB1 should be able to indicate whether the cell enables access for eRedCap UEs or not (assuming that eRedCap UE is not allowed to access to the legacy cell nor the cell not supporting eRedCap). FFS on the relationship and granularity with the access control/cell barring purpose indication.

- A Rel-18 eRedCap UE should be able to indicate its support via new UE capability signaling specific to Rel-18 eRedCap.

- Introduce R18 eRedCap UE specific IFRI in SIB1.

- The new R18 eRedCap UE specific IFRI functionality works as follows:

  - Controls cell selection/reselection to intra-frequency cells for eRedCap UEs when this cell is considered barred by the eRedCap UE, as specified in TS 38.304 [20].

  - Working assumption (pending check in running CRs): If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap.

- Introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap, similar to R17.

- From RAN2 perspective, there is no need to introduce eRedCap UE specific initial BWP configuration (i.e. no R18 new field and at most one specific initial UL/DL BWP can be configured).

- If the R17 RedCap specific initial BWP is configured, eRedCap UEs always use it as its specific initial BWP (assuming no eRedCap UE specific initial BWP configuration field introduced).

- Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

Agreements in RAN2#122 for further reduced UE complexity in FR1:

- RAN2 confirms there can be cell(s) supporting Rel-18 eRedCap only, i.e., not allowing Rel-17 RedCap UE to camp and access.

- We introduce R18 versions of 1Rx and 2Rx barring bits and we don’t introduce a R18 version of the HD-FDD allowed-bit, i.e., the R17 HD-FDD allowed-bit is reused for and applied by R18 eRedCap UEs.

- All R18 eRedCap UEs uses the two new LCIDs for Msg3/MsgA PUSCH for CCCH/CCCH1 during Random Access, i.e., both those with peak rate reduction + BB BW reduction, and those with only peak rate reduction.

Agreements in RAN1#113 on eRedcap Msg1/MsgA PRACH early indication:

- A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported.

  - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction).

- When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs.

  - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs.

- Additional early indication in MsgA PRACH is not supported.

Furthermore, in RAN#99, the following proposal was endorsed [1]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps

Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively.

Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz.

Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f   in order to have the same peak rate.

Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following:

Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

This document discussed access restrictions for a Rel-18 eRedCap UE.

# Discussion

## Access restrictions for an eRedCap UE

A Rel-18 eRedCap UE can support either 20MHz+PR1 or BW3/PR3+PR1 or even both. However, according to the RAN#99 [1] note 4 the initial access procedure is the same for both. This RAN guidance implies the gNB must be able to serve both 20MHz+PR1 and BW3/PR3+PR1 to qualify as Rel-18 eRedCap gNB. Furthermore, the gNB does not learn whether the UE supports 20MHz+PR1 or BW3/PR3+PR1 until Msg3 is received. In R18 initial access procedure, both Msg3 and Msg4 are band limited. This is why gNB must support BW3/PR3 in initial access, based on RAN guidance.

The gNB can already bar Rel-18 eRedCap UEs based on Rel-17 introduced RedCap HD-FDD allowed-bit, the RAN2#122 introduced Rel-18 versions of 1Rx and 2Rx barring bits and Rel-18 eRedCap UE specific IFRI. There is no need for further granularity access restrictions on this regard and no reason to grow SIB1 in size.

A Rel-18 eRedCap gNB must support both 20MHz+PR1 and BW3/PR3+PR1 to qualify as Rel-18 eRedCap gNB.

RAN2#121bis introduced Rel-18 eRedCap UE specific IFRI in SIB1. The working assumption from RAN2#121bis is that if Rel-18 eRedCap UE specific IFRI is not present in SIB1 such is the case for Rel-17 cells supporting RedCap, an eRedCap UE considers that the cell does not support Rel-18 eRedCap and treats the cell as barred. We propose to confirm the RAN2#121bis working assumption.

Confirm the RAN2#121bis working assumption on R18 eRedCap UE specific IFRI functionality: If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap.

# Conclusion

In this paper, we discussed Rel-18 eRedCap access restrictions with the following observations and proposals:

- A Rel-18 eRedCap gNB must support both 20MHz+PR1 and BW3/PR3+PR1 to qualify as Rel-18 eRedCap gNB.

- Proposal 1	Confirm the RAN2#121bis working assumption on R18 eRedCap UE specific IFRI functionality: If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap

# References

- RP-230778 (Proposal for PR1 in eRedCap)