3GPP TSG-RAN WG2 #123	R2-2308898

Toulouse, France, August 21st – 25th 2023

Agenda Item:	7.16.2.2

Source:	Ericsson

Title:	Data collection for AI/ML

Document for:	Discussion, Decision

# 1	Introduction

From previous meeting agreements and related discussions, in this paper we provide our views on what RAN2 should focus on, when it comes to collection of data for the AIML for PHY SI, while also sharing our thoughts with respect to the previously endorsed table in R2-2302286.

# 2	Discussion

During RAN2#122, RAN2 spent time analysing the possible requirements of data collection frameworks. In this paper, we further analyse those methods, and we try to describe how those can be adopted (if possible) in the context of AIML.

In particular, in the latest RAN2#122 meeting, the following agreements were taken:

RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. P2a: LS to ask RAN1 to provide the required data content per use case and per LCM purpose, when available, and to what extent said data would / should be specified (in detail). P2b: LS to ask RAN1 about the reporting type (e.g., periodic, event triggered, other) of the identified data content. P3: LS to ask RAN1 about the typical size (value or value range) of the identified data content. P4a: For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection. P4b: LS to RAN1 to confirm the WA (in P4a) on the latency requirement, and ask RAN1 about the typical latency requirement (value or value range) to transfer the identified data content. P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed. P6b: LS to RAN1 to confirm the WA (in P6a) on RRC state of data collection. P5a: For the data generation entity and termination entity deployed at different entities, RAN2 assumes: - For CSI enhancement and beam management use cases: - - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - - For model monitoring at NW side, performance metrics can be generated by UE/gNB and terminated at LMF. P5b: LS to RAN1 to confirm the WA (in P5a) on the generation entity and termination entity of the identified data content and ask for supplement, if any.

From the above agreements, we assume that the RAN2 assumptions on the requirements related to data collection will be studied now in RAN1. In the following, we present our view on the data collection frameworks taking into account the RAN2 assumptions on the requirements.

During RAN2#121, RAN2 endorsed in R2-2302286 a table identifying potential data collection frameworks. The table serves as a starting point and could be modified according to the SI progress.

The frameworks identified by RAN2 for possible data collection used are listed in the follow:

- Logged MDT

- Immediate MDT

- L3 measurements

- L1 measurement (CSI reporting)

- UAI

- Early measurements

- LPP

## 2.1	NW-side models

### 2.1.1	Training

For training of NW-side models, the suitability of the above legacy frameworks or new frameworks very much depends on the location of the training function. Hence, before discussing the suitability of the various data collection frameworks, RAN2 should first discuss the location of the training function of NW-side models, which to a very large extent has been discussed during the [Post122][060][AIML] email discussion.

Related to NW-side models, we note that this discussion was already taken by RAN3 in their study item. In the RAN3 TR 37.817, it is assumed that the training function of NW-side models can be gNB-centric or OAM-centric.

From RAN3 TR 37.817: ### 5.1.2 Solutions and standard impacts #### 5.1.2.1 Locations for AI/ML Model Training and AI/ML Model Inference The following solutions can be considered for supporting AI/ML-based network energy saving: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU.

In TR 37.817, RAN3 considers that the AI/ML model training can be located in the OAM or in the gNB.

Given the scope of the Rel-18 RAN SI and the above RAN3 assumptions, it seems straightforward to assume that the data collection for CSI/beam management should be at least RAN-centric, whereas for positioning the LMF can also be involved (as agreed by RAN1). Hence, RAN2 should evaluate the suitability of the existing data collection frameworks (and of any other potentially new collection framework), when applied to a gNB-centric data collection. The gNB-centric data collection implies that the gNB initiates and terminates the data collection.

For training of NW-side models, RAN2 prioritizes discussion on the suitability of data collection frameworks for gNB-centric data collection.

For training of NW-side models, the gNB-centric data collection implies that the gNB configures the UE to transfer data and initiates/terminates a data transferring session.

Related to an OAM-centric data collection, it is not clear yet in RAN1, whereas there are use cases for physical layer parameters collection at the OAM. And most important, RAN1 should evaluate the feasibility of an OAM-centric data collection, and what are the parameters that the OAM needs from the UE to build a NW-side model. The suitability of data collection frameworks for the case of OAM-centric data collection should be studied by RAN2, once RAN1 has progressed on that.

For training of NW-side models, RAN2 evaluates the suitability of data collection frameworks for OAM-centric data collection, once RAN1 has assessed the feasibility OAM-centric data collection.

For training of NW-side models, the OAM-centric data collection implies that the OAM initiates and terminates the data collection from the UE.

For OAM-centric data collection, we could draw inspiration from logged MDT’s framework for NW-side model training, given that training may imply large of amount of data to be transmitted with no critical delay requirements. In fact, in logged MDT, the UE logs certain measurements while being in RRC_IDLE/INACTIVE and it temporarily stores those measurements until the network requests them. Since a great amount of data may have been collected by the UE, the UE can also signal to the network whether there are additional RRC segments available containing MDT info. This way the network can keep requesting the UE to transmit the remaining information.

For the other legacy signalling frameworks currently specified, only a single measurement/sample can be collected and transferred to the network, which may be a limitation for model training.

However, logged MDT can only be enabled for idle/inactive UEs, which is obviously a limitation for the AIML use cases. Additionally, logged MDT (as well as immediate MDT) is not terminated at the gNB. But the configuration is controlled by the OAM and subject to CN signalling. So, the collected data is terminated in the Trace Collection Entity (TCE). Hence, that there might be more suitable for the OAM-centric data collection, rather than for the gNB-centric data collection.

If feasibility of OAM-centric data collection for NW-side models is assessed by RAN1, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode.

On the other hand, for gNB-centric data collection for NW-side model, for the reasons mentioned above, some parts of MDT can be reused for the purposes of NW-side model training, but the framework is not suitable for our scope.

For gNB-centric data collection for NW-side model, the data collection framework may incorporate some parts of logged MDT. However, MDT as such has certain properties that make it not suitable.

For gNB-centric data collection for NW-side model, RAN2 to study a L3 data collection framework that allows the UE to measure and store a set of measurements (details up to RAN1) to be reported to the gNB upon request.

### 2.1.2	Performance monitoring

For performance monitoring of NW-side models, the NW can in principle use off-the-shelf techniques to evaluate the performance of its own models, e.g., legacy L1/L3 measurements. For example, the network can monitor system/link level performance (e.g., throughput, number of beam failures, number of beam switches) of the users whose transmissions/receptions are scheduled/configured based on the NW-side beam prediction model output. By comparing the system/link level performance statistics to a target system KPI(s), the NW can detect or predict potential system/link performance degradation, thereby, triggering LCM actions like fallback and, subsequently, perform error cause analysis if needed.

Whether there is the need to improve the current L1 reporting or L3 RRC measurements for the sake of NW-side model monitoring should be however studied in RAN1. For example, the UE may need to be configured to collect data within a time window and then report measurement samples in the measured period, i.e., somewhat like the data collection mechanism for training, but with a different time scale.

For NW-side performance monitoring, RAN2 waits for RAN1 input on the need to enhance the L1 reporting configuration or the L3 RRC measurement configuration and reporting.

## 2.2	UE-side models

### 2.2.1	Training

Similar to the discussion for the NW-side models, also for the UE-side models, the suitability of the legacy frameworks, or any new potential framework very much depend on the location of the training function.

In our view, for UE-side models, the natural assumption seems that the AIML training function resides in the UE and/or in an UE-controlled OTT server. For UE-side model training, RAN2 to consider UE-centric data collection performed OTT, i.e. no impact on RAN2 protocols.

About data to be collected, it is not clear yet which information the UE needs to acquire to train UE-side models. That should be discussed, if needed, by RAN1, but that can also be left to the UE implementation and no specification work might be needed. 
However, irrespective of whether the specific data that the UE should collect for UE-side training are specified or not, the UE may need at some point certain specific radio configurations, in order to collect data to further train the model. At a first glance, this task could be accomplished by the UE Assistance Information framework (UAI), but the suitability of UAI also depends on RAN1 progress.

For UE-side model training, RAN2 considers (subject to RAN1 progress), the UE Assistance Information framework as a tool for the UE to request aid from the network in training at the UE.

### 2.2.2	Performance monitoring

For UE-side performance monitoring at the UE, the UE can independently monitor its own system/link level performance and it can detect or predict potential performance degradation. So RAN2 does not need to focus on data collection for UE-side performance monitoring at the UE.

However, the UE may also report measurements/events that were performed as a consequence of a certain model being used at the UE. Although the network may not know which model the UE is using, the UE can report to the network information on whether a certain AIML functionality is working properly.

To this end, the following agreement was taken in RAN2#122:

- - For model monitoring at NW side, performance metrics can be generated by UE and terminated at gNB.

The UE may report performance results of model monitoring to the NW, e.g. accuracy performances, as well as the (non)applicability of a certain AIML functionality. 
For this, a mixture of a layer-1, or layer-2, or layer-3 mechanism might be needed. Which specific layer to adopt may depend on the type of information, and on the amount of information to be reported, as well as on RAN1 discussion.

For UE-side performance monitoring at NW side, RAN2 to focus on impacts in layer-2, or layer-3 (possibly including some layer-1 related measurements) for reporting of the outcome of performance monitoring (e.g. performance monitoring results, (non)applicability of AIML functionality). Layer-1 details are left to RAN1.

### 2.2.3	Data Collection from the UE Application

During the last RAN2 meetings, it was discussed at length in RAN2 whether data collection protocols available outside RAN could be used for the purpose of UE-side model training. In particular, EVEX was considered to be a viable candidate. From our point of view the need of any enhancements to protocols defined by other WGs should not be discussed in RAN2.

The need of any enhancements to non-RAN data collection frameworks for UE-side models should be studied in SA WGs.

## 2.3	Summary

The table in the Annex summarises the suitability of the various existing legacy frameworks, taking into account the sidedness of the model, and the entity terminating/initiating the data collection.

For CSI/beam management use cases, RAN2 to agree to Table 1 in Annex A which maps LCM functions to the various existing data collection frameworks considering; the sidedness of the model, and the entity terminating/initiating the data collection.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In TR 37.817, RAN3 considers that the AI/ML model training can be located in the OAM or in the gNB.

Observation 2	For gNB-centric data collection for NW-side model, the data collection framework may incorporate some parts of logged MDT. However, MDT as such has certain properties that make it not suitable.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For training of NW-side models, RAN2 prioritizes discussion on the suitability of data collection frameworks for gNB-centric data collection.

Proposal 2	For training of NW-side models, the gNB-centric data collection implies that the gNB configures the UE to transfer data and initiates/terminates a data transferring session.

Proposal 3	For training of NW-side models, RAN2 evaluates the suitability of data collection frameworks for OAM-centric data collection, once RAN1 has assessed the feasibility OAM-centric data collection.

Proposal 4	For training of NW-side models, the OAM-centric data collection implies that the OAM initiates and terminates the data collection from the UE.

Proposal 5	If feasibility of OAM-centric data collection for NW-side models is assessed by RAN1, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode.

Proposal 6	For gNB-centric data collection for NW-side model, RAN2 to study a L3 data collection framework that allows the UE to measure and store a set of measurements (details up to RAN1) to be reported to the gNB upon request.

Proposal 7	For NW-side performance monitoring, RAN2 waits for RAN1 input on the need to enhance the L1 reporting configuration or the L3 RRC measurement configuration and reporting.

Proposal 8	For UE-side model training, RAN2 considers (subject to RAN1 progress), the UE Assistance Information framework as a tool for the UE to request aid from the network in training at the UE.

Proposal 9	For UE-side performance monitoring at NW side, RAN2 to focus on impacts in layer-2, or layer-3 (possibly including some layer-1 related measurements) for reporting of the outcome of performance monitoring (e.g. performance monitoring results, (non)applicability of AIML functionality). Layer-1 details are left to RAN1.

Proposal 10	The need of any enhancements to non-RAN data collection frameworks for UE-side models should be studied in SA WGs.

Proposal 11	For CSI/beam management use cases, RAN2 to agree to Table 1 in Annex A which maps LCM functions to the various existing data collection frameworks considering; the sidedness of the model, and the entity terminating/initiating the data collection.

# 4	Annex A – Analysis of suitability of the legacy data collection frameworks

|  | Model Training |  |  | Model Performance monitoring |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-side |  | UE-side (only UE-centric) | NW-side |  | UE-side |  |
|  | gNB-centric data collection | OAM-centric data collection |  | gNB-centric data collection | OAM-centric data collection | Monitoring at UE | Monitoring at gNB |
| Logged MDT | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable, provided that enhancements are at least introduced for logging in RRC-Connected mode. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the OAM for the UE-side training. | Not suitable: performance monitoring has stringent latency requirements. Moreover, in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. Additionally, logged MDT measurements are not done in RRC Connected state. | Not suitable: performance monitoring has stringent latency requirements. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Not suitable: performance monitoring has stringent latency requirements. Moreover, in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. |
| Immediate MDT | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable: in Immediate MDT, the initiation of the reporting session and the data collection termination point is in the TCE/OAM, and it works in RRC_Connected mode. Some enhancements may be potentially needed related to large data volume reporting | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the OAM for the UE-side training. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Suitable: in Immediate MDT, the initiation of the reporting session and the data collection termination point is in the TCE/OAM, and it works in RRC_Connected mode. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. | Not suitable: in MDT the initiation of the reporting session and the data collection termination point is in the TCE/OAM. |
| L3 Measurements | Suitable provided that enhancements are introduced for the reporting to the gNB of multiple samples of measurements which are stored by the UE during measurement period. | Not suitable: L3 measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-side training. | Suitable: up to RAN1 to discuss what enhancements are needed. | Not suitable: L3 measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, L3 measurements are not suitable because they are configured by the gNB and reported to the gNB. | Suitable: L3 measurements can be used by the UE to report its model/function applicability conditions, performance monitoring results. FFS in RAN1/2 what should be reported. |
| L1 Measurement (CSI reporting) | Not suitable: there are no stringent latency requirements for training, and data overhead can be an issue. | Not suitable: L1 measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-side training. | Suitable: up to RAN1 to discuss what enhancements are needed. | Not suitable: L1 measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, L1 measurements are not suitable because they are configured by the gNB and reported to the gNB. | Not suitable: For the reporting of model/fucntion applicability conditions, accuracy performance, or desired configurations, L3 measurements or UAI seem more sutiable from RAN2 point of view. This assumption can be revisited based on RAN1 progress. |
| UAI | Not suitable: in legacy the information reported in the UAI and the triggering conditions for the reporting, are typically left to the UE implementation. Hence, it is not clear what would be the gain of UAI versus L3 measurements. | Not suitable:UAI are configured by the gNB and reported to the gNB. | Suitable: The UE can use UAI to request to the gNB some desired configurations in order to train the UE model. | Not suitable: in legacy the information reported in the UAI and the triggering conditions for the reporting, are typically left to the UE implementation. Hence, it is not clear what would be the gain of UAI versus L3/L1 measurements. | Not suitable:UAI are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, UAI is not applicable because it is reported to the gNB. | Suitable: UAI can be used by the UE to report its model/function applicability conditions, performance monitoring results. FFS in RAN1/2 what should be reported. However, since L3 measurements can also be used for the same purpose, RAN2 should discuss whether L3 measurements or UAI is more suitable. |
| Early Measurements | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode. So it is not clear what would be the gain of Early Measurements versus L3 measurements | Not suitable: early measurements are configured by the gNB and reported to the gNB. | Not suitable: The training is performed by the UE (possibly with the aid of OTT server). No need to report trained results to the gNB for the UE-side training. | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode, whereas performance monitoring of NW-side models should be based on UE operations while in connected mode. | Not suitable: early measurements are configured by the gNB and reported to the gNB. | Not suitable: if the monitoring occurs at the UE, early measurements are not suitable because they are configured by the gNB and reported to the gNB. | Not suitable: in legacy the early measurements are collected while in IDLE/INACTIVE mode. So it is not clear what would be the gain of Early Measurements versus L3 measurements reporting or UAI. |
| LPP | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases. | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases | N/A: LPP is not applicble to CSI and beam management use cases |

Table 1: Suitability analysis of legacy data collection frameworks for LCM functions for CSI and beam management use case.