3GPP TSG-RAN WG2 Meeting #123bis	R2-2310265

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.11.2.1

Source:	CMCC

Title:	Discussion on CP open issues

Document for:	Discussion

Introduction

Rapporteurs has listed the open issues to progress the completion of Rel-18 MBS work[1], in this contribution, we’ll discuss the following CP open issues:

- MCCH scheduling

- RRC resumption related issues

- Synchronization indication in the serving cell

- Frequency prioritization

- Special UE handling

- PTM configuration acquisition

- SDT and multicast reception

Discussion

MCCH Scheduling

Multicast MCCH is used for PTM configuration acquisition by RRC_INATIVE UEs, and based on the agreements, UE acquires multicast MCCH if it is notified of MCCH change or reselected to a new cell [2]:

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

In the RRC running CR [3], there’s an FFS that “Editor’s Note: FFS if MCCH is optional and whether “if SIBx is not scheduled in SIB1” needs some rewording”. And in some companies’ opinion, MCCH is only used for PTM configuration update, therefore, it is optional.

But it should be noticed that in Rel-17, the UE need to perform the Session Join procedure to be authorized by the 5GC and get the necessary information of the multicast session. What’s more, the first UE’s joint request may trigger the multicast session establishment towards the NG-RAN and the UE [4]. And it was agreed that  Rel-18 multicast supporting UEs should also perform join procedure. Therefore, the same principle should be applied, in other words, MCCH should be present from the time the session is activated since for RRC_INACTIVE multicast reception UEs, MCCH is used for the necessary information provision.

Besides, since MCCH is also used for the re-selection case, and the network has no information of the reselection UEs, therefore, MCCH cannot just be present once PTM configuration change.

Proposal 1: The gNB starts to deliver MCCH from the time that the session is activated.

RRC Resumption related issues

In previous meetings, UE initiated RRC resumption based on the network configured threshold was discussed, and it was agreed that the threshold can be RSRP or RSRQ, and it is configured per multicast session [5]:

For a UE receiving multicast in RRC_INACTIVE, the UE resumes the RRC connection when the measured RSRP or RSRQ based on the existing measurement requirements (whichever is configured by the NW) of the serving cell becomes lower than the threshold configured by network. FFS whether/how we need to address ping-pong issue The threshold can be configured in PTM configuration per MBS session via RRCRelease or multicast MCCH message. Unless issues are identified with using one of existing resume causes, no new resume causes are introduced for UEs receiving MC in INACTIVE when they resume due to bad quality or lack of SIBx/PTM configuration

In the online discussion, some companies had concerns on the ping-pong issue due to UE’s request for RRC Resumption. In our opinion, the reason that UE request for RRC Resumption is due to the bad reception quality, and once it is switched back to RRC_CONNECTED state, the network may have information of UE’s bad reception quality based on UE’s measurement report, therefore, UE will not be released to RRC_INACTIVE state until the UE’s reception quality becomes good enough. From this point, ping-pong issues can be resolved or avoided based on current measurement mechanism.

Observation 1: Since the network may have information of UE’s reception quality after UE switching back to RRC_CONNECTED state, there will be no ping-pong issue.

Proposal 2: Based on the current measurement mechanism, ping-pong issues can be avoided by the network.

Besides, there’s an open issue on the granularity of the threshold in the discussion of 331 running CR:

- Editor’s note: FFS whether the RSRP/RSRQ is L1 or L3 measurement.

Since the RSRP/RSRQ threshold for RRC Resumption is used by RRC_INACTIVE UEs, and for cell selection/reselection, RSRP/RSRQ is also used. Therefore, it’s rational to follow the RSRP/RSRQ calculating principle for cell selection/reselection.

Proposal 3: RSRP/RSRQ threshold for RRC resumption calculation can follow the same principle/granularity for cell selection/reselection.

Synchronization indication in the serving cell

In last meeting, RAN2 discussed how to handle the MRBs when UE in RRC_INACTIVE state for multicast reception reselects to a new cell, it depends on whether the cells are synchronized or not, while how to indicate the cell is synchronized is still under discussion.

For “non-synchronised“ cell (in terms of PDCP COUNT), upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast. One cell can indicate "synchronized", if by implementation, it follows a common QoS flow to MRB mapping rule and at the same time PDCP COUNT is set according to the MBS QoS Flow SN. FFS how the UE is indicated about cells being synchronized (i.e. what information the NW needs to provide to the UE)

As we agreed that the PTM configuration can be provided in the RRCRelease message if useful, in other words, whether PTM configuration is included in RRCRelsease message is optional. Also, it was agreed that UE will acquire MCCH when it reselect to a new cell, therefore, at least, the indication should be included in MCCH. And it should be indicated per MBS session. Meanwhile, we have agreed to use the same signalling structure of PTM configuration for MCCH and RRCRelsease message, therefore, it can also be indicated in RRCRelsease message.

Proposal 4: Cell synchronized or not should be indicated per MBS session in MCCH and optionally in RRCRelease message.

Frequency prioritization

Frequency prioritization was introduced for Rel-18 multicast reception by RRC_INACTIVE UEs, and dedicated priorities in RRCRelease message is used as legacy:

Dedicated frequencies in RRCRelease can be used by the NW, as legacy FFS whether we need something more, e.g. frequency priorities in MCCH or a solution based on FSAI

In Rel-17 broadcast, frequency is prioritized by UE implementation with the information between the frequency and the MBS service. When it comes to Rel-18 multicast reception, In the current RRC running CR, no change is introduced for multicast reception purpose. Therefore UE cannot identify which frequency is prioritized due to which MBS session.

Observation 2: UE has no knowledge of which frequency is prioritized for MBS session reception.

Observation 3: Current frequency prioritization for Rel-18 MBS is network implementation, but not UE’s preference.

Besides, it’s not clear whether prioritized frequencies in different cells are the same. If the frequency information considering MBS reception is only provided in dedicated signalling, the UE may need to resume RRC connection to acquire the frequency prioritization information if it reselect to other cells, and this does not align with the motivation to use the mixed approach. Therefore, it’s beneficial to provide the information of the service and the neighbour frequency in broadcast way, for example, in MCCH or FSAI related approach, and optionally in RRCRelease message.

Proposal 5: The information between the service and the neighbour frequency can be provided in broadcast way, for example, in MCCH or FSAI related approach, and optionally in RRCRelease message.

Special UE handling

The Special UE indicated by 5GC handling when the session is deactivated was discussed in RAN2#121bis meeting [6], and it was agreed that it can be released to RRC_INACTIVE, but how to ensure its reception when session activated does not reach consensus.

A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

Though the special UE indicated by 5GC assistance information is to ensure its multicast reception is in RRC_CONNECTED, we have agreed both group paging and UE-specific paging can be used for Rel-18 multicast UE. If the special UE does not resume the RRC connection after group paging, UE-specific can be used to move it to RRC_CONNECTED. Therefore, the current mechanism is enough to ensure special UE to resume to RRC_CONNECTED upon session activation.

Proposal 6: If the special UE is not in RRC_CONNECTED after group paging, the UE-specific paging can be used to resume to RRC_CONNECTED upon session activation.

Initial PTM configuration acquisition

In the stage 2 running CR discussion [7], there’s one open issue related to the initial PTM configuration acquisition:

Editor’s Note: FFS whether the UE can get the initial PTM configuration via multicast MCCH.

As mentioned in other issues discussion, the PTM configuration is optionally provided in RRCRelease message, hence, to avoid unnecessary UE RRC states switching, it’s beneficial for the UE to acquire the initial PTM configuration via multicast MCCH if multicast MCCH is present.

Observation 4: UE acquiring the initial PTM configuration via MCCH is helpful to avoid UE RRC states switching.

Besides, there’s one rational scenario for further discussion besides the two identified in the beginning of Rel-18 discussion, in which UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s), which means the UE need to join in the MBS session(s) first as shown in Figure 1.

Figure 1

Observation 5: There’s a scenario that UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s) and needs to join in the MBS session.

Proposal 7: RAN2 is kindly asked to discuss the Scenario where a UE is released to RRC_INACTIVE first and then perform the join procedure for multicast reception.

If the current procedure is used, UE needs to switch to RRC_CONNECTED to complete the join procedure. And one possible outcome is after UE’s join, it will be released to RRC_INACIVE state for MBS data reception based on gNB’s decision for an MBS session or the specific UE. Assuming the time when UE joins the session, the multicast data transmission has started for a period of time, the UE will spend extra time and UE power consumption for the multicast data reception due to the twice RRC states switches. And in some sense, UE will miss some multicast data due to the extra latency.

Observation 6: In this scenario, UE will spend extra time and power consumption for the twice RRC states switches to perform join procedure.

Since that from the core network point of view, both RRC_CONNECTED and RRC_INACTIVE are CM_CONNECTED state, which has no difference to it, and considering the potential impact mentioned above, we propose to support that the join procedure can be triggered or performed by UE in RRC_INACTIVE state without RRC states switch

Proposal 8: Join procedure can be triggered by UE in RRC_INACTIVE state without RRC states switch.

Based on [4], the main purpose of the MBS session join procedure is to indicate the MBS Session ID(s) to the core network to identify the MBS Session UE interested in. Therefore, at least the MBS Session ID(s) should be indicated to the core network when UE is in RRC_INACTIVE. To achieve this, small data transmission mechanism can be considered.

Proposal 9: Small data transmission mechanism can be considered to transmit necessary information of join procedure, for example, MBS Session ID(s).

Besides, based on the current progress of multicast reception for RRC_INACTIVE, it’s gNB to decide which MBS session can be delivered for UEs in RRC_INACTIVE and which UE can receive MBS session in RRC_INACTIVE. Therefore, after the join procedure in RRC_INACTIVE, the gNB can decide the UE’s MBS reception state and indicates in the further procedure.  If UE is indicated to stay in RRC_INACTIVE state for MBS reception, the gNB should provide the related PTM configuration or at least MCCH configuration for the UE.

Observation 7: UE acquiring initial PTM configuration via MCCH is beneficial in the scenario that UE is released to RRC_INACTIVE first and perform join procedure later.

Proposal 10: UE can acquire the initial PTM configuration via MCCH.

SDT and multicast reception

During the running CR discussion phase, one FFS is proposed about SDT and multicast reception:

Editor’s Note: FFS whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together. And if yes, whether UE configured for MBS multicast reception in RRC_INACTIVE monitors group paging during SDT

In legacy, UE monitors paging using TMGI only when DT procedure is not ongoing [7]：

The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - During SDT procedure, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE; - If configured by upper layers for MBS multicast reception, while SDT procedure is not ongoing, monitors a Paging channel for paging using TMGI;

Observation 8： The UE may miss group notification for session activation due to ongoing SDT procedure.

Though the PTM configuration can be provided in MCCH, the UE will not monitor the related RNTI until session activation notification, therefore, it’s proposed to monitor paging message regardless that  SDT procedure is ongoing or not.

Proposal 11: SDT and MBS multicast reception in RRC_INACTIVE state can be configured together.

Proposal 12: UE monitors paging message regardless that  SDT procedure is ongoing or not.

Conclusions

In this contribution, we analyse the CP open issues of multicast reception by RRC inactive UEs, following are the observations and proposals.

Observations:

Observation 1: Since the network may have information of UE’s reception quality after UE switching back to RRC_CONNECTED state, there will be no ping-pang issue.

Observation 2: UE has no information of prioritized frequency is related to which MBS session.

Observation 3: Current frequency prioritization for Rel-18 MBS is network implementation, but not UE’s preference.

Observation 4: UE acquiring the initial PTM configuration via MCCH is helpful to avoid UE RRC states switching.

Observation 5: There’s a scenario that UE is released to RRC_INACTIVE first, then UE is interested in the MBS service(s) and needs to join in the MBS session.

Observation 6: In this scenario, UE will spend extra time and power consumption for the twice RRC states switches to perform join procedure.

Observation 7: UE acquiring initial PTM configuration via MCCH is beneficial in the scenario that UE is released to RRC_INACTIVE first and perform join procedure later.

Observation 8： The UE may miss group notification for session activation due to ongoing SDT procedure.

Proposals:

Proposal 1: The gNB starts to deliver MCCH from the time that the session is activated.

Proposal 2: Based on the current measurement mechanism, ping-pong issues can be avoided by the network.

Proposal 3: RSRP/RSRQ threshold for RRC resumption calculation can follow the same principle/granularity for cell selection/reselection.

Proposal 4: Cell synchronized or not should be indicated per MBS session in MCCH and optionally in RRCRelease message.

Proposal 5: The information between the service and the neighbour frequency can be provided in broadcast way, for example, in MCCH or FSAI related approach, and optionally in RRCRelease message.

Proposal 6: If the special UE is not in RRC_CONNECTED after group paging, the UE-specific paging can be used to resume to RRC_CONNECTED upon session activation.

Proposal 7: RAN2 is kindly asked to discuss the Scenario where a UE is released to RRC_INACTIVE first and then perform the join procedure for multicast reception.

Proposal 8: Join procedure can be triggered by UE in RRC_INACTIVE state without RRC states switch.

Proposal 9: Small data transmission mechanism can be considered to transmit necessary information of join procedure, for example, MBS Session ID(s).

Proposal 10: UE can acquire the initial PTM configuration via MCCH.

Proposal 11: SDT and MBS multicast reception in RRC_INACTIVE state can be configured together.

Proposal 12: UE monitors paging message regardless that  SDT procedure is ongoing or not.

References

- draft R2-230xxxx Open issue list for NR MBS enhancements;

- R2_120_meeing_report;

- R2-2309033 RRC Running CR for eMBS;

- TS 23.747 Architectural enhancements for 5G multicast-broadcast services; Stage 2;

- R2_123_meeing_report;

- R2_121bis_meeing_report;

- R2-2309034 38.300 Running CR for MBS enhancements；

- 3GPP TR 38.331 RRC;