3GPP TSG-RAN WG2 #124	R2-2312832

Chicago, US, 13th – 17th November 2023

Agenda Item:	7.4.5

Source:	Ericsson, CMCC

Title:	Discussion on early measurements enhancements

Document for:	Discussion, Decision

# 1	Introduction

The rel-18 work on further NR mobility enhancements is described in the WID in [1]. The WID includes an objective related to IDLE/INACTIVE mode measurements (objective 7):

- To study and specify how to reuse the IDLE/INACTIVE mode measurement results which are to be reported during and/or after RRC connection setup/resume in order to improve SCell/SCG setup delay [RAN4, RAN2], including:​

- Availability and validation of the IDLE/INACTIVE mode measurement results to be reported [RAN4]; and​

- Definition of corresponding RRM requirements [RAN4]; and​

- If necessary, based on RAN4 outcome, definition of corresponding signaling support [RAN2].​

Note 5: RAN4 will coordinate in due course with RAN2 to start the work.​

Note 6: R4-2220415 serves as baseline for future work in RAN4​

Note 7: With exception of the above scenarios, enhancements on IDLE/INACTIVE mode measurements and on UE behavior in IDLE/INACTIVE mode are not in scope.​

RAN4 has discussed the topic for several meetings and has agreed on new measurements starting at RRC setup. RAN4 has also agreed to introduce a configurable validity timer for existing EMR measurements.

# 2	Discussion

## 2.1	Measurements performed at RRC setup

RAN4 has agreed on new measurements that are performed during the setup of a connection. The UE can indicate availability of measurement results in msg5 according to the following picture:

These new measurements have been discussed in [Post123bis][551][feMob] eEMR SCell setup delay (Nokia), for further discussion in RAN2#124.

## 2.2	Updates to legacy EMR

RAN4 has also discussed additions to existing early measurements. They made an agreement in RAN4#108bis that it should be possible to configure a validity timer to the UE. The UE will only report measurement results that are not older than the configured timer value.

<Agreement>:

- The measurements are considered valid if both of the following conditions are satisfied

  - A) the measurement are performed within the last [X] seconds before it is reported

    - X value is network configured. Signalling details are up to RAN2

    - FFS on the X value(s) and will be decided by RAN4

    - If X is not defined then no requirements will be introduced

  - B) the reported measurement results satisfy measurement accuracy [at the measurement instance]

  - FFS on side conditions

The RAN4 agreement is useful and ensures that the UE does not report very old measurements. The agreement implies that the UE needs to keep track of how old the measurements are in comparison with the time when reporting the measurements, so that it can check whether the measurements are older or newer than the configured timer value.

The information about how old different measurements are is very useful to the network also in order to determine the most suitable carrier for CA/DC setup. As the UE anyhow needs to keep this information, it might as well report it to the network also. With this information the network can configure a larger validity time and thus receive more results that can be used as input to the decision. Otherwise, the network may need to configure a small validity time value, to be certain that the measurements are not too old.

The UE needs to keep a record of how old measurements are compared to the time when the measurements are reported, to be able to compare the time with the configured validity timer value.

Sending this time difference information to the network is thus a minor addition.

The UE reports the time difference between when the EMR measurements are performed and when the EMR measurement are reported.

## 2.3	New UE capabilities

The updates mentioned above require some new UE capabilities, as they are likely to be associated with network configuration. It is proposed to keep the complexity down and introduce one UE capability for measurements performed at setup and one UE capability for the validity timer and reporting.

Introduce a new UE capability for the support of measurements performed at setup.

Introduce a new UE capability for the support of validity timer and reporting of time difference between performing and reporting EMR measurements.

# 3	Conclusion

In the previous sections the following observations were made:

Observation 1	The UE needs to keep a record of how old measurements are compared to the time when the measurements are reported, to be able to compare the time with the configured validity timer value.

Observation 2	Sending this time difference information to the network is thus a minor addition.

Based on the discussion in the previous sections the following is proposed:

Proposal 1	The UE reports the time difference between when the EMR measurements are performed and when the EMR measurement are reported.

Proposal 2	Introduce a new UE capability for the support of measurements performed at setup.

Proposal 3	Introduce a new UE capability for the support of validity timer and reporting of time difference between performing and reporting EMR measurements.

# 4	References

RP-221799, Revised WID on Further NR mobility enhancements, MediaTek Inc., 3GPP TSG RAN Meeting #96, June 6-9, 2022

R4-2314466, LS on improvement on FR2 SCell/SCG setup delay

R2-23xxxxx, [Post123bis][551][feMob] eEMR SCell setup delay (Nokia)

R4-2317428, LS on improvement on FR2 SCell/SCG setup delay

# 5	Annex

ASN.1 impacts on information elements in 38.331 are shown below. Impacts on RRC messages would be similar as shown in the TP in e-mail discussion [Post123bis][551][feMob] eEMR SCell setup delay (Nokia).

START OF CHANGE

### 6.3.2	Radio resource control information elements

#### –	MeasIdleConfig

The IE MeasIdleConfig is used to convey information to UE about measurements requested to be done while in RRC_IDLE or RRC_INACTIVE.

MeasIdleConfig information element

-- ASN1START

-- TAG-MEASIDLECONFIG-START

MeasIdleConfigSIB-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need S

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need S

...,

[[

measSetupCarrierListNR-r18		SEQUENCE (SIZE (1..maxFreqSetup-r18)) OF MeasIdleCarrierNR-r16         OPTIONAL      -- Need S

]]

}

MeasIdleConfigDedicated-r16 ::= SEQUENCE {

measIdleCarrierListNR-r16       SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16          OPTIONAL,     -- Need N

measIdleCarrierListEUTRA-r16    SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16       OPTIONAL,     -- Need N

measIdleDuration-r16            ENUMERATED{sec10, sec30, sec60, sec120, sec180, sec240, sec300, spare},

validityAreaList-r16            ValidityAreaList-r16                                                   OPTIONAL,     -- Need N

...,

[[

measSetupCarrierListNR-r18		SEQUENCE (SIZE (1..maxFreqSetup-r18)) OF MeasIdleCarrierNR-r16         OPTIONAL      -- Need S

]]

}

ValidityAreaList-r16 ::= SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF ValidityArea-r16

ValidityArea-r16 ::=             SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

validityCellList-r16             ValidityCellList                                                     OPTIONAL   -- Need N

}

ValidityCellList ::= SEQUENCE (SIZE (1.. maxCellMeasIdle-r16)) OF PCI-Range

MeasIdleCarrierNR-r16 ::=        SEQUENCE {

carrierFreq-r16                  ARFCN-ValueNR,

ssbSubcarrierSpacing-r16         SubcarrierSpacing,

frequencyBandList                MultiFrequencyBandListNR                                             OPTIONAL,  -- Need R

measCellListNR-r16               CellListNR-r16                                                       OPTIONAL,  -- Need R

reportQuantities-r16             ENUMERATED {rsrp, rsrq, both},

qualityThreshold-r16             SEQUENCE {

idleRSRP-Threshold-NR-r16        RSRP-Range                                                           OPTIONAL,  -- Need R

idleRSRQ-Threshold-NR-r16        RSRQ-Range                                                           OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need R

ssb-MeasConfig-r16               SEQUENCE {

nrofSS-BlocksToAverage-r16          INTEGER (2..maxNrofSS-BlocksToAverage)                            OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation-r16 ThresholdNR                                                       OPTIONAL,   -- Need S

smtc-r16                            SSB-MTC                                                           OPTIONAL,   -- Need S

ssb-ToMeasure-r16                   SSB-ToMeasure                                                     OPTIONAL,   -- Need S

deriveSSB-IndexFromCell-r16         BOOLEAN,

ss-RSSI-Measurement-r16             SS-RSSI-Measurement                                               OPTIONAL    -- Need S

}                                                                                                     OPTIONAL,  -- Need S

beamMeasConfigIdle-r16           BeamMeasConfigIdle-NR-r16                                            OPTIONAL,  -- Need R

...,

[[

validityTimerMeasIdle-r18				ENUMERATED {value1, value2, value3, value4}                       OPTIONAL  -- Need R

reportTimeDifference-r18                ENUMERATED {true}                                                 OPTIONAL  -- Need R

]]

}

MeasIdleCarrierEUTRA-r16 ::=     SEQUENCE {

carrierFreqEUTRA-r16             ARFCN-ValueEUTRA,

allowedMeasBandwidth-r16         EUTRA-AllowedMeasBandwidth,

measCellListEUTRA-r16            CellListEUTRA-r16                                                    OPTIONAL,  -- Need R

reportQuantitiesEUTRA-r16        ENUMERATED {rsrp, rsrq, both},

qualityThresholdEUTRA-r16        SEQUENCE {

idleRSRP-Threshold-EUTRA-r16     RSRP-RangeEUTRA                                                      OPTIONAL,  -- Need R

idleRSRQ-Threshold-EUTRA-r16     RSRQ-RangeEUTRA-r16                                                  OPTIONAL   -- Need R

}                                                                                                     OPTIONAL,  -- Need S

...

}

CellListNR-r16  ::=       SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF PCI-Range

CellListEUTRA-r16  ::=    SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange

BeamMeasConfigIdle-NR-r16  ::=   SEQUENCE {

reportQuantityRS-Indexes-r16     ENUMERATED {rsrp, rsrq, both},

maxNrofRS-IndexesToReport-r16    INTEGER (1.. maxNrofIndexesToReport),

includeBeamMeasurements-r16      BOOLEAN

}

RSRQ-RangeEUTRA-r16 ::=   INTEGER (-30..46)

-- TAG-MEASIDLECONFIG-STOP

-- ASN1STOP

MeasIdleConfig field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

beamMeasConfigIdle Indicates the beam level measurement configuration.

carrierFreq Indicates the NR carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

carrierFreqEUTRA Indicates the E-UTRA carrier frequency to be used for measurements during RRC_IDLE or RRC_INACTIVE.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR idle/inactive measurement parameters apply. The UE shall select the first listed band which it supports in the frequencyBandList field to represent the NR neighbour carrier frequency.

includeBeamMeasurements Indicates whether or not the UE shall include beam measurements in the NR idle/inactive measurement results.

maxNrofRS-IndexesToReport Max number of beam indices to include in the idle/inactive measurement result.

measCellListEUTRA Indicates the list of E-UTRA cells which the UE is requested to measure and report for idle/inactive measurements.

measCellListNR Indicates the list of NR cells which the UE is requested to measure and report for idle/inactive measurements.

measIdleCarrierListEUTRA Indicates the E-UTRA carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleCarrierListNR Indicates the NR carriers to be measured during RRC_IDLE or RRC_INACTIVE.

measIdleDuration Indicates the duration for performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. Value sec10 correspond to 10 seconds, value sec30 to 30 seconds and so on.

measSetupCarrierListNR Indicates the list of NR cells which the UE is requested to measure and report for measurements performed at setup.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

qualityThreshold Indicates the quality thresholds for reporting the measured cells for idle/inactive NR measurements.

qualityThresholdEUTRA Indicates the quality thresholds for reporting the measured cells for idle/inactive E-UTRA measurements.

reportQuantities Indicates which measurement quantities UE is requested to report in the idle/inactive measurement report.

reportQuantitiesEUTRA Indicates which E-UTRA measurement quantities the UE is requested to report in the idle/inactive measurement report.

reportQuantityRS-Indexes Indicates which measurement information per beam index the UE shall include in the NR idle/inactive measurement results.

reportTimeDifference Indicates that the time difference between when the measurement was performed and when the measurement is reported shall be included for the NR idle/inactive measurement results.

smtc Indicates the measurement timing configuration for inter-frequency measurement. If this field is absent in VarMeasIdleConfig, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssbSubcarrierSpacing Indicates subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent in VarMeasIdleConfig, the UE measures on all SS-blocks.

ss-RSSI-Measurement Indicates the SSB-based RSSI measurement configuration. If the field is absent in VarMeasIdleConfig, the UE behaviour is defined in TS 38.215 [89], clause 5.1.3.

validityAreaList Indicates the list of frequencies and optionally, for each frequency, a list of cells within which the UE is required to perform measurements while in RRC_IDLE and RRC_INACTIVE.

validityTimerMeasIdle Indicates the value of the validity timer. The UE reports idle/inactive measurement results that are not older than the timer value indicates. FFS on timer values, to be decided by RAN4.

#### –	MeasResultIdleNR

The IE MeasResultIdleNR covers the NR measurement results performed in RRC_IDLE and RRC_INACTIVE.

MeasResultIdleNR information element

-- ASN1START

-- TAG-MEASRESULTIDLENR-START

MeasResultIdleNR-r16 ::=  SEQUENCE {

measResultServingCell-r16 SEQUENCE {

rsrp-Result-r16           RSRP-Range                                                                        OPTIONAL,

rsrq-Result-r16           RSRQ-Range                                                                        OPTIONAL,

resultsSSB-Indexes-r16    ResultsPerSSB-IndexList-r16                                                       OPTIONAL

},

measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16    OPTIONAL,

...,

[[

measResultPerCarrierListSetupNR-r18 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16    OPTIONAL

]]

}

MeasResultsPerCarrierIdleNR-r16 ::=   SEQUENCE {

carrierFreq-r16                       ARFCN-ValueNR,

measResultsPerCellListIdleNR-r16      SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16,

... ,

[[

timeDifferenceReportMeas-r18          FFS                                                                       OPTIONAL

]]

}

MeasResultsPerCellIdleNR-r16 ::=  SEQUENCE {

physCellId-r16                    PhysCellId,

measIdleResultNR-r16              SEQUENCE {

rsrp-Result-r16                   RSRP-Range                                                              OPTIONAL,

rsrq-Result-r16                   RSRQ-Range                                                              OPTIONAL,

resultsSSB-Indexes-r16            ResultsPerSSB-IndexList-r16                                             OPTIONAL

},

...

}

ResultsPerSSB-IndexList-r16 ::=   SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16

ResultsPerSSB-IndexIdle-r16 ::=   SEQUENCE {

ssb-Index-r16                     SSB-Index,

ssb-Results-r16                   SEQUENCE {

ssb-RSRP-Result-r16               RSRP-Range                                                              OPTIONAL,

ssb-RSRQ-Result-r16               RSRQ-Range                                                              OPTIONAL

}                                                                                                     OPTIONAL

}

-- TAG-MEASRESULTIDLENR-STOP

-- ASN1STOP

MeasResultIdleNR field descriptions

carrierFreq Indicates the NR carrier frequency.

measResultIdleNR Idle/inactive or setup measurement results for an NR cell (optionally including beam level measurements).

measResultServingCell Measured results of the serving cell (i.e., PCell) from idle/inactive measurements.

measResultsPerCellListIdleNR List of idle/inactive measured results for the maximum number of reported best cells for a given NR carrier.

resultsSSB-Indexes Beam level measurement results (indexes and optionally, beam measurements).

timeDifferenceReportMeas This filed indicates the time difference between since when the measurement was performed and when the measurement is reported.

END OF CHANGE