3GPP TSG-RAN2 Meeting #124	R2-2312960

Chicago, US, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0735 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for Mission Critical UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA6 has expressed concerns about potential data loss after a silent period during a group call in LS to RAN2 in R2-2309485. It is difficult to meet the SA6 defined KPIs on latency for a mission critical UE if it has been released to RRC_INACTIVE or RRC_IDLE as indicated in R2-2309485. gNB can alleviate the problem by keeping such UEs in RRC_CONNECTED, where the latency requirements can be met more reliably. During congestion the gNB may have to release a mission critical UE to RRC_INACTIVE to ensure continuation of multicast reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A NOTE is added to section 16.10.5.2 where it is clarified that the gNB may decide to not release a mission critical UE based on the 5QI value or other QoS parameter to meet the KPIs requirements specified by SA6. However during congestion, the gNB may decide to release mission critical UEs to RRC_INACTIVE to ensure continuation of multicast reception. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: MBS, MCPTT Inter-operability: There are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Exceeding latency KPIs and/or possible data loss in case mission critical UEs with a mission critical MBS session are released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 16.10.5.2	Configuration

A UE can receive data of MBS multicast session only in RRC_CONNECTED state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB may send RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE.

When there is temporarily no data to be sent to the UEs for a multicast session that is active, the gNB may move the UE to RRC_INACTIVE state. When an MBS multicast session is deactivated, the gNB may move the UE to RRC_IDLE or RRC_INACTIVE state. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC_IDLE or RRC_INACTIVE state when a multicast session has been activated by the CN. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC_INACTIVE state when the session is already activated and the gNB has multicast session data to deliver. Upon reception of the group notification, the UEs reconnect to the network or resume the connection and transition to RRC_CONNECTED state. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in clause 9.2.5. Paging message for group notification contains MBS session ID which is utilized to page all UEs in RRC_IDLE and RRC_INACTIVE states that joined the associated MBS multicast session, i.e., UEs are not paged individually. The UE stops monitoring for group notifications related to a specific multicast session, i.e., stops checking for the MBS session ID in the Paging message, when the UE enters RRC_CONNECTED state. The UE does not monitor for group notifications for these cases, i.e., once this UE leaves this multicast session or the network requests the UE to leave, or the network releases the multicast session.

NOTE:	The gNB may decide, based on the mission critical 5QI value(s) or other QoS parameters of the QoS flow(s) (as specified in TS 23.501 clause 5.7.4) to not release a mission critical UE to ensure that packet loss and delay requirements are met (as specified in TS 22.179 clause 6.15.3). When there is congestion the gNB may decide to release a mission critical UE to RRC_INACTIVE to ensure continuation of multicast reception.

If the UE in RRC_IDLE state that joined an MBS multicast session is camping on the gNB not supporting MBS, the UE may be notified by CN-initiated paging where CN pages each UE individually due to session activation or data availability, as described in clause 9.2.5. If the UE in RRC_INACTIVE state that joined MBS multicast session is camping on the gNB not supporting MBS, the UE may be notified individually by RAN-initiated paging due to session activation or data availability, as described in clause 9.2.5.

<End of modified section>