3GPP TSG-RAN WG2 Meeting #124	R2-2312983

Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.12.2.2

Source:	Ericsson

Title:	Support of UE on-board indication to the network

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we provide our understanding on whether a UE should report to the network whether is an on-board UE or not.

# 2	Discussion

In the RAN2#123 meeting, the following agreement has been reached in the context of IDLE/INACTIVE mode enhancements for mobile IAB:

It is left to UE implementation to determine whether the UE is physically on a moving vehicle and when it applies mobile IAB cell reselection prioritization for agreed scenarios.

Chair Comment: The last agreement is different to and supersedes earlier agreements that states that RAN2 shall specify on-board criterion.

From the agreement, it is clear that RAN2 understanding that a UE can determine on whether is an on-board UE or not based on its own implementation. Nevertheless, the understanding is that is extremely difficult for a UE to determine with a 100% certainty that is inside a moving vehicle.

According to this, while it is indeed fine for the UE to determine by implementation which is an on-board UE and to use such information to perform the cell (re)selection procedure, it is also true that network cannot completely trust a possible indication from the UE that is an on-board UE.

Therefore, there are no benefits for the network is a UE reports that is an on-board UE, as this information is not trustable. Thus, we propose:

A UE which determines by its implementation that is in a moving vehicle, does not report any on-board indication to the network.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE which determines by its implementation that is in a moving vehicle, does not report any on-board indication to the network.

# 4	References