3GPP TSG-RAN WG2 Meeting #124	R2-2313160

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:	7.8.3

Source:	LG Electronics Inc.

Title:	Flight Path Reporting

Document for:	Discussion and Decision

# 1. Introduction

The following is the agreement regarding flight path information reached in the RAN2 # meeting.

RAN2 #120 - A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. - UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. - UAI message can also be used to indicate the UE has flight path availability. RAN2 #121bis - Flightpath update indication in UAI is configurable by the network RAN2 #123 - Confirm that same indication is used for both initial and updated flightpath plan

In this contribution, we continue the discussion on reporting a flight path available indication during HO.

# 2. Discussion

During a HO process, the target cell can obtain flight path information via inter-node signaling. This reduces the need for the UE to report its flight path information to the target cell after completing the handover. However, given the possibility of flight path information changing during HO, the source cell cannot guarantee the information's up-to-dateness.

To obtain the latest flight path information immediately after HO in target cell, it can utilize the UAI 1-second rule. If the UE has provided UE assistance information related to flight path information to the source cell within 1 second before the HO, the UE can retransmit the concerned UE assistance information to the target cell immediately after the HO in accordance with the following procedure.

2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup: 3> if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or … 4> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

Therefore, we propose using the 'UAI 1-second rule' to re-initiate the reporting of the flight path information available indication after HO. This would require the target cell to configure flight path-related configuration in HO message, but it is under network implementation. There is no need to modify the current CR.

Proposal 1. To use the UAI 1-second rule to re-initiate reporting the flight path information available indication after HO. There is no spec impact.

# 3. Conclusion

Follow is our proposal:

Proposal 1. To use the UAI 1-second rule to re-initiate reporting the flight path information available indication after HO. There is no spec impact.

# 4. References

[1] R2-xxx-UAV-RRC-CR-Update-v05_rapp