3GPP TSG-RAN WG2 Meeting #124	R2-2313424

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1725 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the CG-SDT initiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2023-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, for the CG-SDT initiation based on the cg-SDT-TimeAlignmentTimer, the first available CG occasion is used as a reference point to determine if the CG-SDT procedure can be initiated. However, since multiple CG configurations can be configured for CG-SDT associated with different SSB(s), the first available CG occasion may not be a valid CG occasion to transmit the CCCH message according to clause 5.8.2 of TS 38.321. Hence, if the CG-SDT is initiated based on the current specification, the cg-SDT-TimeAlignmentTimer may expire before the first valid CG occasion is available which can be used to transmit the CCCH message. In this case the CG-SDT procedure will immediately fail and based on the SDT failure, UE enters RRC_IDLE mode. This should not be intended behaviour. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the CG-SDT initiation condition based on the valid TA, only the first available valid CG occasion is accounted. Impact analysis Impacted functionality: Small Data Transmission. Inter-operability: - If the network is implemented according to the CR and the UE is not, no inter-operability issues. - If the UE is implemented according to the CR and the network is not, no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may initiate CG-SDT procedure even though the cg-SDT-TimeAlignmentTimer will expire before the first valid CG occasion which can be used to transmit CCCH message. In this case, SDT failure is declared and UE enters RRC_IDLE mode unnecessarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available valid CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialUplinkBWP:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

End of Changes