3GPP TSG RAN WG2#124	R2-2313913

Chicago, USA, 13th - 17th November 2023

3GPP TSG-RAN WG4 Meeting #109	R4-2321371

Chicago, USA, November 13 – November 17, 2023

Title:	Reply LS to RAN2 on UL Timing Adjustment Solutions in HST FR2

Response to:	R2-2311615

Release:	Release 18

Work Item:	NR_HST_FR2_Enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:	Dimitri Gold

E-mail Address:	dimitri.gold@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In LS Reply R4-2314299, RAN4 has provided the answers to the questions from RAN2 and described PC6 UE behaviour depending on 1-bit MAC CE indication, i.e., for Case “0” and Case “1”. RAN2 discussed the topic further and askes from RAN4

“to clarify if it would be possible to design new MAC CE to only correspond to case “1” as presented in R4-2314299.”

Firstly, RAN4 would like to clarify the following RAN2 confusion from LS Reply R2-2311615:

“But it caused confusion in RAN2 because companies think NW can send legacy MAC CE to the UE when NW thinks the timing difference is not large to address the case ”0” in RAN4’s LS”, as UE will not apply large one shot UL timing adjustment.”

In Rel-17, when legacy MAC CE (i.e., MAC CE intended for indicating target TCI state for PDCCH in 6.1.3.15) is received by the PC6 UE, the UE evaluates DL timing difference and may apply large one-shot UL timing adjustment as specified in clause 7.1.2.3 of TS 38.133. Additionally, a longer TCI state switching delay for PC6 UE is specified in clause 8.10.3A.

RAN4 expects that Rel-18 PC6 UE requirements including UL transmit timing and MAC-CE based TCI state switching delay stay the same as they were defined in Rel-17 when legacy MAC CE is received.

RAN4 sees the following options to introduce new MAC CE:

- Option 1:

Follow RAN4’s existing agreement to introduce 1-bit new MAC CE indication corresponding to Case “0” and Case “1” as described in RAN4 LS Reply R4-231422 and specified in version 18.3.0 of Rel-18 TS 38.133.

- If new MAC CE indicates ‘1’, the UE follows the requirements specified in Clauses 7.1.2.3 and 8.10.3A of TS 38.133.

- If new MAC CE indicates ‘0’, the UE doesn’t need to evaluate the DL timing difference and follows requirements from 7.1.2.1, and a shorter TCI state switching delay as specified in 8.10.3 of TS 38.133 applies.

Hence, there are benefits of signaling both ‘1’ and ‘0’.

- Option 2:

Follow RAN2’s suggestion for simplified MAC CE design, by modifying the UE requirements to

- When new MAC-CE is received then UE follows Case “1” as described in LS Reply R4-2314299.

- When new MAC CE is not used, but legacy Rel-15 MAC CE is received, Rel-17 PC6 UE requirements are followed.

The downside of this option is that enhanced PC6 UE requirements corresponding to Case “0” cannot not be applied, i.e., shorter TCI state switching delay and gradual UL timing adjustment at TCI state switch when Rel-17 one shot large timing adjustment is enabled.

2. Actions:

To RAN2 group.

ACTION: 	RAN4 respectfully asks RAN2 to introduce new MAC CE intended for indicating target TCI state for PDCCH in HST FR2 scenario considering the options described above.

3. Date of Next TSG-RAN WG2 Meeting:

RAN4#110	from 2024-02-26	to 2024-03-01		Athens, GR

RAN4#110bis	from 2024-04-15	to 2024-04-19		China (TBC), CN