3GPP TSG-RAN WG2 Meeting #125	R2-2401214

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	7.25.1.6

Source:	China Telecom

Title:	Discussion on SUL issues for ATG

Document for:	Discussion and Decision

- Introduction

The paper aims to clarify the SUL issues for ATG in Rel-18.

- Discussion

Generally, ATG network refers to an in-flight connectivity technique that utilizes ground-based cell towers to transmit signals to the onboard ATG terminal’s aircraft antenna. In Rel-18, RAN4 has discussed the RF requirements for ATG, but the ATG research only focused on the FR1 single carrier scenario [1][2]. And in TS38.104, it has agreed that “there is no multi-band operation requirement for ATG base station” [3]. Per TR 38.876, ATG operates within existing NR operating bands and does not need new bands. Specifically, the following NR bands are agreed for ATG deployment in Rel-18.

Table 5-1: ATG operating bands

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

It can be observed that ATG operation bands do not include the SUL bands, indicating that SUL is not supported for ATG operation in Rel-18.

Observation: In Rel-18, the ATG operation bands do not include the SUL bands indicating that the SUL is not supported for ATG operation.

Based on the above description, it can be known that the ATG operation only support the single carrier. And it has illustrated that CA and DC are not supported for ATG in Rel-18 [3]. However, there is no description of the SUL issues for ATG. Therefore, to add a stage-2 description on SUL is not supported for ATG in this release.

Proposal 1: SUL is not supported for ATG in this release.

Proposal 2: To agree the corresponding CR [4].

- Conclusions

Based on the above analysis, we have the following observation and proposals:

Observation: In Rel-18, the ATG operation bands do not include the SUL bands indicating that the SUL is not supported for ATG operation.

Proposal 1: SUL is not supported for ATG in this release.

Proposal 2: To agree the corresponding CR [4].

- References

- RP-232228, CMCC views on Rel-19 RAN4 topics, RAN#101, CMCC

- R4-2321919, Big CR for TS 38.101-1 for NR ATG, RAN4 #109, CMCC

- R4-2321028, CR for TS 38.104 on adding RF requirements for ATG BS, RAN4 #109, ZTE Corporation, CMCC, CATT

- R2-2401223, Rel-18 CR on Clarification to SUL issues for ATG