3GPP TSG-RAN WG2 Meeting #125	R2-2401308

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.21.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Capability for DPC reporting

WID/SID:	NR_cov_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the UE capability of the DPC reporting.

# 2	Discussion

While the DPC reporting in the PHR formats were agreed, it seems RAN2 did not agree on a UE capability based on which the NW would be able to configure the feature to the UE. RAN4 indicated in their LS to RAN2 in their previous meeting the following (R4-2321998):

- Q2: What is the granularity of the information to be reported (e.g., per UE / per cell / other option)? Answer from RAN4: - The capability of ΔPPowerClass reporting feature is per UE with following options - UE to report ΔPPowerClass reported per serving cell for only single band single CC operation. - UE to report ΔPPowerClass reported per serving cell for only single band single CC operation as well as ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC per configured band combination and ΔPPowerClass per serving cell of configured band combination. - Network configures the reporting of ΔPPowerClass and/or ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC for the overhead reduction purpose.

Hence, a new per UE capability for the DPC should be introduced which is only applicable in FR1.

Proposal: Introduce per UE capability for the DPC reporting in TS 38.306.

# 3	Conclusion

Proposal: Introduce per UE capability for the DPC reporting in TS 38.306.