3GPP TSG- RAN2 Meeting #125	R2-2401611

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	6.2

Title: 	Summary of Rel-17 RRC corrections in AI 6.2

Source: 	Huawei, HiSilicon

Document for:	Discussion and decision

# 1. Introduction

This is to summarize the company contributions in AI 6.2 for RRC related corrections/proposals.

# 2. Discussion

38.331 discussions and CRs

| TDoc number | TDoc title | Source | Change summary | Rapp’s suggestions |
| --- | --- | --- | --- | --- |
| R2-2400648 (discussion) | Discussion on AS condition checking for SUI transmission | OPPO | Remove the AS condition checking of SUI transmission for U2N Relay, to avoid the case U2N Remote UE/ Relay UE can perform U2N Relay communications but the related configurations/resources are not available if the AS condition is not satisfied when triggering SUI reporting. | The rapporteur understands this is to avoid duplicated AS threshold checking for relay communication, but without the change, nothing seems to be broken. Considering there is a higher bar for legacy correction, the rapporteur suggests not to pursue the change. |
| R2-2400690 (CR) | Correction on the SRBs of L2 U2N Remote UE | ZTE, Sanechips | Change #1: In 4.2.1, add the measurement for L2 U2N Relay UE. Change #2: In 4.2.2, clarify that the end-to-end SRBs of L2 U2N remote UE are using PC5 Relay RLC channel and Uu Relay RLC channel. | Change #1 is to add the SL relay related UE behavior in 4.2.1, which is a supplement to cover the SL relay cases. For the changes in 4.2.2, it is correct that the current description about SRB transfer in Uu channel is not applicable to L2 U2N remote UE. But the changes for SRB3/4 are not needed, since SL relay does not support SRB3/4. In the last, the E2E SRB is not defined in Rel-17 for U2N relay, it should be removed. The moderator suggests to adopt change #1 and change #2 about SRB0/1/2. [Editorial] |
| R2-2401109 (CR) | Correction on the SRBs of L2 U2N Remote UE | ZTE, Sanechips | Same change in R2-2400690 for Rel-18 |  |
| R2-2400731 (CR) | Misc RRC corrections for Rel-17 SL relay | Huawei, HiSilicon, OPPO | Change the field name in the text procedure to align with the name in the ASN.1 part. sl-InterestedFreqList -> sl-TxInterestedFreqListDisc sl-InterestedFreqListL2U2N -> sl-TxInterestedFreqListL2U2N | Editorial changes, e.g. to correct wrong field/IE names in procedural text. The moderator suggests to agree the changes and take this CR as rapp CR to cover other editorial corrections. [Editorial] |
| R2-2400732 (CR) | Misc RRC corrections for SL relay | Huawei, HiSilicon, OPPO | Apply the same change in R2-2400731 for Rel-18 |  |
| R2-2400945 (CR) | Correction on logical channel identity | Apple | Removed the mentioning of PC5 Relay RLC channel in the description of LogicalChannelIdentity since there is no logical channel identity configured in SL-RLC-ChannelConfig. | For clarification, in current specification, the NW configures the PC5 relay RLC channel to relay UE and remote UE, and uses the SL-RLC-ChannelID to identify a PC5 relay RLC channel ID. After UE receives the PC5 relay RLC channel configuration, the U2N relay UE or remote UE will associate with PC5 relay RLC channel to a sidelink logical channel, and generate the RRCReconfigurationSidelink message to contain the PC5 relay RLC channel configuration to the Rx side. In the RRCReconfigurationSidelink, the LogicalChannelIdentity is used to identity a sidelink logical channel associated with a PC5 relay RLC channel. So the main idea to say LCID is identify RLC channel on PC5 link is correct, but the field name is wrong, can change “SL-RLC-ChannelConfig” to “SL-RLC-ChannelConfigPC5”. The moderator proposes to change “SL-RLC-ChannelConfig” to “SL-RLC-ChannelConfigPC5” in think we need to update the description of LogicalChannelIdentity based on R2-2400945. [Editorial] |
| R2-2401153 (Discussion) | Clarification on preconfiguration usage in U2N relay | Qualcomm Incorporated | Remote UE and Relay UE is allowed to use preconfiguration if SIB12 does not provide configuration for the concerned frequency. | In current spec, if network supports L2 U2N relay operation, it has to include frequency list in SIB12, otherwise relay UE/remote UE cannot set TxResourceRequest in SUI following the current SUI procedure. So, there is no need to do the change for a nonexistent case. The rapporteur suggests not to pursue the changes. |

Proposal 1: The changes in Rapp Misc CR R2-2400731/ R2-2400732 are agreeable.

Proposal 2: The changes in 4.2.1 and SRB0/1/2 related changes in 4.2.2 in R2-2400690/R2-2401109 are agreeable.

Proposal 3: Base on R2-2400945, Change “SL-RLC-ChannelConfig” to “SL-RLC-ChannelConfigPC5” in the description of LogicalChannelIdentity.

Proposal 4: Not to pursue the changes in R2-2400648 and R2-2401153.

3. Conclusion

38.331 corrections

Proposal 2: The changes in 4.2.1 and SRB0/1/2 related changes in 4.2.2 in R2-2400690/R2-2401109 are agreeable.

Proposal 3: Base on R2-2400945, Change “SL-RLC-ChannelConfig” to “SL-RLC-ChannelConfigPC5” in the description of LogicalChannelIdentity.

Proposal 4: Not to pursue the changes in R2-2400648 and R2-2401153.