3GPP TSG-RAN WG2 Meeting#127bis      					           R2-2408658

Hefei , CN, Oct 14th – 18th, 2024

Source: 			ZTE corporation, Sanechips

Title: 	QoE configuration release during inter-RAT mobility

Agenda item:		7.14.2

Document for: 	Discussion and Decision

# Introduction

This contribution intends to discuss the remaining issues on QoE configuration release during inter-RAT mobility with proposed resolutions.

# Discussion

Mobility from NR to EUTRA

RAN2-127 Agreement: =>UE shall release its stored NR QoE configuration and possible QoE reports/variables for below cases: - Case 1: UE reselects to EUTRA after during RRC connection reestablishment and establish connections to EUTRA - Case 2: UE is in NR RRC IDLE state and then the UE moves to LTE cell. =>We intend to correct this from Rel-18. FFS where (LTE or NR specs) and how to address this in specs.

In RAN2-127 meeting, RAN2 agreed UE should release its stored NR QoE configuration and possible QoE reports/variables when reselecting to EUTRA during RRC connection reestablishment, or when moving from an NR RRC IDLE state to an LTE cell.

---------------------------------------------------------- From 38.331 ---------------------------------------------------------------

5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

[...]

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	inform upper layers about the release of the RAN visible application layer measurement configuration;

2>	discard any RAN visible application layer measurement reports received from upper layers;

2>	initiate the procedure in 5.5b.1.2;

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

[...]

5.5b.1.1	General

This procedure specifies the area handling and storing of application layer measurement report containers by a UE in RRC_IDLE and RRC_INACTIVE that has an application layer measurement configuration with appLayerIdleInactiveConfig configured.

5.5b.1.2	Initiation

While in RRC_INACTIVE and RRC_IDLE state, the UE shall:

1>	store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

NOTE:	The UE continues storing application layer measurement report containers for MBS broadcast service in RRC_CONNECTED as long as stored application layer measurement configurations/reports have not been retrieved.

[...]

---------------------------------------------------------- From 38.331 ---------------------------------------------------------------

It is noticed that based on existing specs UE will perform going to RRC_IDLE as cited above, where UE will goes to subclause 5.5b.1.2 which is dedicated to QoE idle/inactive procedure.

Observation 1: For both agreed cases UE will initiate going to RRC_IDLE where UE will go to subclause 5.5b.1.2 dedicated for idle/inactive QoE procedure.

Therefore the easiest approach to capture above agreements would be to capture in this section that UE shall release its stored NR QoE configuration and possible QoE reports/variables when reselecting to EUTRA cell,m which also avoid impact to LTE specs. Based on above analysis, it is proposed that:

Proposal 1: Capture in 5.5b.1.2 of NR specs that UE shall release its stored NR QoE configuration and possible QoE reports/variables if reselects to EUTRA cell.

An TP have been provided in Annex 1 for P1, it is proposed that RAN2 discusses and agrees on the TP.

Proposal 1a: RAN2 agrees on P1 and adopts the TP in the Annex.

Mobility from EUTRA to NR

RAN2-127 Agreement: =>R2-2407090 is postponed to check if specs already covers properly the needed behaviour

During the RAN2-127 meeting, RAN2 discussed necessity of CR in R2-2407090 and postponed the conclusion to next meeting to allow companies to have a further check if current specs fulfills the proper UE behavior. Based on companies comments received online, below two questions are identified in order to resolve the issue:

- Whether QoE supports LTE-5GC

- Whether UE will automatically perform subclause 5.3.12 based on specs specified in 5.4.3.4 for 5GC cases.

RAN3#121 Toulouse QoE measurement continuity from NR to LTE/5GC and vice versa can be supported by the current Area Scope for QMC IE.

As for question 1, although R2 has never explicitly discussed these. RAN3 do reached above agreements in RAN3#121 which indicates support of LTE/5GC and such scenario is also captured in stage 2 text that QoE measurement collection continuity for intra-system intra-RAT/inter-RAT handover is supported.

------------------------------------------------------------------ From 38300 ----------------------------------------------------------

21.3	QoE Measurement Continuity for Mobility

QoE measurement collection continuity for intra-system intra-RAT/inter-RAT handover is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area scope. A UE continues an ongoing QoE measurement even if it leaves the area scope, unless the network indicates to the UE to release the application layer measurement configuration.

[...]

------------------------------------------------------------------ From 38300 ----------------------------------------------------------

Observation 2: Based on RAN3 agreements and stage 2 text in section 21.3 of 38.300, it implicitly indicates that QoE is supported for LTE-5GC.

With the understanding that 5GC is supported, the next step would be to check if UE will automatically perform subclause 5.3.12 based on specs specified in 5.4.3.4 when connecting to 5GC as cited in below text.

------------------------------------------------------------------ From 36331 ----------------------------------------------------------

5.4.3.4	Successful completion of the mobility from E-UTRA

Upon successfully completing the handover, the cell change order or enhanced 1xRTT CS fallback, the UE shall:

1>	if the targetRAT-Type in the received MobilityFromEUTRACommand is set to eutra (intra-E-UTRA inter-system HO):

2>	indicate to the upper layers associated to the source system the release of the RRC connection together with the release cause 'other';

2>	the procedure ends;

1>	else if the UE was connected to 5GC prior to the reception of the MobilityFromEUTRACommand and the targetRAT-Type in the received MobilityFromEUTRACommand is set to nr:

2>	reset MAC;

2>	stop all timers that are running except T325, T330;

2>	release ran-NotificationAreaInfo, if stored;

2>	release the AS security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if stored;

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP entity for all established RBs;

NOTE 1:	PDCP and SDAP configured by the source configurations RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[..]

5.3.12	UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE

Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall:

[...]

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

[...]

1>	else:

2>	release all radio resources, including release of the MAC configuration, the RLC entity and the associated PDCP entity and SDAP (if any) for all established RBs, except for the following:

-	pur-Config, if stored;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

------------------------------------------------------------------ From 36331 ----------------------------------------------------------

According to the current specs, when the UE leaves from LTE-EPC to NR, the current LTE specs already allow the UE to release the LTE QoE configuration. While for LTE-5GC to NR, there is no such requirement specified. However, some companies believe that when the UE leaves the connected state, it will perform the process in 5.3.12, and there is no need to update the current specs.

Based on the situation we’ve done some digging up on the discussion history and below are what we have found. It is observed that the text highlighted in green was agreed in R2-1818933 which was outcome of post-meeting email discussion [103bis][20]. Based on companies’ comments provided in Q4 in the report of [103bis][20] in R2-1817356[5], the highlighted text is written in this way on purpose with the intention to align UE behavior specified in NR and LTE RRC specs, also to allow UE to keep some PDCP/SDAP configuration for delta configuration for intra-system mobility. The procedure text is further cleaned up in RAN#106 in CR R2-1908518 to make UE behavior consistent without unnecessary cross-RAT interactions.

Observation 3: Current text in subclause 5.4.3.4 when connecting 5GC is to align UE behavior in NR/LTE specs, while also allowing UE to keep some PDCP/SDAP configuration for delta configuration for intra-system mobility as captured in NOTE 1 in 5.4.3.4.

If the UE is assumed to perform actions as specified in 5.3.12 by default after completion of the mobility from E-UTRA while connecting to 5GC, then UE will release all SDAP and PDCP configurations per above cited text highlighted in blue. This behavior contradicts with the intended UE behavior for intra-system inter-RAT mobility as captured in note 1 of subclause 5.4.3.4.

Observation 4: If the UE performs the process in 5.3.12 by default, it should release all SDAP and PDCP configurations which contradicts with the intended UE behavior for intra-system inter-RAT mobility as captured in note 1 of subclause 5.4.3.4.

Based on above observations, we tend to think UE will not perform subclause 5.3.12 by default. However during offline discussion companies still have different view and cannot converge on which ways to go. Considering the situation, it is proposed to discuss online and select among below three options to resolve the issue.

Therefore it is proposed:

Proposal 2: RAN2	discusses and selects among below three options to release QoE configuration during mobility from EUTRA to NR:

- Option1: Capture UE behavior in chairman’s notes with no specs impact.

- Option 2: Capture UE behavior in procedure text as in R2-2407090.

- Option 3: Capture UE behavior in a note: "NOTE x: Release all radio resource as specified in this subclause includes release of application layer measurement configurations configured by source RAT (e.g., serviceType) prior to the handover, including informing upper layers about release of the application layer measurement configurations and discarding application layer measurement reports if any. "

# Conclusion and proposals

Based on above analysis, we have the following proposals:

Observation 1: For both agreed cases UE will initiate going to RRC_IDLE where UE will go to subclause 5.5b.1.2 dedicated for idle/inactive QoE procedure.

Proposal 1: Capture in 5.5b.1.2 of NR specs that UE shall release its stored NR QoE configuration and possible QoE reports/variables if reselects to EUTRA cell.

Proposal 1a: RAN2 agrees on P1 and adopts the TP in the Annex.

Observation 2: Based on RAN3 agreements and stage 2 text in section 21.3 of 38.300, it implicitly indicates that QoE is supported for LTE-5GC.

Observation3: Current text in subclause 5.4.3.4 when connecting 5GC is to align UE behavior in NR/LTE specs, while also allowing UE to keep some PDCP/SDAP configuration for delta configuration for intra-system mobility as captured in note 1 in 5.4.3.4.

Observation 4: If the UE performs the process in 5.3.12 by default, it should release all SDAP and PDCP configurations which contradicts with the intended UE behavior for intra-system inter-RAT mobility as captured in note 1 of subclause 5.4.3.4.

Observation 5: With existing text UE shall also release QoE configurations configured by source RAT, which can be captured in a note for clarification instead of changing existing procedure description.

Proposal 2: RAN2	discusses and selects among below three options to release QoE configuration during mobility from EUTRA to NR:

- Option1: Capture UE behavior in chairman’s notes with no specs impact.

- Option 2: Capture UE behavior in procedure text as in R2-2407090.

- Option 3: Capture UE behavior in a note: "NOTE x: Release all radio resource as specified in this subclause includes release of application layer measurement configurations configured by source RAT (e.g., serviceType) prior to the handover, including informing upper layers about release of the application layer measurement configurations and discarding application layer measurement reports if any. "

# Reference

- R2-2407735 [AT127][504][QoE] Release of QoE configurations (ZTE)

- TS 38.331-i20

- TS 36.331-i20

- 36331_CR3808_(Rel-15)_R2-1818933 CR to 36331 on release after completion of inter-RAT HO Huawei

- R2-1817356 Report of [103bis#20][NR] Inter-RAT HO

# Annex 1 TP for P1

- --------------------------------------- change to TS 38331 -----------------------------------------

5.5b.1.1	General

This procedure specifies the area handling, storing of application layer measurement report containers and releasing  application layer measurement configurations/reports by a UE in RRC_IDLE and RRC_INACTIVE that has an application layer measurement configuration with appLayerIdleInactiveConfig configured.

5.5b.1.2	Initiation

While in RRC_INACTIVE and RRC_IDLE state, the UE shall:

1>	store any previously or subsequently received application layer measurement report containers associated with the measConfigAppLayerId for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers;

NOTE:	The UE continues storing application layer measurement report containers for MBS broadcast service in RRC_CONNECTED as long as stored application layer measurement configurations/reports have not been retrieved.

1>	if the memory reserved for storing application layer measurement report containers becomes full:

2>	if the reports are associated with a configuration including appLayerMeasPriority:

3>	discard reports in priority order where reports with the lowest priority are discarded first, and among reports with equal associated priority, discard reports in the order they were received, where older reports are discarded first;

2>	else:

3>	discard reports in the order they were received, where older reports are discarded first;

1>	if qoe-AreaScope is configured:

2>	if the cell where the UE receives the service when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the cell where the UE receives the service when the UE transits from RRC_CONNECTED state to RRC_INACTIVE or RRC_IDLE state is not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.

2>	if the UE re-selects to a new cell which is part of the area indicated by qoe-AreaScope and the previous cell was not part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being inside the area;

2>	if the UE re-selects to a new cell which is not part of the area indicated by qoe-AreaScope and the previous cell was part of the area indicated by qoe-AreaScope:

3>	inform upper layers of being outside the area.

1>	if the UE re-selects to an E-UTRA cell, the UE shall for each stored application layer measurement configuration:

2>	discard stored application layer measurement reports, if any;

2> inform upper layers about the release of the application layer measurement configurations;

2>	release the application layer measurement configurations including their fields in UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

2>	consider itself not to be configured to send application layer measurement reports.

--------------------------------------- change to TS 38331 -------------------------------------------