3GPP TSG-RAN WG2 Meeting #129	 R2-2500244

Athens, Greece, Feb. 17th - 21st , 2025

Source:	CATT, Turkcell

Title:	Simulation results of event A3 prediction

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, the following agreements were reached on indirect measurement event prediction for temporal domain case A and temporal domain case B:

Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

And the calculation method for F1 score was discussed with the following agreements:

4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2

In this document, we focus on indirect measurement event prediction for temporal domain case A and case B, and the simulation results of F1 score based on the cell level results (sub case 2) are provided.

Discussion

2.1	F1 score

The F1 score is the harmonic mean of precision and recall, and its value range is from 0 to 1. The higher the F1 score, the better the performance of the AI model. According to the formula, Precision refers to the proportion of true positive samples among all samples predicted as positive; Recall refers to the proportion of samples correctly predicted as positive among all samples that are actually positive. In AI prediction for A3 event, Precision means when AI predicts an A3 event, real A3 events may be triggered with Precision percentage. Recall means when a real A3 event is triggered, the AI model will predict an A3 event with Recall percentage.

Figure 1 shows the trend between Precision, Recall and F1 score assuming Precision and Recall range from 0 to 1. When Recall is fixed, the higher of Precision, the higher of the F1 score and vice versa. Likely, when Precision is fixed, the higher of Recall, the higher of the F1 score and vice versa. In the following sections, we calculate the F1 score for both for temporal domain case A in FR2 and for temporal domain case B for FR1 in AI mobility.

Observation 0a: When Recall is fixed, the higher of Precision results in higher F1 score and vice versa

Observation 0b: When Precision is fixed, the higher of Recall results in higher F1 score and vice versa.

Figure 1: Relationship between Precision, Recall and F1 score

2.2	FR2 to FR2 temporal domain case A

Based on the RAN2 agreements, PW with 400ms and TTT with 320ms are used as baseline. In our simulation, sliding L1/L3 filtering is used, and the OW length is set to 400ms and 800ms, i.e., the case of 5N:5N and 10N:5N (N represents a measurement period) are simulated. The case of 5N:5N is shown in Figure 2. And the basic simulation assumption is shown in Table 1.

Figure 2: RSRP predication for measurement event prediction for temporal domain case A

Table 1: Agreed values for Case A for indirect prediction

| Parameters | baseline value |
| --- | --- |
| A3 event offset (db) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 90 |
| OW length (ms, note1) | 400ms/800ms |
| PW length (ms, note1) | 400 |
| Max ETD (ms, note1) | 80 |

If PW with 400ms and TTT with 320ms are applied in the simulation, there are 5 predicted RSRP values in PW and 4 predicted RSRP values within TTT. In RAN2#127bis meeting, it was agreed that:

- Companies can provide multiple real time RSRP value(s) and/or average RSRP value over the entire window and should indicate in their simulation results what they have used.   The companies should at least provide the results of only one value it should be the last value at the end of the PW.   We will add two columns in the spreadsheet to capture the last  value and the average value.

Correspondingly, the following 2 options are provided on how to select the target cell:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

Normally, when A3 event is triggered, for network-sided model, the UE may report the latest RSRP of the corresponding measurement object to the network in the measurement report, and the network can select the target cell based on the UE measurement report. And Option 2 follows this principle. However, unlike actual measurement, in AI prediction, the accuracy of RSRP difference increases as the prediction point index increases within the prediction window based on our simulation results [1]. I.e. the prediction accuracy decreases as the prediction point index increases within TTT. For UE sided-model, the UE can average the RSRP within the TTT and this averages the prediction accuracy and makes the prediction more precise. However, considering L3 RSRP has been filtered, Option 1 may bring over-filtering to the RSRP measurement results. We choose Option 2 in our simulation.

According to the analysis above, it is proposed that:

Proposal 1: For indirect measurement event prediction, when there are multiple predicted RSRP values within TTT, the following 2 options to select target cell can be considered:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

The simulation parameters and results are provided in Table 2 and Table 3.

Table 2 Simulation parameters and results for indirect measurement event prediction for temporal domain case A

| Report parameters |  | CATT |  |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |
|  | UE speed (30,60,90,120 Km/h) | 90 |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |
|  | Whether LOSsoft is modeled or not | Yes |  |
|  | spatial consistency option (A or B) | Procedure A |  |
|  | Number of TX beams | 32 |  |
|  | Number of RX beams | 4 |  |
|  | Measurement reduction rate(50%,…Note2) | N/A |  |
|  | Observation window(Note3) | 400ms | 800ms |
|  | Prediction window (Note3) | 400ms |  |
|  | Any other parameters (Note 4) | N/A |  |
| Data Size (Number of Samples) | Training/validity | ≈194K/48K |  |
|  | Testing | ≈48K |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |  |
|  | Model complexity in a number of parameters(M) | 5.601M |  |
|  | Model complexity in model size (e.g. Mbyte) | 64.138MB |  |
|  | Computational complexity [FLOPs] | 23.44M | 55.09M |
| Metrics | OW vs. PW | [400ms, 400ms] | [800ms, 400ms] |
|  | Average L3 cell level RSRP difference (dB) | 1.459 | 1.427 |

Table 3 F1 score/Precision/Recall for temporal domain case A

| [OW, PW](ms) | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| [400, 400] | 0.8946 | 0.8905 | 0.8925 |
| [800, 400] | 0.9438 | 0.9414 | 0.9462 |

Based on the simulation result above, we find that for temporal domain case A:

- Precision is around 0.90 for case OW vs. OP = [400ms, 400ms]. It means when AI predicts an A3 event, real A3 events may be triggered with around 90%. I.e. when 10 A3 events are predicted, 9 real A3 events are triggered and 1 predicted A3 event is falsely predicted.

- Recall is around 0.90. It means when a real A3 event is triggered, the AI model will predict an A3 event with 90%. I.e. when 10 real A3 events are triggered, the AI model will predict 9 A3 events and 1 real A3 event is missed.

- Based on the Precision and Recall, F1 score is around 0.90.

- When OW increases from 400ms to 800ms, the prediction accuracy increases which leads to Precision, Recall and F1 score increases correspondingly.

Observation 1: For indirect measurement event prediction for temporal domain case A with 400ms/800ms OW and 400ms PW via sliding L1/L3 filtering, it was observed that:

- Precision, Recall and F1 score are around 0.90;

- Precision, Recall and F1 score increases as OW increases.

In addition, in order to show the influence of the size of ETD on F1 score, the results of F1 score with different ETDs are illustrated in Figure 3.

Figure 3: F1 score with different ETD

The simulation results show that F1 score increases slowly with the increase of ETD. Even though the ETD is enlarged to 500ms, only 0.898 for F1 score is achieved which is still below 0.9.

Observation 2: F1 score increases slowly with the increase of ETD, and when ETD increases to 500ms, F1 score is still around 0.9 for indirect measurement event prediction for temporal domain case A.

Based on the above simulation result, F1 score remains relatively stable with the increase of ETD, and it can be concluded that at the time when both actual measurement event and prediction measurement event satisfy the A3 event, incorrect selection of target cell can cause false event detection and missed event detection. This is caused by the inconsistency between the real target cell and the predicted target cell due to AI prediction errors.

2.3	FR1 to FR1 temporal domain case B

Temporal domain case B can be used to predict the measurement result(s) in the future time instance by historical measurement(s). Based on the post #127 email discussion [2], two examples are provided. In our simulation, we use Example 2 for Case B, and skipping pattern is shown in Figure 4.

Figure 4: 2 Skipping pattern of intra-frequency temporal domain case B

Furthermore, 3 filtering options were agreed in the last RAN2 meeting. In our simulation, we use filtering option 3.

Filtering option 1: L3 filtering is based on its L1 filtered result and the immediate last skipped measurement result ; Filtering option 2: L3 filtering is based on its L1 filtered result i.e. no L3 filtering if the immediate last result is skipped; Filtering option 3: L3 filtering is based on the L1 filtered result and last actual measurement result i.e. the skipped result(s) in between is ignored.

In RAN2 meeting, agreements on A3 event prediction for temporal domain Case B were agreed as shown in Table 4. Based on the agreements, it can be seen that PW with 400ms and TTT with 320ms are used the baseline.

Table 4: Agreed values for Case B for indirect prediction

| Parameters | baseline value | Note |
| --- | --- | --- |
| A3 event offset (db) | 2 | Open for 3db |
| TTT (ms) | 320 | Open for one shorter value |
| UE speed (km/h) | 30 | Open for 60 and 90km/h |
| OW length (ms,note1) | N/A | Up to implementation |
| PW length (ms,note1) | 200 (non-sliding) | Open for more values |
| Max ETD (ms, note1) | 80 | Open for more values |
| MRRT | 50% | Open for more values |

The simulation parameters and results for indirect measurement event prediction for temporal domain case B are shown in Table 5.

Table 5 Simulation parameters and results for indirect measurement event prediction for temporal domain case B

| Report parameters |  | CATT |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 2 |  |  |
|  | UE speed (30,60,90,120 Km/h) | 30Km/h |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A |  |  |
|  | Whether LOSsoft is modeled or not | Yes |  |  |
|  | spatial consistency option (A or B) | Procedure A |  |  |
|  | Number of TX beams | 4 |  |  |
|  | Number of RX beams | 1 |  |  |
|  | Measurement reduction rate(50%,…Note2) | 50% |  |  |
|  | Observation window(Note3) | N/A |  |  |
|  | Prediction window (Note3) | N/A |  |  |
|  | Any other parameters (Note 4) | N/A |  |  |
| Data Size (Number of Samples) | Training/validity | 72000，9000 |  |  |
|  | Testing | 9000 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP of all cells |  |  |
|  | Model output(Note 6) | Cell level RSRP of all cells |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | MLP |  |  |
|  | Model complexity in a number of parameters(M) | 0.9167M |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 0.086MB |  |  |
|  | Computational complexity [FLOPs] | 19.68M |  |  |
| Metrics | MRRT | 50% |  |  |
|  | Average L3 cell level RSRP difference (dB) | 0.313 |  |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | Precision | Recall | F1 score |
|  |  | 0.8594 | 0.5699 | 0.6854 |

Based on the simulation result above, we find that the Precision, Recall and F1 score of indirect measurement event prediction for temporal domain case B are around 0.85, 0.56 and 0.68, respectively. And it can be seen that Precision of temporal domain Case B (0.8594) is almost the same as that of temporal domain Case A (0.8946).

Observation 3: Precision for temporal domain Case B is almost the same as that of temporal domain Case A which is around 0.9.

But Recall for temporal domain Case B (0.5699) is relatively low compared with that of temporal domain case A (0.8905). It means when an actual A3 event is triggered, only 56.99% of events are predicted by AI model, i.e. lots of A3 events were missed by AI model. In order to investigate the reason, we gathered the statistic of RSRP difference within 2dB and 3dB between serving cell and target cell. If the RSRP difference is within 2dB and 3dB, a slight error deviation in AI prediction may lead missed event detection which results in larger Recall. And we found that proportion of handover number with RSRP difference within 2dB and 3dB between serving cell and target cell for temporal domain Case B is 75% while it is 32% for temporal domain Case A. This means, prediction for temporal domain Case B is more easily to be impacted by inaccuracy of AI model. Even the average RSRP difference is 0.313 dB for temporal domain Case B, it may lead to A3 event leaving or TTT starting with delay more easily, which results in larger missed event detection (n2) and larger Recall correspondingly for temporal domain Case B.

Observation 4a: The RSRP difference within range (2dB, 3dB) between serving cell and target cell is higher for temporal domain Case B than that of temporal domain Case A.

Observation 4b: Smaller RSRP difference between serving cell and target cell leads to A3 event leaving or TTT starting with delay more easily for temporal domain Case B.

Observation 4c: Missed event detection occurs more easily which leads larger n2 and smaller Recall for temporal domain Case B.

Based on Observation 0a/0b, it can be seen that smaller Recall will lead to lower F1 score even Precision remains the same value. Hence, it can be seen that F1 score for temporal domain Case B is smaller than that of for temporal domain Case A.

Observation 4d: F1 score for temporal domain Case B is smaller than that of for temporal domain Case A.

We checked the impacts of ETD length to F1 score and the simulation results with an ETD of 0ms, 80ms, 200ms and 400ms are shown in Table 6.

Table 6 Simulation parameters and results with different ETD for indirect measurement event prediction for temporal domain case B

| ETD (ms) | Precision | Recall | F1 score |
| --- | --- | --- | --- |
| 0 | 0.8594 | 0.5699 | 0.6854 |
| 80 | 0.8594 | 0.5699 | 0.6854 |
| 200 | 0.9688 | 0.6425 | 0.7726 |
| 400 | 0.9766 | 0.6477 | 0.7788 |

It can be seen that the Precision, Recall and F1 score increases as ETD increases, especially for Precision. It means as ETD increases, more actual A3 events are triggered during the ETD. But it does limited help to Recall. This is because AI prediction inaccuracy leads to high missed event detection (n2), even A3 is triggered and it may leave the A3 which is impacted by ETD length.

Observation 5: As ETD increases, for temporal domain Case B:

- Precision increases rapidly;

- Recall increases with small gain;

- F1 score increases.

Considering non-sliding option is selected for our simulation, there is one RSRP every 200ms. Consequently, Predictions with ETD which is smaller than 200ms have the same effect to F1 score. Based on the simulation results, it can be seen that Precision, Recall and F1 score is the same between the case of ETD with 0ms and 80ms which proves the analysis.

Observation 6: With non-sliding option, Precision, Recall and F1 score keeps unchanged with ETD which is smaller than 200ms which is the L3 sample period for temporal domain Case B.

Conclusion

In this document, we shows the simulation results of F1 score for indirect measurement event prediction for temporal domain case A and case B, and it can be seen that:

F1 score:

Observation 0a: When Recall is fixed, the higher of Precision results in higher F1 score and vice versa

Observation 0b: When Precision is fixed, the higher of Recall results in higher F1 score and vice versa.

Temporal domain case A:

Proposal 1: For indirect measurement event prediction, when there are multiple predicted RSRP values within a PW/TTT, the following 2 options to select target cell can be considered:

- Option1: the average RSRP within the TTT;

- Option2: the last RSRP value at the end of the TTT.

Observation 1: For indirect measurement event prediction for temporal domain case A with 400ms/800ms OW and 400ms PW via sliding L1/L3 filtering, it was observed that:

- Precision, Recall and F1 score are around 0.90;

- Precision, Recall and F1 score increases as OW increases.

Observation 2: F1 score increases slowly with the increase of ETD, and when ETD increases to 500ms, F1 score is still around 0.9 for indirect measurement event prediction for temporal domain case A.

Temporal domain case B:

Observation 3: Precision for temporal domain Case B is almost the same as that of temporal domain Case A which is around 0.9.

Observation 4a: The RSRP difference within range (2dB, 3dB) between serving cell and target cell is higher for temporal domain Case B than that of temporal domain Case A.

Observation 4b: Smaller RSRP difference between serving cell and target cell leads to A3 event leaving or TTT starting with delay more easily for temporal domain Case B.

Observation 4c: Missed event detection occurs more easily which leads larger n2 and smaller Recall for temporal domain Case B.

Observation 4d: F1 score for temporal domain Case B is smaller than that of for temporal domain Case A.

Observation 5: As ETD increases, for temporal domain Case B:

- Precision increases rapidly;

- Recall increases with small gain;

- F1 score increases.

Observation 6: With non-sliding option, Precision, Recall and F1 score keeps unchanged with ETD which is smaller than 200ms which is the L3 sample period for temporal domain Case B.

Reference

- R2-2409651, Simulation results of RRM Measurement Prediction, CATT, Turkcell;

- R2-2407885, Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO), Oppo.