3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500249

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on Asymmetric DL sTRP/UL mTRP

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- 1. Introduction

For Rel-19 MIMO, during the RAN2#128 meeting, the following agreements are made for Asymmetric DL sTRP/UL mTRP [1].

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In this contribution, we will further discuss the detail spec impacts for asymmetric DL sTRP/UL mTRP deployment.

- 2. PL offset

In RAN1#118 meeting, RAN1 discussed the issue of PL offset configuration and update, and sent the LS [2] to RAN2 as follows:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In the following, we will analyze the MAC CE impact for the asymmetric DL sTRP/UL mTRP deployment scenarios.

What channels are applicable to

During the last RAN2 meeting, a new MAC CE was agreed to be introduced for PL offset update for asymmetric DL sTRP/UL mTRP. And for this MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset. Based on RAN1 agreements, the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. For example, for SRS, the DCI-indicated TCI state is applied if followUnifiedTCI-StateSRS is configured, otherwise for periodic SRS resource TCI state is configured by RRC, and for SP/AP SRS resource TCI state is indicated by MAC CE.

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

How to indicate the associate TCI state(s) in the new MAC CE

Based on the above discussion, the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH. Different channel may have different TCI state indication methods. Thus, it’s better to indicate the TCI state ID in the new PL offset updated MAC CE to explicitly indicate the PL offset updated associate with which TCI state.

Proposal 2: TCI state ID is included in the new MAC CE.

Other fields in the new MAC CE

Similarly with the joint or separate TCI state(s) activation/deactivation MAC CE, the Serving Cell ID is needed in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits. Moreover, DL/UL BWP ID is also needed in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212. DL BWP ID is referred to the joint TCI state(s) which PL offset associated with, while UL BWP ID is referred to the UL TCI state(s). However, the PL offset is updated after the corresponding joint or UL TCI state(s) activation. And a UE cannot be configured joint and separate TCI states simultaneously. Thus, the UE knows what kind of TCI state(s), i.e., joint or UL, is associated with the PL offset in the new MAC CE. Only including one field of DL/UL BWP ID is enough in the new MAC CE. The length of the DL/UL BWP ID field is 2 bits.

Proposal 3: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 4: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

- 3. 2TA for single-DCI mTRP

During RAN1#119 meeting, RAN1 sent another LS[3] to check whether the following agreement would cause any MAC impact.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

From RAN2 perspective, the current 2TA operation is associated with the TA of a TAG. For example, there is no description of restriction on coresetPoolIndex in MAC spec. Thus, there is no big MAC impact due to 2TA for single-DCI mTRP. There may be some impacts on RRC spec for the field description, e.g., the coresetPoolIndex restriction and a single n-TimingAdvanceoffset configuration.

Observation 1: There is no big MAC impact on 2TA for single-DCI mTRP.

Proposal 5: RAN2 confirm the RAN1 agreements on 2TA for single-DCI mTRP and further discuss MAC and RRC impacts during Stage 3 running CR phase on the following aspects:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- 4. PHR

For PHR aspect, RAN1 has made the following agreements.

RAN1#118 Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

From RAN2 perspective, whether PHR trigger is impacted for the asymmetric DL sTRP/UL mTRP scenarios should be discussed. In TS 38.321 clause 5.4.6, the key trigger of PHR is that any path loss changes more than a threshold.

### 5.4.6 Power Headroom Reporting … A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

In asymmetric DL sTRP/UL-only mTRP scenario, for a joint/UL TCI state configured with a PL offset, the UL transmission power is determined based on the PL and the PL offset of the PL RS. Thus, the PHR should be triggered when the change of PL of a PL RS is larger than a threshold, where the PL takes into account the PL offset of the PL RS.

The PL value for the PL RS is calculated within the PHY layer, which is transparent to the MAC layer. Thus, the PL used to determine the PHR trigger would already be taken into account the PL offset in PHY layer. There is no spec impact on PHR triggering condition in MAC.

Proposal 6: For PHR trigger, if PL offset is configured to joint/UL TCI state(s) associated to a PL RS, the PL offset of a joint/UL TCI state is applied in PHY layer to determine the PL change of the PL RS associated to the joint/UL TCI state. There is no spec impact on PHR triggering condition in MAC.

- 5. Conclusion

This paper discusses the spec impacts for Asymmetric DL sTRP/UL mTRP. The contribution of this paper is summarized as follows.

Observation 1: There is no big MAC impact on 2TA for single-DCI mTRP.

Proposal 1: The PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 2: TCI state ID is included in the new MAC CE.

Proposal 3: Serving Cell ID is included in the new MAC CE to indicate the identity of the Serving Cell for which the MAC CE applies. The length of the Serving Cell ID field is 5 bits.

Proposal 4: One field of DL/UL BWP ID is included in the new MAC CE to indicate a DL/UL BWP for which the MAC CE applies. The length of the DL/UL BWP ID field is 2 bits.

Proposal 5: RAN2 confirm the RAN1 agreements on 2TA for single-DCI mTRP and further discuss MAC and RRC impacts during Stage 3 running CR phase on the following aspects:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

Proposal 6: For PHR trigger, if PL offset is configured to joint/UL TCI state(s) associated to a PL RS, the PL offset of a joint/UL TCI state is applied in PHY layer to determine the PL change of the PL RS associated to the joint/UL TCI state. There is no spec impact on PHR triggering condition in MAC.

- 6. Reference

- R2-2410912	, Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD,	Vice Chairman (CATT) report

- R2-2407906, LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5 (R1-2407285; contact: Samsung)

- R2-2500008, LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 (R1-2410758; contact: Samsung)