3GPP TSG-RAN WG2 Meeting #129	 R2-2500283

Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.4.3

Source: 	NEC

Title:  	Discussion on LP-WUS RRM relaxation and offloading

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#128 meeting, the following agreement are made:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

In this contribution, we further discuss the design of relaxation/offloading criterion for serving cell and neighbor cell.

- 2 Discussion

- 2.1 measurement relaxation

Fig.1 relationship between relaxation threshold and entry/exit condition

In this section, we will discuss how to design measurement relaxation criterion based on the framework of Fig.1.

MR serving cell measurement relaxation

Basically there are two scenarios as shown in Fig.1, for scenario-1 (i.e., the relaxation threshold is lower than the entry condition of LP-WUS monitoring), once the UE has fulfilled relaxation criterion but still not fulfilled entry condition of LP-WUS monitoring, the UE could perform relaxed MR serving cell measurement without entering LP-WUS monitoring mode. It is noted that in Rel-16 measurement relaxation discussion, serving cell measurement relaxation was also discussed but the answer is NO, as serving cell measurement is really important, companies don’t want to sacrifice possible UE experience degradation for power saving reason. The same principle could be used for LP-WUS, it is not promising to let non-LP-WUS monitoring UE perform relaxed MR serving cell measurement relaxation.

Observation-1: if the threshold of serving cell RRM relaxation is lower than the one of LP-WUS monitoring entry condition, the non-LP-WUS monitoring UE could perform relaxed MR serving cell measurement, which is not promising.

For scenario-2 (i.e., the relaxation threshold is greater than the entry condition of LP-WUS monitoring), once the UE has fulfilled the entry condition of LP-WUS monitoring mode but still not fulfilled relaxation criterion, the LP-WUS monitoring UE still need to perform normal MR serving cell measurement, based on measurement requirement specified by RAN4, normal MR serving cell measurement can be every I-DRX (e.g., 320/640ms), it is not acceptable that a LP-WUS monitoring UE should turn on MR every 320/640ms for normal measurement, such frequent MR transition could overkill the power saving gain, also noted that MR transition duration should be 400/800ms, the MR cannot even enter ultra-deep-sleep mode.

Observation-2: if the threshold of serving cell RRM relaxation is greater than the one of LP-WUS monitoring entry condition, the LP-WUS monitoring UE may perform normal MR serving cell measurement (of which measurement period could be every I-DRX cycle), which is not acceptable.

Based on the above analysis, it is promising that LP-WUS monitoring and serving cell measurement relaxation should be performed simultaneously, therefore, we propose:

Proposal-1: consider entry condition of LP-WUS monitoring as the entry condition for MR serving cell relaxation. Whether LR measurement is needed can follow the design on entry condition of LP-WUS monitoring.

Proposal-2: consider exit condition of LP-WUS monitoring as the exit condition for serving cell MR relaxation. Whether MR and/or LR measurement is needed can follow the design on exit condition of LP-WUS monitoring.

MR neighbour cell measurement relaxation

RAN2 has agreed that not-at-cell-edge is reused for neighbour cell measurement relaxation, based on this understanding, we need to further discuss the relationship between MR neighbour cell relaxation criterion and entry/exit condition of LP-WUS monitoring.

For scenario-1 (i.e., the relaxation threshold is lower than entry condition of LP-WUS monitoring), once the UE has fulfilled relaxation criterion but not fulfilled entry condition of LP-WUS monitoring, the UE could perform MR neighbour cell measurement relaxation without entering LP-WUS monitoring mode. In this scenario the UE behaviour is just similar to the legacy Rel-16 measurement relaxation. Difference is that LP-WUS introduced MR neighbour cell measurement relaxation is performed more relaxed, i.e., the relaxed factor K is greater than the legacy value (i.e., K=3). We understand it is feasible but related to RAN4 requirement, i.e., a further MR neighbour cell measurement relaxation (e.g., K > 3) will be introduced for the UE without entering LP-WUS monitoring.

Proposal-3: if the threshold of neighbour cell relaxation is lower than that one of LP-WUS monitoring entry condition, RAN2 to confirm it is feasible that the UE may perform measurement relaxation defined by LP-WUS feature without entering LP-WUS monitoring mode, as like Rel-16 measurement relaxation behaviour but with a further relaxed factor determined by RAN4.

For scenario-2 (i.e., the relaxation threshold is greater than entry condition), once the UE has fulfilled entry condition1 of LP-WUS monitoring but not fulfilled relaxation criterion, the LP-WUS monitoring UE may perform normal MR neighbour cell measurement. In this scenario, based on RAN4 requirement (taking intra-frequency as an example), the LP-WUS monitoring UE shall perform normal MR neighbour cell measurement every Tmeasure,NR_Intra and the typical value is 1280ms. Considering the MR transition duration may be 400/800ms, technically speaking, it still can work but power saving gain may be marginal.

Proposal-4: if the threshold of neighbour cell relaxation is greater than that one of LP-WUS monitoring entry condition, RAN2 to confirm it is feasible that the LP-WUS monitoring UE can perform normal neighbour cell MR measurement, but the power saving gain may be marginal.

- 2.2 measurement offloading

Background:

Case #1 have been agreed in RAN4

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

In this section we discuss the relationship between MR serving cell and MR neighbour cell measurement offloading, based on RAN4 understanding, it is assumed that both should be performed simultaneously, but one detail is that case#1 identified by RAN4 does not preclude an intermediate state (i.e., serving cell MR measurement is OFF, FFS the condition of neighbour cell).

The reason why the UE need to perform neighbour cell measurement is to ensure the mobility, i.e., cell re-selection in RRC_IDLE/INACTIVE. Our understanding is that once the UE has fulfilled serving cell measurement offloading criterion (i.e., case#1, MR serving cell measurement offloading is off), it means the signal strength/quality is already good enough, in this scenario, there will be no mobility issue. As MR serving cell measurement has been stopped, let along neighbour cell MR measurement. Therefore neighbour cell MR measurement offloading should also be performed (i.e., FFS in case#1 can be removed).

Proposal-5: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements are offloaded.

- 3 Conclusions

Observation-1: if the threshold of serving cell RRM relaxation is lower than the one of LP-WUS monitoring entry condition, the non-LP-WUS monitoring UE could perform relaxed MR serving cell measurement, which is not promising.

Observation-2: if the threshold of serving cell RRM relaxation is greater than the one of LP-WUS monitoring entry condition, the LP-WUS monitoring UE may perform normal MR serving cell measurement (of which measurement period could be every I-DRX cycle), which is not acceptable.

Proposal-1: consider entry condition of LP-WUS monitoring as the entry condition for MR serving cell relaxation. Whether LR measurement is needed can follow the design on entry condition of LP-WUS monitoring.

Proposal-2: consider exit condition of LP-WUS monitoring as the exit condition for serving cell MR relaxation. Whether MR and/or LR measurement is needed can follow the design on exit condition of LP-WUS monitoring.

Proposal-3: if the threshold of neighbour cell relaxation is lower than that one of LP-WUS monitoring entry condition, RAN2 to confirm it is feasible that the UE may perform measurement relaxation defined by LP-WUS feature without entering LP-WUS monitoring mode, as like Rel-16 measurement relaxation behaviour but with a further relaxed factor determined by RAN4.

Proposal-4: if the threshold of neighbour cell relaxation is greater than that one of LP-WUS monitoring entry condition, RAN2 to confirm it is feasible that the LP-WUS monitoring UE can perform normal neighbour cell MR measurement, but the power saving gain may be marginal.

Proposal-5: once the criteria for MR serving cell measurement offloading is met, then both MR serving cell and neighbour cell measurements are offloaded.

- 4 References