3GPP TSG-RAN WG2 Meeting #129	R2-2500342

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Remaining issues on PWS support for NB-IoT

Agenda Item:	8.9.4

Document for:	Discussion and decision

# Introduction

The WID [1] supports PWS for NB-IOT with the following description:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In the RAN2#128 meeting, the following agreements were achieved:

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

In this contribution, we provide our consideration on the remaining issues to support PWS for NB-IoT.

# Discussion

## 2.1 Whether to acquire SIB1-NB for reception of PWS SIB

It is important to note that there are specific requirements for delivery of ETWS message, as outlined in TS 22.268 [2]:

-	Quick Warning Notification delivery after the occurrence of Earthquake or Tsunami.

Earthquake and Tsunami propagate very fast. The duration time between the actual occurrence of the disaster and its arrival is very short. The order of the duration time is around seconds or minutes at most. Therefore the Warning Notifications shall be delivered quickly to the users in the emergency impacted area so that they could take any actions to escape from danger.

The amount of data included in the ETWS primary notification is limited to a few bytes to ensure efficient delivery. However, the ETWS secondary notification and CMAS notification are expected to contain elements that instruct the UE to take specific actions, which may involve larger data volume:

-	Event Description

-	Area Affected

-	Recommended Action

-	Expiration Time (with time zone)

-	Sending Agency

Besides, additional elements may be included, based on regulatory requirements. In such cases, the message may need to be delivered in multiple segments, which could result in longer latency.

It is also observed that there is growing necessity to ensure that the public needs the capability to receive the alerts, warnings and critical information regarding emergencies timely and accurately. This is essential for taking prompt actions to avoid serious injury, property loss or even threat to life.

There is an FFS issue from the last meeting on whether SIB1-NB can be skipped when acquiring the PWS related SIB(s). From our point of view, it is beneficial to skip SIB1-NB to decrease the corresponding waiting period and processing time. The most straightforward approach would be for the NW to send the scheduling information in advance and indicate whether the PWS SIBs are being broadcast or not, similar to NR on-demand SI si-BroadcastStatus indication. If the PWS SIBs are not being broadcast, the UE would just store the scheduling information. When the ETWS/CMAS notification is sent via paging, UE can then directly use the stored scheduling information for the acquisition of the PWS SIBs.

Proposal 1: NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not similar to NR on-demand SI si-BroadcastStatus indication.

## 2.2 PWS reception for RRC connected UE

During the last meeting, RAN2 discussed PWS reception in RRC CONNECTED. Due to the concern that this may require additional UE capability for monitoring a common search space in addition to the UE specific search space, an LS was sent to RAN1 to check this. In our view, it would be very challenging to require NB-IoT UEs to support this. It is worth noting that even for eMTC, only UEs in CE support this (i.e., BL UEs don’t support it). The discussion during the last meeting was based on the solution where an NB-IoT UE receives direct indication information using SI-RNTI. However, this is not the only possible approach to enable PWS reception for UEs currently in RRC CONNECTED. RAN2 can explore alternative solutions that do not have RAN1 impact:

- Option 1: The network releases the UE to RRC IDLE and indicates/instructs the UE to read the PWS SIB(s) immediately after transitioning to RRC IDLE.

- Option 2: The network configures the UE with the gaps to read the PWS SIB(s).

- Option 3: The network delivers the PWS SIB(s) via dedicated RRC message.

For Option3, we understand there will be a signaling overhead issue due to the large size of the PWS SIB message. So we propose RAN2 to further discuss Option 1 and Option 2.

Proposal 2: Discuss the following candidate RAN2 solutions for PWS reception for UEs currently in RRC CONNECTED:

- Option 1: The network releases the UE to RRC IDLE and instructs the UE to read the PWS SIB(s) immediately after transitioning to RRC IDLE,

- Option 2: The network re-configures the UE with the gaps to read the PWS SIB(s).

# Conclusion

This contribution discussed the support of PWS for NB-IoT. Proposals are summarized as follows:

Proposal 1: NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not similar to NR on-demand SI si-BroadcastStatus indication.

Proposal 2: Discuss the following candidate RAN2 solutions for PWS reception for UEs currently in RRC CONNECTED:

- Option 1: The network releases the UE to RRC IDLE and instructs the UE to read the PWS SIB(s) immediately after transitioning to RRC IDLE,

- Option 2: The network re-configures the UE with the gaps to read the PWS SIB(s).

# References

RP-243278, Revised WID: Non-Terrestrial Networks (NTN) for IoT Phase 3.

3GPP TS 22.268, Public Warning System (PWS) requirements