3GPP TSG-RAN WG2 Meeting #129	R2-2500476

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.3

Source: 	Fujitsu

Title: 	Discussions on event triggered L1 measurement reporting

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution will provide more considerations on the event triggered L1 measurement reporting, including:

- Event evaluation and triggering

- MAC CE for measurement report

2. Discussion

Beam(s) for event evaluation

According to RAN2 agreement [2], RS associated with indicated beam for the serving cell is used for event evaluation. However, if the indicated beam is associated with PCI of non-serving cell, LTM cell switch based on event-triggered L1 measurement report cannot be supported. In case of inter-cell beam management, the UE is using the beam from the neighbouring cell associated to the serving cell and thus the RS from the neighbouring cell is used for event evaluation. Event e.g. LTM2/3/5 may not be satisfied since the quality of beam of serving cell used for event evaluation, i.e. the RS from the neighbouring cell, is good.

To support LTM cell switch after ICBM, a set of RS(s) can be configured for the serving cell for L1 measurement. Alternatively, RS associated with activated TCI states of the serving cell can be used for event evaluation.

Proposal 1: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 1bis: For L1 event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

The trigger of L1 measurement reporting

RAN2 has agreed that the L1 measurement report can be triggered when the leaving condition is met, if configured by the network. In event-triggered L3 measurement reporting, a cell list is maintained to evaluate the leaving condition. RAN2 needs to discuss which beam will be used to evaluate the leaving condition of LTM events. For example, both beam 1 and beam 2 are used for the LTM event evaluation while only beam 1 of candidate LTM cell satisfies the LTM event and MR MAC CE including the information of the beam is transmitted to the network. The issue is whether only beam 1 is used to evaluate the leaving condition of the LTM event, or any beam, e.g. both beam 1 and beam 2 can be used.

Proposal 2: RAN2 to discuss which beam is used for the leaving condition evaluation, triggered beam, any beam satisfying the event, or any beam.

MAC CE for measurement report (MR MAC CE)

Regarding the MR MAC CE, triggered event information, e.g. ReportConfigID will be included. RAN2 has also agreed that beam quantity of up to N beams will be included in the MAC CE. The network controls if the beam(s) not satisfying the event could be reported according to N beams in the MR MAC CE. In addition, the truncated measurement report MAC CE is supported. We need to discuss the information included in the truncated MR MAC CE.

Proposal 3: The following information can be contained in the (truncated) MR MAC CE according to network configuration:

- For both MR MAC CEs: the beam satisfying the LTM event (beam information + beam quantity)

- For non-truncated MR MAC CE: the beam for the candidate cell to which the beam satisfying the event belongs, no matter if satisfying the LTM event (beam quantity)

- For non-truncated MR MAC CE: current beam (beam quantity)

It is possible that multiple beams satisfying the LTM events belong to different LTM candidate cell. The question is if the information including beam information and beam quantity of these beams can be included in one MR MAC CE or in different MR MAC CEs. It has impacts to the MAC CE design and RAN2 needs to discuss this issue.

Proposal 4: RAN2 to discuss if one MR MAC CE can contain multiple beams satisfying the LTM event from different LTM candidate cells.

Other

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered during T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expires.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

A similar mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 5: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

3. Conclusion

In this contribution, we have some discussions on potential measurement enhancements for LTM and made the following proposals:

Event evaluation and triggering

Proposal 1: RAN2 is asked to support LTM cell switch after ICBM.

Proposal 1bis: For L1 event evaluations, if LTM cell switch after ICBM is supported, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

Proposal 2: RAN2 to discuss which beam is used for the leaving condition evaluation, triggered beam, any beam satisfying the event, or any beam.

MAC CE for measurement report

Proposal 3: The following information can be contained in the (truncated) MR MAC CE according to network configuration:

- For both MR MAC CEs: the beam satisfying the LTM event (beam information + beam quantity)

- For non-truncated MR MAC CE: the beam for the candidate cell to which the beam satisfying the event belongs, no matter if satisfying the LTM event (beam quantity)

- For non-truncated MR MAC CE: current beam (beam quantity)

Proposal 4: RAN2 to discuss if one MR MAC CE can contain multiple beams satisfying the LTM event from different LTM candidate cells.

Other

Proposal 5: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

Reference

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#105, Sep. 9-12, 2024

- R2-2410911, “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128