3GPP TSG-RAN WG2 Meeting #129	R2-2500492

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.0.2.16

Source:	Nokia, Nokia Shanghai Bell

Title:	Analysis of current text on transmission of PDCP SN Gap report

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution continues the discussion on how PDCP SN Gap reporting currently is, and how it should be, specified.

# 2	Discussion

Below we analyse the specification text for the transmit operation of the SN gap report (§5.16.1 of 38.323). The specification text begins with the following.

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and

Here the highlighted word seems out of place, since there is no prior occurrence of “PDCP SDU(s)”.

Proposal 1: RAN2 deletes “the” from the current Gap-report trigger condition “the PDCP SDU(s) are discarded as specified in clause 5.3”, since there is no prior occurrence of “PDCP SDU(s)” in that section.

The specification then continues with the following.

- there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

In our understanding, the purpose of the last condition highlighted above is to prevent a race condition where both a PDCP SDU and its discard indication may both be undergoing HARQ transmissions.

Observation 1: the current trigger condition “have not been submitted by RLC to lower layers” is to ensure that only a PDCP SDU or its discard indication is transmitted at MAC, but not both.

We would like to confirm as the agreed intention that this same race-condition prevention should apply not only to the gap-report triggering but also to its content.

Proposal 2: RAN2 agrees the intention that, like the current trigger condition, also the PDCP SN gap-report content setting ensures that only a PDCP SDU or its discard indication is transmitted at MAC, but not both.

The specification then continues with the following.

If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

Here, given the references to “the … discarded PDCP SDU” defined earlier:

-	Setting the FDC field only account for those earlier-defined SDUs that “have not been submitted by RLC to lower layers”; and

-	Also allocating the bitmap size only accounts for those earlier-defined SDUs that “have not been submitted by RLC to lower layers”,

-	Thereby making the current text for both the FDC field and the bitmap size compliant to Proposal 2.

Observation 2: by the current specification text on compiling PDCP SN gap report, given the references to “the … discarded PDCP SDU”, setting the FDC field and allocating the bitmap size only account for the earlier-mentioned SDUs that “have not been submitted by RLC to lower layers”.

Given the previous discussion on the topic, we would like to confirm this as the agreed meaning of the current text.

Proposal 3: agree that in the current specification text on compiling PDCP SN gap report, references to “the … discarded PDCP SDU” are to “the discarded PDCP SDU(s)…” in the preceding trigger condition that “…have not been submitted by RLC to lower layers.”

Finally, the specification ends with the following.

- setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

In contrast, here the discard indications in the bitmap account for all discarded SDUs. In our view, this deviates – for no justifiable reason – from Observations 1 and 2: why should there be different criteria for:

-	the SDU(s) triggering the report and the SDU(s) actually reported;

-	the SDU(s) defining the bitmap size and the SDU(s) defining the bitmap content;

-	the discard indication in the FDC field and those in the bitmap?

Observation 3: the discard indications in the bitmap account for all discarded SDUs, which is an unjustified deviation from Observations 1 and 2.

We think the above green-highlighted specification text should be aligned with the preceding text, to make it equally compliant with Proposal 2. Also, the above purple-highlighted specification text also needs to be corrected so as to not exclude PDCP SDUs that have been discarded but have also been already submitted to lower layers.

Proposal 4: RAN2 align the specification text on discard indications in the bitmap, to also account only for the earlier-mentioned SDUs that “have not been submitted by RLC to lower layers”.

# 3	Conclusion

This document has made the following observations:

Observation 1: the current trigger condition “have not been submitted by RLC to lower layers” is to ensure that only a PDCP SDU or its discard indication is transmitted at MAC, but not both.

Observation 2: by the current specification text on compiling PDCP SN gap report, given the references to “the … discarded PDCP SDU”, setting the FDC field and allocating the bitmap size only account for the earlier-mentioned SDUs that “have not been submitted by RLC to lower layers”.

Observation 3: the discard indications in the bitmap account for all discarded SDUs, which is an unjustified deviation from Observations 1 and 2.

And proposed the following:

Proposal 1: RAN2 deletes “the” from the current Gap-report trigger condition “the PDCP SDU(s) are discarded as specified in clause 5.3”, since there is no prior occurrence of “PDCP SDU(s)” in that section.

Proposal 2: RAN2 agrees the intention that, like the current trigger condition, also the PDCP SN gap-report content setting ensures that only a PDCP SDU or its discard indication is transmitted at MAC, but not both.

Proposal 3: agree that in the current specification text on compiling PDCP SN gap report, references to “the … discarded PDCP SDU” are to “the discarded PDCP SDU(s)…” in the preceding trigger condition that “…have not been submitted by RLC to lower layers.”

Proposal 4: to fix Observation 3 and to align with Observation 2, RAN2 align the specification text on discard indications in the bitmap, to also account only for the earlier-mentioned SDUs that “have not been submitted by RLC to lower layers”.

The corresponding CR is provided in R2-2501011.