3GPP TSG-RAN WG2 Meeting #129	R2-2500565

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on L1 event triggered measurement reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. Regarding the event-triggered L1 measurement reporting for LTM, the following agreements were reached.

Agreements on L1 event triggered MR

1.	TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

2.	TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

3.	TTT is applied to the leaving condition.

4.	Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

5.	Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

6.	SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

7.	For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

8.	For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

9.	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

10.	Support the truncated measurement report MAC CE.

In this contribution, we would like to share our opinions on detailed design for MR MAC CE.

- Discussion

Indication of Triggered event information

In RAN2#127bis, it was agreed to include triggered event information in MR MAC CE. Further details on indication of triggered event information may need discussion. During the post meeting discussion, the majority view is to use ltm-CSI-ReportConfigId to indicate the triggered event information. For the format of MR MAC CE, it seems to support to report only one ltm-CSI-ReportConfigId in one MR MAC CE by default, which may need further discussion in RAN2. In our opinion, it is possible to have multiple available L1 measurement results with different ltm-CSI-ReportConfigId at the time to transmit the MR MAC CE. To allow reporting multiple L1 measurement results with different ltm-CSI-ReportConfigId in one MR MAC CE will help reduce the overhead and provide more flexibility for L1 event triggered measurement reporting. Thus, we suggest RAN2 consider to support including multiple ReportConfigIds when designing the MR MAC CE format.

Proposal 1: RAN2 considers to support including multiple ReportConfigIds when designing the MR MAC CE format.

In previous RAN2 meeting, it was agreed that MR MAC CE can include up to N beams. If Proposal 1 is accepted, it may need further clarify whether N beams limitation applies to one ReportConfigId or the whole MR MAC CE. Since N is configured by the NW, there seems no need to have a separate limitation on each of the reported beam number of multiple ReportConfigIds. Thus, we suggest RAN2 confirm that one MR MAC CE can totally include up to N beams in the case that multiple ReportConfigId are reported.

Proposal 2: RAN2 confirms that one MR MAC CE can totally include up to N beams if the case that multiple ReportConfigIds reported in one MR MAC CE is supported.

Indication of RS type

In last RAN2 meeting, the following agreements were reached on the configuration and reporting of RS type.

Agreements in RAN2#128

1. Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

2. Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

3. A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

Based on the above agreement, the NW needs to know the reported MR MAC CE is associated to SSB or CSI-RS reference signals. To indicate the RS type for MR MAC CE, the following options could be considered.

- Option 1: Use different LCIDs or eLCIDs to indicate different RS types

- Option 2: Define a field in MR MAC CE to indicate different RS types

Both of the above options could work. Considering the truncated MR MAC CE is also supported, if we go for Option 1, at least 4 LCIDs or eLCIDs will be needed. To save the reserved LCID or eLCID resources, we slightly prefer Option 2.

Proposal 3: The field for indicating RS type shall be included in MR MAC CE.

Moreover, if Proposal 1 is agreed, one issue may need further discussion is whether L1 measurement results associated to different RS types could be reported in one MR MAC CE. In our opinion, allowing to report L1 measurement results associated to different RS types in one MR MAC CE is beneficial, which can reduce the overhead on reporting multiple MR MAC CEs. In that case, multiple RS type fields might be needed and each RS type field is related to one ReportConfigId.

Proposal 4: Multiple fields for indicating RS type could be included in MR MAC CE if the case that multiple ReportConfigIds reported in one MR MAC CE is supported. Each RS type field is related to one ReportConfigId.

Whether to report differential RSRP values in MR MAC CE

In Rel-18 LTM, CSI report procedure is used for reporting L1 measurements for LTM. SSBRI is used to indicate the beam information associated to L1-RSRP that reported within a single L1 measurement report instance. If more than one beam is configured to be included in L1 measurement report, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in TS 38.133.

For L1 event triggered measurement reporting, whether to support reporting differential RSRP values in MR MAC CE was discussed briefly in last RAN2 meeting. It is obvious that reporting differential RSRP values can reduce the overhead for reporting MR MAC CE. Since it is already supported in R18 L1 measurement reporting for LTM, no further impact is foreseen on RAN1 or RAN4. Thus, we support to report differential RSRP values in MR MAC CE. It is better to consider reporting differential RSRP values in MR MAC CE at the beginning of MR MAC CE format design.

Proposal 5: Support to report differential RSRP values in MR MAC CE.

- Conclusion

In this contribution, we discuss L1 event triggered measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 considers to support including multiple ReportConfigIds when designing the MR MAC CE format.

Proposal 2: RAN2 confirms that one MR MAC CE can totally include up to N beams if the case that multiple ReportConfigIds reported in one MR MAC CE is supported.

Proposal 3: The field for indicating RS type shall be included in MR MAC CE.

Proposal 4: Multiple fields for indicating RS type could be included in MR MAC CE if the case that multiple ReportConfigIds reported in one MR MAC CE is supported. Each RS type field is related to one ReportConfigId.

Proposal 5: Support to report differential RSRP values in MR MAC CE.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-24xxxx	Draft report of 3GPP TSG RAN WG2 meeting #127