3GPP TSG-RAN WG2 Meeting #129	R2-2500574

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.3

Source:	China Telecom

Title:	Discussion on NW side data collection for AIML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#127bis and RAN2#128 meeting [1], NW-side data collection was discussed, and the following agreements were made with several FFSs.

Agreements (RAN2#127)

1. 	As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

2. 	UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

3. 	Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

4. 	FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

5. 	FFS on event based data collection/logging

6. 	On-demand request from the network is supported.   FFS details on signalling

Agreements (RAN2#127b)

1. 	Periodic logging is supported for training data collection procedure in R19

2. 	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3. 	Periodic reporting of logged data is not supported.

4. 	On-demand reporting of the logged measurements will be specified

5. 	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.  FFS of details of the message

6. 	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how availability indication is sent to the network.

7. 	Low priority SRB will be used.    FFS new SRB or use of SRB4

8. 	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Agreements (RAN2#128)

1. 	Focus on the following three radio condition event based logging

-  L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

-  Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

-  L1 beam level measurement

2. 	Measurements on aperiodic CSI resources are not reported for NW sided data collection.

3. 	Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

4. 	The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

5. 	The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

6. 	The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

This contribution will discuss the remaining issues related to data availability for NW side data collection.

# 2 Discussion

## 2.1 FFS#1: Trigger/definition of availability indication

In RAN2#127bis meetings, RAN2 agreed that the UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. But the trigger/definition of availability indication was left as FFS.

6. 	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how availability indication is sent to the network.

According to the previous discussion, we understand that at least following definition of availability indication is required:

- The availability indication refers to UE logging the available measurement result for NW retrieval.

Proposal 1: Support at least following definition of availability indication

- The availability indication refers to UE logging the available measurement result for NW retrieval.

As for when UE triggers transmission of the availability indication, we think there should be a threshold for logged data size to trigger the transmission of the availability indication, and it should be configurable by NW depending on its needs.

Proposal 2. The availability indication can be triggered when the size of logged data exceeds a threshold which is configured by NW.

In RAN2#127 meeting, RAN2 agreed that measurements for data collection purposes and logging based can be controlled based on power state of the UE. In RAN2#128 meeting, RAN2 agreed that the UE reports to the network when the power state is low. In addition, in RAN2#127 meeting, RAN2 also agreed that when UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. Based on these agreements, we think that the availability indication refers to UE status, including that power state is low and AS buffer is full. Therefore, the power state and AS buffer can be used to trigger availability indication. That is, when the power state is low or when the AS buffer is full, the UE can stop measurement and send the availability indication to the network.

Proposal 3: The availability indication also refers to UE status, including that power state is low and AS buffer is full.

Proposal 4: The availability indication can be also triggered when the power state is low or when the AS buffer is full.

## 2.2 FFS#2: How availability indication is sent to the network

In RAN2#127bis meetings, RAN2 agreed that the UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. But how availability indication is sent to the network was left as FFS.

6. 	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how availability indication is sent to the network.

In legacy logged MDT, the UE indicates the availability of logged measurements within RRC complete messages (i.e., RRCReconfigurationComplete / RRCSetupComplete / RRCResumeComplete / RRCReestablishmentComplete). For instance, considering a handover case, if the network's data collection configuration is kept in the UE, the UE must inform the new gNB of its availability. This notification allows the new gNB to request retrieval of the UE’s logged data as needed. Moreover, as the availability indication information is UE side assistance information which may be helpful to network, so we assume UAI can be also used to deliver this information to network.

Proposal 5: Support both the RRC complete messages and UAI message for reporting the availability indication.

## 2.2 FFS#3: When the UE reports to the network when buffer is or may become full

In the RAN2#128 meeting, RAN2 agreed that the UE reports to the network when buffer is or may become full. But when it reports (before and/or after) is still FFS.

5. 	The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

According to the previous discussion, we understand that the UE should follow the measurement configuration from network. When the buffer is or may become full, the UE should at least report, and the network should de-configure. Therefore, maybe the UE should tell the network a bit earlier before the buffer is full.

Proposal 6: The UE reports to the network before the buffer is full.

# 3 Conclusion

In this contribution, we further discuss remaining issues related to data availability for NW-side data collection. We have following proposals:

Proposal 1: Support at least following definition of availability indication

- The availability indication refers to UE logging the available measurement result for NW retrieval.

Proposal 2. The availability indication can be triggered when the size of logged data exceeds a threshold which is configured by NW.

Proposal 3: The availability indication also refers to UE status, including that power state is low and AS buffer is full.

Proposal 4: The availability indication can be also triggered when the power state is low or when the AS buffer is full.

Proposal 5: Support both the RRC complete messages and UAI message for reporting the availability indication.

Proposal 6: The UE reports to the network before the buffer is full.

# 4 References

[1] RAN2#127b, Chair Notes.

[2] RAN2#128, Chair Notes.