3GPP TSG-RAN WG2 Meeting #129	R2-2500631

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.6.3

Source:		Lenovo

Title:		Event based L1 measurement report

Document for:		Discussion and Decision

- Introduction

The following has been agreed in the RAN2#128 meeting [1]. This contribution aims to discuss the potential issues for the event-based measurement report.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

- Discussion

The following LTM events based on beam specific quality of serving cell and candidate cells.

- Event LTM2: Beam of serving cell becomes worse than an absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. TTT is applied to both entering condition and leaving condition. RAN2 does not discuss whether the TTT for both entering condition and leaving condition is same one. In legacy L3 event, a single TTT is configured for both entering condition and leaving condition. Therefore, we propose:

Proposal 1: A single TTT is applied to both entering condition and leaving condition for a L1 event.

Once the condition (entering condition or leaving condition) during TTT is satisfied, measurement report MAC CE is triggered. For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. Furthermore, network controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. RAN2 does not discuss whether all beams (except for current beam) are from a single candidate cell or multiple candidate cells. The measurement report MAC CE is triggered after the first event is satisfied during TTT. It is possible that the second event is satisfied before this MR MAC CE is transmitted i.e., MAC PDU assembly. One option is that separate MR MAC CEs corresponding to the different satisfied event are reported. To reduce the signaling overhead, it is better that one MR MAC CE is used to accommodate all the reported information for multiple satisfied events. Then, a MAC PDU shall contain at most one MR MAC CE, even when multiple events have triggered a MR MAC CE. In this way, a single MR MAC CE can contain the beams from multiple candidate cells. Specifically, the first beam from the first candidate cell triggers the first event. The second beam from the second candidate cell triggers the second event. Both the first beam and second beam can be included in a single MR MAC CE.

Proposal 2: A measurement report MAC CE is allowed to include the beams from multiple candidate cells which trigger the corresponding events.

Proposal 3: A MAC PDU shall contain at most one MR MAC CE, even when multiple events have triggered a MR MAC CE.

It was agreed that a single measurement report MAC CE is triggered if entering condition or leaving condition during TTT is satisfied. As usual, cell switching is not triggered towards a candidate cell if the leaving condition corresponding to this candidate cell is triggered. Therefore, it is helpful for serving cell to differentiate whether it is triggered by entering condition or leaving condition.

Proposal 4: If a measurement report MAC CE is triggered due to the leaving condition, an indication in the MAC CE indicates the LTM candidate cell satisfies the leaving condition.

Conclusion

In this contribution, the following proposals are given based on the discussion:

Proposal 1: A single TTT is applied to both entering condition and leaving condition for a L1 event.

Proposal 2: A measurement report MAC CE is allowed to include the beams from multiple candidate cells which trigger the corresponding events.

Proposal 3: A MAC PDU shall contain at most one MR MAC CE, even when multiple events have triggered a MR MAC CE.

Proposal 4: If a measurement report MAC CE is triggered due to the leaving condition, an indication in the MAC CE indicates the LTM candidate cell satisfies the leaving condition.

References

[1] RAN2#128 chair notes for Rel-19 mobility