3GPP TSG-RAN WG2 Meeting #129	R2-2500647

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.3

Source:	HONOR

Title: 	Discussion on on-demand SIB1

Document for:	Discussion and decision

1	Introduction

In RAN2#128, the following aspects were agreed to be specified in the on-demand SIB1 work item [1]:

Agreements for OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In this contribution, we will analyze remaining issues of OD-SIB1 reception, including how to bar the NES cell, triggering conditions of UL-WUS, broadcasting schemes of SIBx, OD-OSI in NES cell and handling of UE in RRC_CONNECTED.

- Discussion

2.1 Legacy UE bars the OD-SIB1 Cell

RAN1 has agreed to take K_SSB > 23 for FR1 and K_SSB > 11 for FR2 as work assumption [2].

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster. - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

From RAN2 side, it is suggested that indication in MIB, e.g. via ssb-SubcarrierOffset could be adopted to bar the NES cell by legacy UE [3].

Agreement Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

According to [4], legacy UE will still take K_SSB > 23 for FR1 and K_SSB > 11 for FR2 as NCD-SSB indicator and try to find SIB1 instead of barring the NES cell directly, which will lead to unnecessary SIB1 acquisition procedure.

Observation 1: If SSB transmitted in the NES cell with K_SSB > 23 for FR1 and K_SSB >11 for FR2, legacy UE will try to acquire SIB1 instead of barring the NES cell directly.

Considering barring NES cell, it is straightforward to introduce a dedicated NES cell indicator via PBCH to distinguish whether the current cell is NES cell or not. Based on RAN1 assumption, we should consider using the unused K_SSB to bar the NES cell, e.g. K_SSB = 30 for FR1 and K_SSB = 14 for FR2.

On the one hand, for legacy UEs, this indicator can be used to directly bar the cell which is aligned with the conclusion of the RAN2#127bis meeting.

On the other hand, for NES UEs, this indicator can instruct them to obtain the UL-WUS configuration at first to avoid invalid SIB1 acquisition if they select the NES cell before obtaining the UL-WUS configuration.

Proposal 1: RAN2 to discuss and consider to introduce the NES cell indicator to indicate the NES cell and bar legacy UEs, e.g. using K_SSB = 30 for FR1 and K_SSB = 14 for FR2.

Proposal 2: RAN2 to send a LS to RAN1 on how to bar the legacy UE via K_SSB in NES cell.

2.2 Triggering conditions of UL-WUS

RAN2#128 has discussed SIB1 acquisition issue and it is agreed that the K_SSB value of NES cell could be K_SSB < 24 for FR1 and K_SSB < 12 for FR2 [1].

Agreement NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

It can be seen that it is contradictory with the work assumption in RAN1. And it will lead to unnecessary evaluation and camping for legacy UEs when SSB is set in the scope of K_SSB < 24 for FR1 and K_SSB < 12 for FR2.

Proposal 3: RAN2 to confirm whether the K_SSB transmitted in the NES cell can be configured as K_SSB < 24 for FR1 and K_SSB < 12 for FR2.

According to agreements of RAN2#127bis meeting, RAN2 has agreed that a NES UE needs to check if the SIB1 is currently being broadcasted or provided on demand for that NES cell before requesting the SIB1 of that cell [3].

Agreement If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

If the K_SSB value could be changed with SIB1 broadcasting status, the UE could decide the SIB1 broadcasting status via K_SSB value in PBCH. As for the NES cell with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, the NES UE will acquire the broadcast SIB1 in legacy. That is, the UE will receive SIB1 directly without triggering the UL-WUS for SIB1.

Proposal 4.1: If the K_SSB value could be K_SSB < 24 for FR1 and K_SSB < 12 for FR2 in NES cell, the UE should decide SIB1 broadcasting status according to the K_SSB value, i.e. the UE will not trigger UL-WUS to request OD-SIB1 when K_SSB < 24 for FR1 and K_SSB < 12 for FR2, else the UE should trigger UL-WUS to request SIB1.

Based on RAN1 conclusion, if the K_SSB value will be always set to K_SSB > 23 for FR1 and K_SSB > 11 for FR2 or specific values, the UE could not know the SIB1 broadcasting status via K_SSB directly. Following options should be considered to indicate the broadcast status of SIB1:

Option 1: Detecting SIB1 broadcasting status based on the scheduling information contained in UL-WUS configuration.

RAN1 has agreed to include searchSpaceZero and controlResourceSetZero for on-demand SIB1 in UL-WUS configuration and this makes it possible to detect SIB1 to check whether SIB1 is broadcasting or not [2].

Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

However, this may introduce camping delay with long detecting period, for example, to detect at least the period of scheduling window. This will impact the user experience if there are data packets waiting for transmission or reception.

Option 2: Detecting SIB1 broadcasting status via indicator in PBCH.

The SIB1 broadcasting status could be indicated in both Cell A and NES cell. However, it may lead to frequent SIBx modification and status exchange between NES cell and Cell A if SIB1 broadcasting status is provided in Cell A. Therefore, the SIB1 broadcasting status should be indicated in NES cell. Since only SSB is transmitted in NES cell, it is natural to determine that the NES cell does not broadcast SIB1 based on indicator in PBCH, e.g K_SSB or content of MIB.

In MIB, there still exist some bits that may not be used in NES cell, e.g. spare bits, pdcch-ConfigSIB1, cellBarred, which could be re-defined to indicate whether SIB1 is broadcasting or not.

Proposal 4.2: If the K_SSB value will be always set to K_SSB > 23 for FR1 and K_SSB > 11 for FR2 in NES cell, RAN2 to discuss how to indicate the SIB1 broadcasting status in the NES cell, e.g. via certain K_SSB or bits not used in NES cell’s MIB (including spare bits, pdcch-ConfigSIB1, cellBarred).

2.3 Broadcasting of UL-WUS Configuration (SIBx)

It has been agreed that NES cell’s WUS configuration will be included in a new SIB(SIBx) which is different with MSI. For energy saving, the SIBx should be able to be provided on-demand and the UE could acquire the SIBx via performing SI request.

Proposal 5: SIBx, containing UL-WUS configuration, could be provided on-demand.

In legacy spec, there are cell specific SIB and area specific SIB. The area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells identified by systemInformationAreaID. If the UE stores the valid SIB, it is not needed to re-acquire the SIB while moving into other cells with the same systemInformationAreaID.

It is beneficial for UE’s power consumption and signalling reduction in the air interface if the SIBx could be area specific as legacy OSI.

Proposal 6: SIBx, containing UL-WUS configuration, should be area specific with reusing the legacy areaScope/systemInformationAreaID.

2.4 OD-OSI in NES Cell

The UE should acquire the SIB1 for si-RequestResources/si-BroadcastStatus before sending the request for on demand system information. If SIB1 is provided on-demand, the UE needs to trigger a RA for SIB1 and then the other RA requesting for OSI. This may lead to frequent SIB1 requests with lots of network energy consumption. It is beneficial if the UE could trigger the OSI request without re-acquire SIB1 to check the broadcasting status.

Proposal 7: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 if SIB1 is not broadcasting.

si-RequestResources includes the preamble used for specific SIB provided on demand and the preamble may change with the change of the other SIB’s broadcasting status. To avoid incorrect mapping between si-RequestResources and SI message, network should configure the si-RequestResources for specific SIB/SI message instead of explicitly mapping based on broadcasting status to avoid incorrect MSG1-based request. Or the network should not enable the MSG1-based OSI in NES cell which is up to network implementation.

Proposal 8: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

2.5 Handling of UE in RRC_Connected

In NES cell, legacy UEs in RRC_CONNECTED state could initiate Random Access for some reasons, such as obtaining other system messages. These UEs will receive the MAC PDU including one or more MAC subPDUs constructed according to the data format depicted below [5], which may include RAR for OD-SIB1.

TS 38.321 MAC PDU (Random Access Response) A MAC PDU consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists one of the following: - a MAC subheader with Backoff Indicator only; - a MAC subheader with RAPID only (i.e. acknowledgment for SI request); - a MAC subheader with RAPID and MAC RAR. A MAC subheader with Backoff Indicator consists of five header fields E/T/R/R/BI as described in Figure 6.1.5-1. A MAC subPDU with Backoff Indicator only is placed at the beginning of the MAC PDU, if included. 'MAC subPDU(s) with RAPID only' and 'MAC subPDU(s) with RAPID and MAC RAR' can be placed anywhere between MAC subPDU with Backoff Indicator only (if any) and padding (if any). A MAC subheader with RAPID consists of three header fields E/T/RAPID as described in Figure 6.1.5-2. Padding is placed at the end of the MAC PDU if present. Presence and length of padding is implicit based on TB size, size of MAC subPDU(s).

Legacy UEs in RRC_CONNECTED state may decode the MAC PDU consisting of OD-SIB1 RARincorrectly if the size of the OD-SIB1 RAR is different with the size of existing RAR, i.e. 1 byte or 8 bytes. Since legacy UE will skip 1 byte or 8 bytes to find the next MAC subPDU.

The content of RAR for OD-SIB1 request and its corresponding MAC subPDU format has not been determined. One option is to restrict the size of OD-SIB1 RAR to 1 byte, or 8 bytesRAN1 is discussing the contents of RAR in response to UL-WUS using the legacy RAR for OSI as a starting point [6], e.g 1byte if the RAPID is the one for OD-SIB1. If there is other information should be included in the RAR, 8bytes is necessary to avoid incorrect decoding which is up to RAN1. This may lead to unnecessary padding.

Agreement At least the contents of RAR in response to SI request are included in RAR in response to UL WUS.

Observation 2: The length of OD-SIB1 RAR could be 1byte or 8bytes to solve the legacy UE’s incorrect decoding issue. And unnecessary padding will be introduced if 8 bytes is required which is up to the content of RAR discussed in RAN1.

Without restricting the length of OD-SIB1 RAR, the RAR could be  placed at the end of the MAC PDU and before padding (if present). In this case, the UE in RRC_CONNECTED state can determine that it is the last subPDU through “E” in the subheader as the legacy procedure, thus avoiding finding the next MAC subPDU with skip fixed bits. Meanwhile, the size of OD-SIB1 could be any bytes to accommodate necessary information for SIB1 acquisition.

Proposal 9: MAC subPDU for OD-SIB1 RAR should be placed at the end of the MAC PDU and before padding (if any).

3	Conclusions

In this contribution, we discussed several detailed on-demand SIB1 issues, and made the following observations and proposals:

Observations

Observation 1: If SSB transmitted in the NES cell with K_SSB > 23 for FR1 and K_SSB >11 for FR2, legacy UE will try to acquire SIB1 instead of barring the NES cell directly.

Observation 2: The length of OD-SIB1 RAR could be 1byte or 8bytes to solve the legacy UE’s incorrect decoding issue. And unnecessary padding will be introduced if 8 bytes is required which is up to the content of RAR discussed in RAN1.

Proposals

Proposal 1: RAN2 to discuss and consider to introduce the NES cell indicator to indicate the NES cell and bar legacy UEs, e.g. using K_SSB = 30 for FR1 and K_SSB = 14 for FR2.

Proposal 2: RAN2 to send a LS to RAN1 on how to bar the legacy UE via K_SSB in NES cell.

Proposal 3: RAN2 to confirm whether the K_SSB transmitted in the NES cell can be configured as K_SSB < 24 for FR1 and K_SSB < 12 for FR2.

Proposal 4.1: If the K_SSB value could be K_SSB < 24 for FR1 and K_SSB < 12 for FR2 in NES cell, the UE should decide SIB1 broadcasting status according to the K_SSB value, i.e. the UE will not trigger UL-WUS to request OD-SIB1 when K_SSB < 24 for FR1 and K_SSB < 12 for FR2, else the UE should trigger UL-WUS to request SIB1.

Proposal 4.2: If the K_SSB value will be always set to K_SSB > 23 for FR1 and K_SSB > 11 for FR2 in NES cell, RAN2 to discuss how to indicate the SIB1 broadcasting status in the NES cell, e.g. via certain K_SSB or bits not used in NES cell’s MIB (including spare bits, pdcch-ConfigSIB1, cellBarred).

Proposal 5: SIBx, containing UL-WUS configuration, could be provided on-demand.

Proposal 6: SIBx, containing UL-WUS configuration, should be area specific with reusing the legacy areaScope/systemInformationAreaID.

Proposal 7: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 if SIB1 is not broadcasting.

Proposal 8: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

Proposal 9: MAC subPDU for OD-SIB1 RAR should be placed at the end of the MAC PDU and before padding (if any).

4	References

- Draft_RAN2_128_Meeting_Report_v1

- R1-2409341 Report of RAN1#118bis meeting ETSI MCC

- R2-2409502 RAN2#127bis Meeting Report MCC report

- TS 38.213 V18.4.0, Physical layer procedures for control, 2024-09

- TS 38.321 V18.3.0, Medium Access Control (MAC) protocol specification, 2024-09

- Draft_Minutes_report_RAN1#119_v030