3GPP TSG-RAN WG2 Meeting #129	R2-2500648

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.2

Source:	HONOR

Title: 	Further discussion on inter-CU LTM

Document for:	Discussion and Decision

1	Introduction

In Rel-19 Mobility enhancements WI, the following objective was achieved:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured. - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged. - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released. - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling. - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#128, the following agreements were made:

- Rel-19 Set ID is configured for a candidate configuration. For DRB: - Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, > UE performs PDCP re-establishment to all radio bearers. - Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, > For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer. > For SN terminated bearer without change of termination point, UE performs PDCP re-establishment. > If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment. - For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. - For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard). - RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. - RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19. - From RAN2 perspective, the following coexistence cases in NR-DC can be supported: - Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM - Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM - In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations. - RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. - RAN2 to support intra-CU MCG LTM with SCG configuration. - It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. - RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this contribution, we discuss the remaining issues in inter-CU LTM with non-DC and DC scenarios.

2. Discussion

During UL synchronization with LTM candidate cell(s), after performing TA acquisition based on CFRA, the source-DU obtains TA via the interaction between CU-CU and CU-DU nodes. In detail, the candidate DU notifies the associated CU of the obtained TA. The candidate CU forwards the TA to the source CU. Finally, the source DU obtains the TA from the source CU. Therefore, it is natural for source DU to manage TA validity. If not, while the TA is about to be used, the source DU has to obtain TA from source CU, which will have negative impacts on cell switch latency.

Upon cell switch, if the TA associated with candidate cell can continue to be maintained by NW side, the signalling overhead might be reduced, since it might avoid re-obtaining TA in the following period of time. To achieve this purpose, the source CU forwards TA information to the target CU, then, the target DU obtains the TA from the target CU.

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

In 37.340, the procedure of inter-Master Node handover with/without Secondary Node change is specified. Since it is similar to the scenario that inter-CU LTM of MN with SN released or kept, the procedure of inter-Master Node handover with/without Secondary Node change can be taken as the baseline.

In the previous meeting, it was agreed that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). That is, SCG configuration can be kept or released based on NW side decision. In addition, we also need to consider how to indicate UE on this decision. For the handling on SCG configuration, there are two options as follows:

Option 1: RRC pre-configuration

In the LTM preparation phase, the SCG handling indication can be configured in the process of generating candidate cell configuration. It is straightforward that the SCG handling indication is determined by candidate MN.

Observation 1: The candidate MN is responsible to generate SCG handling indication.

Option 2: cell switch MAC CE

From the perspective of the SCG indication transmission, a cell switch MAC CE can be used to indicate the operation on SCG. In order to achieve this purpose, it is necessary for the target MN to provide the SCG indication information to the source MN through additional signalling.

Proposal 2: It is suggested to provide the indication of SCG kept or released via RRC pre-configuration.

In subsequent inter-CU MCG LTM, if candidate MNs with SCG released and with SCG kept are configured simultaneously, the issue that SCG has been released prior to MCG LTM with SCG kept, should be taken into consideration. For instance, the UE switches to a target MN1 with SCG released, then the UE switches to a target MN2 with SCG kept.

Regarding the identified issue, there are probably two main solutions. One solution is to configure a consistent SCG handling between MNs, which will have small impacts on UE. However, this is based on the restriction on the NW configuration. Alternatively, it is allowed to configure candidate MNs with SCG released and with SCG kept. If SCG has been released and the target MN has configured with SCG kept indication, an SCG addition procedure needs to be triggered before the LTM cell switch command. After completing SCG addition procedure, the UE is able to apply SCG configuration and perform the MN LTM with SCG kept.

Observation 2:  From RAN2 point of view, for the case inter-CU MCG LTM with SCG addition, it has a minimal standard impact.

If SCG addition is adopted to solve this issue, another issue such as when and how to trigger SCG addition, should be taken into consideration. There are two options as follows:

Option1: SCG addition will always be executed before cell switch

For instance, before cell switch, if SCG is released, no matter SCG is configured with SCG released or SCG kept in the target MN, a SCG addition procedure will be triggered by serving MN.

Option2: SCG addition will be executed before cell switch, when it is indicated with SCG kept in the target MN

In this option, if the serving MN find the SCG is already released, in order to apply SCG configuration after cell switch, SCG addition should be executed before cell switch. However, this requires the serving MN to perceive the indication information of the target MN, which is currently not supported, since the indication is configured in a container and the serving MN doesn't even decode it. One feasible approach is that target MN needs to explicitly provide the SCG handling indication to serving MN.

Compared with the two options, option 1 might result in unnecessary SCG addition action, which will have a negative impact on signalling overhead over the air interface. Therefore, option 2 is preferred.

Proposal 3: Before cell switch, if SCG is released, and it is indicated with SCG kept in the target MN, a SCG addition procedure will be triggered.

In the mentioned SCG addition procedure, the serving MN is responsible for SCG addition. After completing SCG addition, the UE can apply SCG after switching to the target MN. However, if the difference between the current added SCG configuration and the SCG configuration in the target MN happens, the issue how to apply SCG configuration should be taken into further discussion.

Proposal 4: In SCG addition procedure, if the current added SCG configuration is different from that in the target MN, the issue how to apply SCG configuration should be taken into further discussion.

3 Conclusions

In this contribution, we discussed the issues related to inter-CU LTM. Based on the discussion, the following observations and proposals are concluded:

Observation 1: The candidate MN is responsible to generate SCG handling indication.

Observation 2: From RAN2 point of view, for the case inter-CU MCG LTM with SCG addition, it has a minimal standard impact.

Proposal 1: After cell switch, the TA associated with candidate cell(s) can continue to be kept in the following serving DU.

Proposal 2: It is suggested to provide the indication of SCG kept or released via RRC pre-configuration.

Proposal 3: Before cell switch, if SCG is released, and it is indicated with SCG kept in the target MN, a SCG addition procedure will be triggered.

Proposal 4: In SCG addition procedure, if the current added SCG configuration is different from that in the target MN, the issue how to apply SCG configuration should be taken into further discussion.

4 References

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4.

- TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- TS 38.401, Technical Specification Group Radio Access Network; NG-RAN; Architecture description

- S3-241898, Draft reply LS on security handling for inter-CU LTM in non-DC cases