3GPP TSG-RAN WG2 Meeting #129	R2-2500694

Athens, Greece, 17 - 21 Feb, 2025

Title: 	Discussion on Ku band numerology

Source: 	Huawei, HiSilicon

Agenda item:	8.19

Document Type:	Discussion and Decision

- Introduction

An LS on Ku band has been received from RAN4 [1]. RAN WG4 has determined that both FR1 and FR2 numerology are feasible for Ku band, and would like to ask RAN1 and RAN2 the feasibility of defining two sets of bands for the exact same frequency range, one for FR1 numerology and the other for FR2 numerology:

To RAN WG1 and RAN WG2 ACTION: RAN WG4 respectively asks RAN WG1 and RAN WG2 if they see any reason why two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology cannot be defined

The WI has a checkpoint at RAN#106 [2]:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106.

The issue has been discussed in RAN#106, and RAN has approved the revised WID as follows [3]:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

In this contribution, we provide our analysis from RAN2 perspective.

- Discussion

In RAN2 spec, there are multiple places where SCS is configured, for instance in MIB, in SIB1, in BWP configuration, in measObjectNR configuration etc. Even if two sets of bands are defined for Ku band, the cell can simply configures the numerology it supports (or the numerology of neighbour cells in case of inter-frequency measurements) as in the existing procedure, and no problem is foreseen.

On the other hand, in the existing spec, only “FR1” “FR2-1” “FR2-NTN” “FR2-2” are defined and used, for instance in BWP configuration:

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz For the initial DL BWP and operation in licensed spectrum this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Except for SUL, the network ensures the same subcarrier spacing is used in active DL BWP and active UL BWP within a serving cell. For the initial DL BWP and operation with shared spectrum channel access, the value of this field corresponds to the subcarrier spacing of the SSB associated to the initial DL BWP.

The reason for not mentioning “FR1-NTN” is that, the existing definition of “FR1-NTN” has the same frequency range with FR1 in terrestrial network, while “FR2-NTN” has a different frequency range with FR2 in terrestrial network, see below Table 5.1-1 in TS 38.101-5 and Table 5.1-1 in TS 38.101-1 respectively:

Table 5.1-1: Definition of NTN frequency ranges (in TS 38.101-5)

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Table 5.1-1: Definition of frequency ranges (in TS 38.101-1)

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

Considering the following frequency range for Ku band does not fall into FR1 definition, a new terminology “FR1-NTN” needs to be introduced to RAN2 spec [3]:

Table 1: Ku Band Downlink and Uplink Spectrum for ITU Regions 1 and 3 Ku band for GSO and Non GSO Space Segment; Downlink (Space to Earth); 10.70 - 12.75 GHz. Uplink (Earth to Space); 12.75-13.25 GHz & 13.75-14.5 GHz. Table 2: Ku Band Downlink and Uplink Spectrum for ITU Region 2 Ku band for GSO and Non GSO Space Segment; Downlink (Space to Earth); 10.70 - 12.70 GHz. Uplink (Earth to Space); 12.70-13.25 GHz & 13.75-14.5 GHz.

Observation 1: The existing RAN2 procedure can be reused even if two sets of Ku bands are introduced, but the terminology of “FR1-NTN” needs to be introduced to RAN2 spec and some field descriptions of SCS need to be updated accordingly.

Proposal 1: From RAN2 perspective, it is feasible to define two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology.

- Conclusion

In this contribution, we Ku band numerology and have the following proposal:

Observation 1: The existing RAN2 procedure can be reused even if two sets of Ku bands are introduced, but the terminology of “FR1-NTN” needs to be introduced to RAN2 spec and some field descriptions of SCS need to be updated accordingly.

Proposal 1: From RAN2 perspective, it is feasible to define two sets of Ku bands, one using FR1 numerology and the other using FR2 numerology.

- Reference

- R4-2419902, LS on Ku band numerology, RAN4 (contact company: Eutelsat)

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat