3GPP TSG RAN WG2 Meeting #129			R2-2500739

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	8.2.4

Source:		Sony

Title:			Considerations on segmentation

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new work item for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution, we discuss our view on segmentation due to limited amount of remaining energy.

# Discussions on energy status based segmentation

Segmentation can be needed in order to fit the data or packages into suitable Transport Blocks (TB) or units for transmission over the radio interface.

For Ambient IoT, e.g. transport block sizes up to 1000 bits has been discussed during study phase [2], but segmentation on data level is understood as too complex and only simplified segmentation may be supported in the A-IoT MAC layer.

From the TR it clarified that how large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.

And the following candidate solutions or restrictions have been listed:

-	The sequence number, the segment number and the number of segments are not supported;

-	An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details;

-	It is beneficial for the reader to be able to trigger a re-transmission of a segment;

-	It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

Below we provide our input on possible other ways forward for alternative segmentation solution.

Figure 1 illustrates a scenario where the devices do not have any limitation regarding their energy resource availability. The total transmission size is length N. A preamble is transmitted at the beginning of the data transmission for synchronization purposes and to indicate the beginning of the data transmission. The preamble may also include the device identity. Midambles may be inserted during the data transmission as pilots and also to maintain synchronization. An indication/postamble at the end of the transmission is needed to show the end of the data transmission, this is particularly important if the total data transmission size is not communicated earlier.

Figure 1 - Illustration of a data transmission with no limitation in energy resource availability

Proposal 1: Each data transmission of a given size consist of a pre-amble, mid-amble, and post-amble.

Such data transmission may not be possible if the energy required to transmit the entire data is larger than the device available energy resource.

Observation 1: For data or packet sizes large than a certain value, the stored energy may not be sufficient.

This may lead to a need for segmentation, were the segment gets a size corresponding to the stored energy at the A-IoT device.

Figure 2 and Figure 3 illustrate two alternatives enabling data segmentation in lower layer of the protocol.

Figure 2 - Example of data segmentation supporting harvesting (alternative I – indication in postamble).

Figure 3 – Example of data segmentation supporting harvesting (alternative II – indication in preamble).

As shown in the figures, each data block, in addition to its data part, may contain:

- a preamble, used to indicate the start of the data blocks and clock accuracy acquisition, and the device ID, prefixed to each data block,

  - the preamble in the first block includes an indication of the data segmentation and/or total number of blocks

  - the preamble in the other blocks may also indicate the size of each transmission block,

- a data block number indication, arranged in descending order. The block number indicator can be transmitted directly after the preamble (as in Figure 3) or it can be transmitted at the end of each block (as in Figure 2). Under this arrangement the block number zero represents the end of data transmission or the last data block.

- a gap may be inserted between each data block long enough allowing the device to have enough energy/to charge the energy needed to transmit the next data block. The minimum time gap can be signaled based on the capability of the device in terms of charging circuitry, energy harvesting source and its corresponding efficiency, energy storage component. A gap may contain the minimum time (i.e., for harvesting time) and a random back-off value to minimize the potential interference with other UE.

Proposal 2: Each data block may contain a preamble, a data block number indication, a gap (time), in addition to the data part.

# Summary

In this contribution, we have discussed our views on aspects related to, random access, procedures.

Proposal 1: Each data transmission of a given size consist of a pre-amble, mid-amble, and post-amble.

Observation 1: For data or packet sizes large than a certain value, the stored energy may not be sufficient.

Proposal 2: Each data block may contain a preamble, a data block number indication, a gap (time), in addition to the data part.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR.38.769