3GPP TSG-RAN WG2 Meeting #129	R2-2500912

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.4.1

Source:	Samsung

Title:	Outstanding issues on LCP enhancements for Rel-19 XR

Document for:	Discussion & Decision

# Introduction

In the RAN2#127-bis meeting (October 2024), RAN2 made the following agreements for LCP enhancements for Rel-19 XR:

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

And additionally, on the topic of terminology, in the same meeting the following was agreed:

For now, we will use “LCH priority-adjusted data”

In the subsequent and most recent RAN2#128 meeting (November 2024), RAN2 additionally agreed the following:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In this paper, we discuss the key remaining issues of the operations related with the additional LCH priority, and the impact of agreements made, especially of the optional support of fallback in 2nd round. We look at the use of the additional LCH priority in MAC procedures other than LCP, as well as handling of other priority traffic e.g. SRBs in light of the additional LCH priority for XR traffic.

# Discussion

## Additional LCH Priority in Other Procedures: general considerations

In legacy NR, LCH priority is used in 1) resource allocation in LCP (which is already being adapted to XR as the agreements above attest), 2) BSR triggering (in some cases, a BSR is triggered only if data of higher priority arrives than priority of data already in the buffer), 3) truncated BSR LCG selection (when we do not have space to send the entire BSR), and 4) intra-UE prioritization. Based on the existing RAN2 agreements, it is currently unclear whether each of these steps should trigger a re-calculation of LCH priorities (i.e. determining whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used), or whether different steps (e.g. MAC PDU assembly and related intra-UE prioritization at point of transmission of the MAC PDU) should use the same LCH priorities or not.

Observation 1: It is currently unclear whether each of the MAC procedures involving use of the priority of LCHs should trigger a re-calculation of LCH priorities (i.e. determining whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used), or whether different, potentially related steps (e.g. MAC PDU assembly and related intra-UE prioritization at point of transmission of the MAC PDU; BSR triggering and related MAC PDU assembly) should use the same LCH priorities.

Observation 2: Agreement made at RAN#128 (stating that the UE can optionally also support fall-back to default priority in the 2nd round of LCP) potentially point in the direction of a unified approach (= the approach where the UE re-checks whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used).

Based on the above, we propose:

Proposal 1. RAN2 to agree that procedures such as BSR triggering and intra-UE prioritization shall also use the additional LCP priority.

Proposal 2. RAN2 to agree one of the following options:

- Each of the MAC procedures involving use of the additional priority of LCHs should trigger a re-calculation of LCH priorities (i.e. the UE re-checks whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used)

- Different, potentially related steps (e.g. MAC PDU assembly and related intra-UE prioritization at point of transmission of the MAC PDU; BSR triggering and related MAC PDU assembly) should use the same LCH priorities

## The intra-UE prioritization case and the additional LCH Priority

Turning our attention now specifically to intra-UE prioritization, and as a reminder, according to 38.300, “in case a configured uplink grant transmissions or a dynamically allocated uplink transmission overlaps in time with a scheduling request transmission, the UE prioritizes the transmission based on the comparison between the priority of the logical channel which triggered the scheduling request and the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDU associated with the overlapping resource”. Therefore, in this legacy intra-UE prioritization, when more than one UL grant(s) and/or SR(s) overlap in time domain, a priority-based comparison among the overlapped UL grant(s) and/or SR(s) is conducted to determine which UL grant or SR should be transmitted. The priority-based comparison is based on legacy (default) LCH priority, i.e., the priority of UL grant is defined as the highest (legacy) priority of LCHs multiplexed in the corresponding MAC PDU, and the priority of SR is defined as the (legacy) priority of the LCH triggering the SR.

Based on 38.321, we also have the following:

“For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.”

The following should therefore be noted:

Observation 3. For the cases of autonomousTx and HARQ retransmission, priority of an uplink grant is determined following the MAC PDU assembly; in all other cases, it is determined prior to the actual MAC PDU assembly and based on data available that can be multiplexed (and not necessarily data that will actually end up in the MAC PDU which will be sent using this grant).

What is different in the case of intra-UE prioritization linked to logical channels that have LCH priority-adjusted data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources, is whether an update to the determined uplink grant priority (for purposes of intra-UE prioritization) is needed following the MAC PDU assembly, e.g. in cases where all of the LCH priority-adjusted data was inserted into the MAC PDU.

(Similar considerations apply for the case of intra-UE overlapping resources prioritization linked to scheduling request triggered by logical channels that have LCH priority-adjusted data to be transmitted, where RAN2 should discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority.)

Based on the above discussion, we propose the following:

Proposal 3. For the case of intra-UE prioritization linked to logical channels that have LCH priority-adjusted data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources, RAN2 to discuss whether an update to the determined uplink grant priority is needed following the MAC PDU assembly.

Proposal 4. For the case of intra-UE overlapping resources prioritization linked to scheduling request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

## Data prioritization over comparatively less urgent MAC CEs

We now turn our attention to the potential use-case of prioritization of data over MAC CEs, which for LCHs with priority-adjusted data, is potentially very relevant as the in-time delivery is important. So far, dedicated configured grant or resource separated by LCP have been introduced over multiple releases of NR to aid in-time delivery. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in LCH priority-adjusted data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs, and propose the following:

Proposal 5. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

As an illustration of some additional underlying issues, and using the BSR MAC CE as an example, a BSR is triggered and LCH#1 uses new/adjusted priority P_a as it contains SDUs close to expiry. By the time MAC PDU assembly starts, data arrives into LCH#2 which has priority P_b1 > P_a (and this was not the case when BSR was triggered, where LCH#2 had a priority P_b<P_a). If we now want to prioritize BSR MAC CE over data or not (in legacy systems we would prioritize the BSR MAC CE, but for XR data we may wish to prioritize data), it is unclear whether we should compare P_b and P_a, or P_b1 and P_a.

Observation 4: Re-calculating LCH priorities at time of MAC PDU assembly relative to priorities at the time of BSR triggering could cause issues in cases where we want to prioritize data over a BSR MAC CE.

Proposal 6. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

## Handling of SRBs

In the past meetings, certain submissions discussed the perceived impact of the additional LCH priority on timely transmission of signaling data. In our view, the handling of this case is best left to network implementation:

Proposal 7. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Observation 1: It is currently unclear whether each of the MAC procedures involving use of the priority of LCHs should trigger a re-calculation of LCH priorities (i.e. determining whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used), or whether different, potentially related steps (e.g. MAC PDU assembly and related intra-UE prioritization at point of transmission of the MAC PDU; BSR triggering and related MAC PDU assembly) should use the same LCH priorities.

Observation 2: Agreement made at RAN#128 (stating that the UE can optionally also support fall-back to default priority in the 2nd round of LCP) potentially point in the direction of a unified approach (= the approach where the UE re-checks whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used).

Proposal 1. RAN2 to agree that procedures such as BSR triggering and intra-UE prioritization shall also use the additional LCP priority.

Proposal 2. RAN2 to agree one of the following options:

- Each of the MAC procedures involving use of the additional priority of LCHs should trigger a re-calculation of LCH priorities (i.e. the UE re-checks whether each LCH contains LCH priority-adjusted data at every given point in time where LCH priority needs to be used)

- Different, potentially related steps (e.g. MAC PDU assembly and related intra-UE prioritization at point of transmission of the MAC PDU; BSR triggering and related MAC PDU assembly) should use the same LCH priorities

Observation 3. For the cases of autonomousTx and HARQ retransmission, priority of an uplink grant is determined following the MAC PDU assembly; in all other cases, it is determined prior to the actual MAC PDU assembly and based on data available that can be multiplexed (and not necessarily data that will actually end up in the MAC PDU which will be sent using this grant).

Proposal 3. For the case of intra-UE prioritization linked to logical channels that have LCH priority-adjusted data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources, RAN2 to discuss whether an update to the determined uplink grant priority is needed following the MAC PDU assembly.

Proposal 4. For the case of intra-UE overlapping resources prioritization linked to scheduling request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to discuss whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

Proposal 5. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

Observation 4: Re-calculating LCH priorities at time of MAC PDU assembly relative to priorities at the time of BSR triggering could cause issues in cases where we want to prioritize data over a BSR MAC CE.

Proposal 6. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

Proposal 7. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.