3GPP TSG-RAN WG2 Meeting #129	R2-2500966

Athens, Greece, 17 - 21 Feb, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on MBS broadcast over NTN

Agenda Item:	8.8.4

Document for:	Discussion and decision

# Introduction

One of the objectives for Rel-19 NTN WI [1] is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #128s meeting, the following agreements in [2] were made:

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this contribution, we provide our views on the remaining issues.

# Discussion

The last RAN2 meeting has introduced a new SIB to convey the representation format of intended geographical areas. The previous RAN2 meetings have narrowed down the methods to provide the intended service areas to the UEs and considered the following possibilities: SIB20, SIB21, or the MBSBroadcastConfiguration message. Each possibility has different implications, since SIB20, SIB21 and MBSBroadcastConfiguration take effect at different stages of the attempt to receive MBS service

- SIB20 is used to convey the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, the time resource for MCCH). If the serving cell sends SIB20, the UE will receive the MCCH of this cell.

- SIB21 provides information related to service continuity for MBS reception, i.e., the mapping of MBS FSAI and the current/ neighbour frequencies. If the MBS capable UE is receiving or interested in receiving an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider the MBS frequency to be the highest priority during the cell selection procedure.

- the MBSBroadcastConfiguration message (provided via MCCH) is used to convey the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbour cells which are also broadcasting this MBS session). The UE will apply the corresponding configuration of an MBS session, e.g., the scheduling information and the MRB configuration per MBS session. Besides, the UE also determines whether the MBS session of interest is provided in any neighbour cell or not.

If the UE is interested to receive an MBS BC service that is relevant only in a number of "intended service areas" (ISAs) but does not know which ones, when the UE enters a new cell, the UE can see whether the new SIB is broadcast and, based on the contents of this new SIB, determine whether it is currently in any of the ISAs. If the UE is not in any of the ISAs, there is no obvious use for the UE to read MCCH, so the UE may skip reading MCCH as long as it is not in close to any of the ISAs in the new SIB.

Proposal 1: If the UE knows that ISAs are used for the MBS BC service of interest but does not know which ISA(s) of the current cell apply to the service, the UE may not monitor MCCH as long as it is not in or close to any of the ISAs in the new SIB.

Some ISAs indicate in the new SIB may not apply to MBS services provided in the current cell (e.g., they only apply to services provided in inter-frequency neighbour cells). In this case, even though the UE would be in or close to such ISAs, there would be no use for the UE to monitor MCCH. To allow UEs in these areas not to monitor MCCH, SIB20 could provide the list of ISA IDs that are used for services in the current frequency.

Proposal 2: SIB20 can provide the list of intended service area ID(s) applicable to services that may be broadcast in the serving cell. The UE may not monitor MCCH as long as it is not in or close to any of the ISAs indicated in SIB20.

When the UE is in an ISA listed in the new SIB, the MBS BC service could be provided there so the UE should acquire MCCH. Then, the UE will know whether the service is broadcast in the cell and the applicable ISAs. Which ISAs are applicable should be fixed information that is not modified for a service, so if the UE is not in an applicable ISA, the UE may stop monitoring MCCH.

Proposal 3: If the UE knows the ISAs applicable for the MBS BC service of interest (by reading such information in MCCH previously), the UE may not monitor MCCH as long as it is not in or close to the ISAs applicable for the service.

The last RAN2 agreed to provide, for each MBS session, one or more intended service area IDs in the MCCH. The MCCH, i.e., MBSBroadcastConfiguration, provides the support status of MBS sessions for the serving cell and for neighbour cells i.e., in addition to the MBS session ID list of the serving cell, it also indicates the MBS session ID list provided by the neighbour cells. If MBSBroadcastConfiguration also provides the intended service area IDs for neighbour cells in addition to current serving cell, upon mobility to a neighbour cell, the UE that has previously obtained the ISA Ids for the service sees the definitions of ISAs from the new SIB and immediately knows whether it is currently in an ISA of the service, and if not, it may not read MCCH.

Observation 1: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid monitoring MCCH when moving to a neighbour cell but out of the ISAs of the service.

Based on neighbour cell information provided in the MBSBroadcast configuration, the UE can request unicast reception of the service of interest before moving to a cell not providing the MBS session. If the UE is in RRC_IDLE, this involves establishing an RRC connection. If the UE is in RRC_CONNECTED, the UE may request to switch to unicast mode for MBS session data reception as it comes closer to a target cell.

However, when moving to the new cell, if the UE is not in any of the intended service area, there is no use for the UE to start performing unicast reception of the service. If the MCCH also provides ISA IDs for neighbour cells and the ISA IDs are aligned between the serving and the neighbour cell, the UE can avoid switching to unicast reception unnecessarily.

Observation 2: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid unnecessarily switching to unicast reception before moving to a neighbour cell but out of the ISAs of the service.

Proposal 4:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 5: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

SIB21 is used to guarantee the service continuity of MBS session when the UE in RRC_IDLE initiates the cell reselection procedure or when the UE is in RRC_CONNECTED. The UE in RRC_IDLE can prioritize an MBS frequency on which the MBS session of interest is provided when the UE performs cell reselection and the UE in RRC_CONNECTED can indicate the frequency of the MBMS service of interest, so that the network keeps the UE on that frequency. If an MBS service is very popular, this may increase the network load in the frequency where it is provided, possibly requiring to restrict other services for the UE, so it is preferrable that the UEs only prioritize or indicate a frequency when they are in the ISA of the service.

The UE may already know the ISAs of the service from reading MCCH in neighbour cells, but it may not be the case. Therefore, it is useful for the cell to indicate ISAs not only for MBS services that may be provided in the current frequency, but also for MBS services that are provided in neighbour frequencies, so the new SIB could include both.

In addition, the set of services provided in one frequency may not use the same set of ISAs as services provided in another frequency. If SIB21 can indicate which ISAs are used in which frequencies, the UE that is currently not located in any ISA where the MBS service of interest is provided can avoid prioritizing or indicating the frequency of the MBS service of interest, in order to preserve other services.

Proposal 6: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

# Conclusion

In this document, we discussed the support of broadcast service via NR NTN, and proposed the following:

Proposal 1: If the UE knows that ISAs are used for the MBS BC service of interest but does not know which ISA(s) of the current cell apply to the service, the UE may not monitor MCCH as long as it is not in or close to any of the ISAs in the new SIB.

Proposal 2: SIB20 can provide the list of intended service area ID(s) applicable to services that may be broadcast in the serving cell. The UE may not monitor MCCH as long as it is not in or close to any of the ISAs indicated in SIB20.

Proposal 3: If the UE knows the ISAs applicable for the MBS BC service of interest (by reading such information in MCCH previously), the UE may not monitor MCCH as long as it is not in or close to the ISAs applicable for the service.

Proposal 4:  Both current serving cell and neighbour cells MBS session specific intended service area ID(s) are indicated in the MCCH.

Proposal 5: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

Proposal 6: SIB21 can indicate the subset of ISAs used for each frequency and the UE needs not prioritize (RRC_IDLE) or indicate (RRC_CONNECTED) the frequency of an MBS service of interest if the UE is not in any of the ISAs used in the frequency that provides the service.

# References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2410913, Report from Break-Out Session on NR NTN and IoT NTN, Session chair (ZTE)