3GPP TSG-RAN WG2 Meeting #129        	                       R2-2501049

Athens, GR, 17th-21st February 2025

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the NW side data collection and achieved the following agreements.

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

This contribution will discuss the remaining issues for NW side data collection.

- Discussion

- 2.1 NW side data collection for beam management

- Issue#1: FFS in last meeting

- In last meeting, RAN2 agreed that the UE reports to the network when the power state is low and when buffer is or may become full. FFS when it reports (before and/or after), and FFS how the reason for triggering of indication for the status (e.g. low power state, low memory) is signaled and if the reporting can be part of availability indication.

- Anyway, the UE will sent the availability indication to the network when the power state is low or buffer is full. For simplicity, we prefer to report the reason for triggering of indication for the status as part of availability indication.

- Proposal 1: The reason for triggering of indication for the status (e.g. low power state, low memory) is signaled as part of availability indication.

- In RAN2#127 meeting, RAN2 agreed that when UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. Therefore, we think it can be reported to the network after buffer is full. And this will not introduce additional impacts on UE power consumption since the measurement is stopped.

- Proposal 2: It can be reported to the network after buffer is full.

- In legacy logged MDT, the UE can indicate the available measurements via UE-measurementAvailable [2] in msg5, e.g. RRCSetupComplete, RRCResumeComplete, RRCReestablishComplete message.

- For NW data collection in AI/ML, considering the UE is in connected mode, msg5 is not applicable for availability indication. We think UAI can be used to send the availability indication to the network.

- Proposal 3: The availability indication can be sent to the network via UEAssistanceInformation massage.

- Issue#2: Data type for NW sided model and UE sided model

- During the study phase, RAN1 reply LS [4] provided the data content and typical data size for beam management use case. It can be seen that they are common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |
| Inference | UE-side | Beam prediction results | Small (10s of bits) | Time-critical | RAN1 has agreed to consider L1 signalling for this reporting |
|  | NW-side | L1-RSRPs, and Beam-IDs if needed, for Set B | See Note 1 for L1-RSRPs | Time-critical |  |
| Monitoring | UE-side | Event occurrence and/or calculated performance metrics (from UE to NW) See Note 4 | Small (10s of bits) | Near-real-time |  |
|  | UE-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |
|  | NW-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |

- Proposal 4: The data content is common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

- Issue#3: FFS on memory size

- RAN2 agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. In legacy logged MDT and QoE measurement, the minimum AS layer memory size is 64KB [4].

- TS 38.306, Section 4.2.18 Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. - TS 38.306, Section 6 Features; Condition. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

- For NW data collection, considering there are multiple use cases which needs training data, we think the legacy value is baseline and other larger value is preferred, i.e. the minimum AS layer memory size is at least 64KB.

- Proposal 5: The minimum AS layer memory size for training data logging is at least 64KB.

- Issue#4: FFS on logged time

- For temporal beam prediction, the time info is necessary for model training. In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [3]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [3]) which indicates the time of logging measurement results would be included in UEInformationResponse message. The similar principle can be reused for training data reporting to reduce the signaling overhead.

- Proposal 6: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logged measurement results, similar as logged MDT.

- 2.2 NW side data collection for Positioning

- 2.2.1 Positioning with gNB-sided model (case 3a)

- Figure 3. Positioning with gNB-sided model (case 3a)

- During the R18 study, the entity for gNB-sided model training can reside at gNB or OAM. Based on the following RAN1 agreement, the label and its related data (e.g., time stamp) for training can be generated by at least LMF. So the training data will be transmitted to gNB from LMF, and this has no impact on LPP.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

- Observation 1: For case 3a, the label and its related data for training can be generated by LMF then sent to gNB based on RAN1 agreement, which has no impact on LPP.

- Based on the following RAN1 agreement and conclusion, the measurements for determining model input are based on the existing SRS in R19 discussion, and the existing procedure can be reused in terms of SRS configuration for case 3a.

RAN1#116 Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. RAN1#118 Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

- Observation 2: The existing procedure of UL SRS configuration and measurement can be reused for case 3a based on RAN1 agreement.

- Based on observation 1 and 2, we think there is no additional impacts on Uu interface for case 3a for now. If further agreement is made in RAN1, we can revise it based on the RAN1’s progress.

- Proposal 7: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface for now based on RAN1’s agreement. And we can revise it if RAN1 has further progress on this case.

- 2.2.2 Positioning with LMF-sided model (case 3b)

- During the R18 study, the entity for LMF-sided model training resides at LMF and the LMF is the termination point for training data. Therefore the LMF-centric data collection should be considered, i.e. LMF configures the UE/gNB to initiate/terminate the data collection procedure.

- Figure 4. Positioning with LMF-sided model (case 3b)

For the content of collected data for LMF-sided model training, at least measurements (corresponding to model input, e.g. timing, power, and/or phase info) and location coordinates as model output are needed based on RAN1 reply LS [5]. During the R19 discussion, RAN1 has achieved the following agreements on the collected data for LMF-sided model.

RAN1#116bis Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. RAN1#118 Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

- For training data generation of case 3b, the label and its related data can be generated by PRU or LMF or TRP/gNB. If the training data is generated by gNB and terminated at LMF, NRPPa will be used and should be discussed in RAN3. If the training data is generated by PRU or non-PRU UE and terminated at LMF, LPP will be used and in RAN2 scope. Therefore, RAN2 should focus on the latter case.

- Proposal 8: For positioning with LMF-side model (case 3b), RAN2 focus on the case where the training data is generated by PRU or non-PRU UE and terminated at LMF.

# 3	Conclusion

Here are the proposals for NW side data collection.

Beam management

- Proposal 1: The reason for triggering of indication for the status (e.g. low power state, low memory) is signaled as part of availability indication.

- Proposal 2: It can be reported to the network after buffer is full.

- Proposal 3: The availability indication can be sent to the network via UEAssistanceInformation massage.

- Proposal 4: The data content is common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

- Proposal 5: The minimum AS layer memory size for training data logging is at least 64KB.

- Proposal 6: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logged measurement results, similar as logged MDT.

Positioning

- Observation 1: For case 3a, the label and its related data for training can be generated by LMF then sent to gNB based on RAN1 agreement, which has no impact on LPP.

- Observation 2: The existing procedure of UL SRS configuration and measurement can be reused for case 3a based on RAN1 agreement.

- Proposal 7: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface for now based on RAN1’s agreement. And we can revise it if RAN1 has further progress on this case.

- Proposal 8: For positioning with LMF-side model (case 3b), RAN2 focus on the case where the training data is generated by PRU or non-PRU UE and terminated at LMF.

# 4	Reference

- Draft_R2_128_Meeting_Report_v1

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions

- TS 38.306 NR; User Equipment (UE) radio access capabilities

- R2-2311720 Reply LS on Data Collection Requirements and Assumptions (R1-2310681)