3GPP TSG-RAN WG2 Meeting #129	R2-2501085

Athens, Greece, Feb. 17th – 21st, 2025

Source:	vivo

Title:	RA report optimization for SDT

Agenda Item:	8.10.5.1

Document for:	Discussion and Decision

- Introduction

At the RAN2#128 meeting, the following agreements were reached regarding failure causes for failed SDT in the RA report:

Agreements - The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. - The following failure causes can be logged for failed SDT in RA report: ⁻ upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; ⁻ upon random access problem indication is received from MCG MAC while SDT procedure is ongoing; ⁻ upon T319a expires; ⁻ upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; ⁻ FFS: unsuccessfully completed upon cell re-selection.

On the other hand, RAN3 sent an LS [1] to RAN2 to include the following information in the RA report for RA-SDT.

(3) Regarding R18 Leftovers: On SON for RA-SDT, RAN3 sees benefits in including in RA Report the time from the start of RA-SDT until the reporting of the RA report. The network can use it to determine the configuration that was applied at the time of the start of RA-SDT.

In this contribution, we will provide our views on the above issues.

- Discussion

  - Failure Cause in RA report

For the failure cause, the SDT procedure will be unsuccessfully completed for the following cases:

Once initiated, the SDT procedure is either: - successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or - unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or integrity check failure while SDT procedure is ongoing, or expiry of SDT-specific timing alignment timer or configuredGrantTimer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

From our understanding, within the causes, cell re-selection is not an abnormal case and is not needed for configuration optimization from network’s perspective. Therefore, cell re-selection does not need to be indicated in the failure cause of RA-SDT in RA-report.

Proposal 1: No need to include cell re-selection as a failure cause for failed RA-SDT in RA-Report.

  - RAN3 requirement of the elapsed time

In the LS [1], RAN3 sees benefits in including in RA Report the time from the start of RA-SDT until the reporting of the RA report. The network can use it to determine the configuration that was applied at the time of the start of RA-SDT. We think the intention is valid and RAN2 can further discuss the format of the elapsed time.

From our understanding, the intention of the elapsed time is similar to the timeSinceSHR in SHR and timeSinceFailure in RLF report.

timeSinceSHR This field is used to indicate the time elapsed since the execution of the last MobilityFromNRCommand towards the target EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link or handover) failure.

Therefore, we propose:

Proposal 2: Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value 172800 means 172800s or longer.

- Conclusion

Based on the above analysis, we make the following proposals:

Proposal 1: No need to include cell re-selection as a failure cause for failed RA-SDT in RA-Report.

Proposal 2: Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value 172800 means 172800s or longer.

- Reference

[1]	R2-2500026	LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT