3GPP TSG-RAN WG2 Meeting #129					R2-2501110

Athens, Greece, Feb. 17th - 21st, 2025

Title: 	Discussion on UE-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.4

Document for:	Discussion/Decision

- Introduction

This contribution is to address the following aspects as mentioned in the draft agenda of RAN2#129:

Discuss any of the aspects identified in RANP WF

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

- Discussion

Background

In [1], RAN2 has made some analysis of different data collection options for UE-side model training.

In [2], regarding couple of questions from SA2, RAN2 has concluded the answers for them. It is noted that for some questions, RAN2 has not evaluated/analyzed them, and thus more discussions in RAN2 may be needed.

In [3], a WF was endorsed at RAN#106, and we understand that some of them are related to the discussions in [2].

Discussion on the aspects identified in RANP WF for data collection options

For RANP WF, we think that the analysis of E2E procedures is needed.

In general, UE-sided data collection has two parts:

Data collection configuration. It includes data collection configuration trigger, configuration, and data collection process at UE side.

Data transfer. Once the UE has collected the data, data transfer to UE server/OTT server needs to be triggered.

At RAN2#128 meeting, when discussing LCM for UE-sided model for BM, RAN2 made the following agreements [4]:

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

For the above highlighted part, it is related to Data collection configuration and it can be triggered by UE or by network.

In the RANP WF[3], it mentions the following, and we understand that Option 1a, 2 and 3 should be further analyzed.

For RAN2

- During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b)

  - Study RAN aspects related to data transfer over UP

  - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

  - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

  - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

  - Discuss on the scalability aspects of CP

  - RAN2 can Liaise with other WGs if/when needed

In the following sections, we analyze Option 2, Option 3 and Option 1a separately. For each option, UE-triggered and NW-triggered options are listed and analyzed. It is noted that we assume UE-sided model for beam management here.

Option 2 (via CN)

UE-triggered data collection

For UE-triggered data collection, Step 1, 2 and 3 are aligned with "the network can provide the data collection configuration (at any point in time) with UE request" in RAN2#128 minutes. And then, Step 4, 5 and 6 are about data transfer. Details can be found in figure 1.

In figure 1, it can be seen that NW has the control of both data collection configuration and data transfer, e.g. after step 2, NG-RAN can decide whether to send data collection configuration to the UE, and after step 4, CN NF can decide whether to initiate data transfer for the UE.

Figure 1: UE-triggered data collection (data collection configuration + data transfer)

In addition to figure 1, it may happen that UE has received data collection configuration and data collection is already occurred, and then only data transfer is network control. Details can be found in figure 2. Similar to figure 1, NW has the control of data transfer.

Figure 2: UE-triggered data collection (data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

For data transfer over UP, it relies on PDU Session Establishment procedure, and then UE could use PDU Session to transfer data from UE to CN NF.

For Step 4, the motivation is that CN NF may have no idea whether the UE has available data to transfer or not, so this procedure may provide some useful information for CN NF. It may be transmitted via NAS (CP), PDU session (UP), or signaling in Uu+signalling in Ng (mixed). CP and UP options should be transparent to NG-RAN, while mixed option may have impacts to NG-RAN.

For NW decision after Step 4, we provide analysis in the following bullet number (3).

For Step 5, it should be transparent to NG-RAN.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

For data collection configuration, NG-RAN is involved and it has control of data collection configuration, as per the existing agreement, as mentioned above.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

For data transfer, even if NG-RAN is transparent if using NAS or PDU session for doing data transfer, it still needs NG-RAN resources to do that. In this case, for NW decision after Step 4, the network may need to take the following aspects into account: congestion status in Uu, RAN resources for data transfer, and perhaps network policies. Thus, CN NF may need to co-ordinate with NG-RAN before initiating Step 5. For example, if there is congestion in Uu, CN NF may decide to postpone Step 5, and otherwise, CN NF may decide to initiate Step 5 as soon as possible. However, RAN2 can assume this to be handled using existing PDU session management procedures and existing admission control and QoS framework, unless agreed otherwise by SA2.

For visibility aspect, RAN2 has already agreed on Opt A) Full visibility for standardized data contents, while Opt B) Partial visibility and Opt C) No standardized visibility are FFS. In our understanding, Opt A) may have two ways:

- if NAS is used for data transfer, all data contents are standardized

- if PDU session is used for data transfer, all data contents are standardized

For Opt B) and Opt C), MNO has no visibility for some data contents, and then it may lead to some issues, such as user privacy leakage, test challenges, management challenges and so on. In summary, we do not support Opt B) and Opt C) for visibility aspect.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

As we analyzed in (1), for Step 4, if it is transmitted via signaling in Uu+signalling in Ng (mixed), it may impact Ng interface and CN NF.

(5) Others

For Step 1, we think it can be left to UE implementation. The discussion of Step 6 is up to SA2.

NW-triggered data collection

For NW-triggered data collection, Step 1, 2 and 3 are aligned with "the network can provide the data collection configuration (at any point in time) without UE request" in RAN2#128 minutes. And then, Step 4, 5 and 6 are about data transfer. Details can be found in figure 3.

In figure 3, it can be seen that NW has the control of both data collection configuration and data transfer, e.g. after step 2, NG-RAN can decide whether to send data collection configuration to the UE, and after step 4, CN NF can decide whether to initiate data transfer procedure for the UE.

Figure 3: NW-triggered data collection (data collection configuration + data transfer)

In addition to figure 3, we think it is possible that the UE is doing data collection, and then it only needs to co-ordinate with NW to do data transfer. Details can be found in figure 4. Similar to figure 3, NW has the control of data transfer.

Figure 4: NW-triggered data collection (data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

For data transfer over UP, it relies on PDU Session Establishment procedure, and then UE could use PDU Session to transfer data from UE to CN NF.

During Step 2 and 3, we think CN NF should know which UE has been configured with data collection configuration, and then CN NF can request the UE to do data transfer. It may be transmitted via NAS (CP), PDU session (UP), or signaling in Ng+signalling in Uu (mixed). CP and UP options should be transparent to NG-RAN, while mixed option may have impacts to NG-RAN.

For NW decision after Step 4, we provide analysis in the following (3).

For Step 5, it should be transparent to NG-RAN.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

In figure 3, after CN NF has got data collection request from OTT server, there are two possibilities for NW to choose proper UEs:

a) it is NG-RAN's responsibility to choose proper UEs. CN NF may request NG-RAN to select some UEs to initiate data collection configuration. After step 2, NG-RAN may accept or reject the request from CN NF by its own decisions. If NG-RAN accepts the request from CN NF, it may trigger the data collection configuration for some UEs, and NG-RAN is involved and it has control of data collection configuration.

b) it is CN's responsibility to choose proper UEs. CN NF may select some UEs, and request NG-RAN to initiate data collection configuration for these UEs. And then, NG-RAN may trigger the data collection configuration for some UEs, and NG-RAN is involved and it has control of data collection configuration.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

There is similar analysis as (3) in section 2.2.1.1.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

As we analyzed in (1), for Step 4, if it is transmitted via signaling in Ng+signaling in Uu (mixed), it may impact Ng interface and CN NF.

(5) Others

The discussions of Step 1 and Step 6 are up to SA2.

Summary

For Option 2, the following table shows a summary of analysis.

|  | UE-triggered data collection | NW-triggered data collection |
| --- | --- | --- |
| (1) Study RAN aspects related to data transfer over UP | As a baseline, data transfer request sent from the UE to CN can be transparent to RAN, i.e. rely on direct signalling between the UE and CN. Whether there is any impact on RAN depends on detailed solution which should be discussed by SA2. | As a baseline, data transfer request sent from the CN to UE can be transparent to RAN, i.e. rely on direct signalling between the UE and CN. Whether there is any impact on RAN depends on detailed solution which should be discussed by SA2. |
| (2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection | NG-RAN is involved and it has control of data collection configuration | Either NG-RAN or CN can be responsible for choosing proper UEs for data collection. NG-RAN is involved and it has control of data collection configruation. |
| (3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) | When CN NF needs some information for NG-RAN (e.g. congestion status in Uu), NG-RAN may be involved, but existing procedures can be used for that purpose. Visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage. | The same as the analysis of UE-triggered data collection |
| (4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain | See analysis for bullet number (1) | See analysis for bullet number (1) |
| (5) Others | For Step 1, we think it can be left to UE implementation. The discussion of Step 6 is up to SA2. | The discussions of Step 1 and Step 6 are up to SA2. |

We have the following proposals:

Proposal 1: For RAN aspects related to data transfer over UP and aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain for option 2, as a baseline, data transfer request between UE and CN can be transparent to RAN, i.e. rely on direct signalling between the UE and CN. Whether there is any impact on RAN depends on detailed solution which should be discussed by SA2.

Proposal 2: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 2, NG-RAN is involved and it has control of data collection configuration. For NW-triggered data collection, either NG-RAN or CN can be responsible for choosing proper UEs for data collection.

Proposal 3: For NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) for option 2, when CN NF needs some information for NG-RAN (e.g. congestion status in Uu), NG-RAN may be involved, but existing procedures can be used for that purpose. visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage.

Proposal 4: For option 2, the interaction between CN NF and OTT server can be left to SA2.

Option 3 (via OAM)

In the SA5 LS [5], it mentions the following:

For option 3:

CP tunnel:

- The feasibility for the functionality of “Server for data collection for UE-side model training” to be carried by TCE as defined in SA5 MDT mechanism or MCE as defined in SA5 QoE mechanism for CP tunnel can be studied in SA5. SA5 will evaluate potential enhancement that may be needed based on the progress in RAN.

UP tunnel:

- The feasibility of UP tunnel between UE and OAM needs to be studied.

It can be seen that Option 3-CP can reference MDT framework, and Option 3-UP is to be studied. So we focus on Option 3-CP in the following sections.

Observation 1: For Option 3, RAN2 can focus on CP option for now, and UP option can wait for SA5 progress.

UE-triggered data collection

For UE-triggered data collection, Step 1, 2 and 3 are aligned with "the network can provide the data collection configuration (at any point in time) with UE request" in RAN2#128 minutes. And then, Step 4, 5 and 6 are about data transfer (RRC signalling is used for Step 4 and Step 5 in Uu interface). Details can be found in figure 5.

In figure 5, it can be seen that NW has the control of both data collection configuration and data transfer, e.g. after step 2, NG-RAN can decide whether to send data collection configuration to the UE, and after step 4, NG-RAN can decide whether to initiate data transfer for the UE.

Figure 5: UE-triggered data collection (data collection configuration + data transfer)

In addition to figure 5, it may happen that UE has received data collection configuration and data collection is already occurred, and then only data transfer is network control. Details can be found in figure 6. Similar to figure 5, NW has the control of data transfer.

Figure 6: UE-triggered data collection (data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

As we mentioned above, Option 3-UP is still under SA5 study, so we suggest to wait for SA5 progress.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

For data collection configuration, NG-RAN is involved and it has control of data collection configuration.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

For data transfer, NG-RAN uses RRC signalling to do Step 4 and Step 5. For NW decision after Step 4, the network may take the following aspects into account: congestion status in Uu, RAN resources for data transfer, and perhaps network policies, and the network decision can be left to implementation.

For visibility aspect, RAN2 has already agreed on Opt A) Full visibility for standardized data contents, while Opt B) Partial visibility and Opt C) No standardized visibility are FFS. In the current MDT mechanism, all data contents are standardized, so we think if new data is to be collected via MDT mechanism, all new data should be standardized.

For Opt B) and Opt C), MNO has no visibility for some data contents, and then it may lead to some issues, such as user privacy leakage, test challenges, management challenges and so on. In summary, we do not support Opt B) and Opt C) for visibility aspect.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

As shown in figure 5 and 6, once NG-RAN has collected data from the UE, it can transfer the data from NG-RAN to OAM, and it is possible to enhance the current MDT mechanism to do that.

(5) Others

For Step 1, we think it can be left to UE implementation. The discussion of Step 6 is up to other WGs.

NW-triggered data collection

For NW-triggered data collection, Step 1, 2 and 3 are aligned with "the network can provide the data collection configuration (at any point in time) without UE request" in RAN2#128 minutes. And then, Step 4, 5 and 6 are about data transfer. Details can be found in figure 7.

In figure 7, it can be seen that NW has the control of both data collection configuration and data transfer, e.g. after step 2, NG-RAN can decide whether to send data collection configuration to the UE, and after step 4, NG-RAN can decide whether to initiate data transfer procedure for the UE.

Figure 7: NW-triggered data collection (data collection configuration + data transfer)

In addition to figure 7, we think it is possible that the UE is doing data collection, and then it only needs to co-ordinate with NW to do data transfer. Details can be found in figure 8. Similar to figure 7, NW has the control of data transfer.

Figure 8: NW-triggered data collection (data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

As we mentioned above, Option 3-UP is still under SA5 study, so we suggest to wait for SA5 progress.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

For data collection configuration, NG-RAN is involved and it has control of data collection configuration.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

There is similar analysis as (3) in section 2.2.2.1.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

As shown in figure 7 and 8, once NG-RAN has collected data from the UE, it can transfer the data from NG-RAN to OAM, and it is possible to enhance the current MDT mechanism to do that.

(5) Others

The discussions of Step 1 and Step 6 are up to other WGs.

Summary

For Option 3, the following table shows a summary of analysis.

|  | UE-triggered data collection | NW-triggered data collection |
| --- | --- | --- |
| (1) Study RAN aspects related to data transfer over UP | Option 3-UP is to wait for SA5 progress. | Option 3-UP is to wait for SA5 progress. |
| (2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection | NG-RAN is involved and it has control of data collection configuration | NG-RAN is involved and it has control of data collection configuration |
| (3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) | There is NG-RAN involvement. Visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage. | The same as the analysis of UE-triggered data collection |
| (4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain | The data transfer from NG-RAN to OAM can be based on the current MDT mechanism. | The same as the analysis of UE-triggered data collection |
| (5) Others | For Step 1, we think it can be left to UE implementation. The discussion of Step 6 is up to SA5. | The discussions of Step 1 and Step 6 are up to other SA5. |

We have the following observations and proposals:

Observation 1: For Option 3, RAN2 can focus on CP option for now, and UP option can wait for SA5 progress.

Proposal 5: For RAN aspects related to data transfer over UP and aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain for option 3, Option 3-UP is to wait for SA5 progress, and Option 3-CP can be based on the current MDT mechanism.

Proposal 6: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 3, NG-RAN is involved and it has control of data collection configuration.

Proposal 7: For NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) for option 3, there is NG-RAN involvement. Visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage.

Proposal 8: For option 3, the interaction between OAM and OTT server can be left to SA5.

Option 1a

UE-triggered data collection

For UE-triggered data collection, the signalling flow is similar to section 2.2.1.1 and 2.2.2.1, i.e. Step 1, 2, and 3 can re-use the above analysis for UE-triggered data collection. For the data transfer, Option 1a can be up to UE implementation and no 3GPP specification impact is expected.

Unlike UE-triggered data collection for Option 2/3, we do not provide UE-triggered data collection for data transfer only as it is totally up to UE implementation.

Figure 9: UE-triggered data collection (data collection configuration + data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

No RAN impacts.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

For data collection configuration, NG-RAN is involved and it has control of data collection configuration.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

No NG-RAN involvement.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

No 3GPP impacts.

(5) Others

For Step 1, we think it can be left to UE implementation.

NW-triggered data collection

For UE-triggered data collection, the signalling flow is similar to section 2.2.1.2 and 2.2.2.2, i.e. Step 1, 2, and 3 can re-use the above analysis for NW-triggered data collection. For the data transfer, Option 1a can be up to UE implementation and no 3GPP specification impact is expected.

Figure 10: NW-triggered data collection (data collection configuration + data transfer)

Here is an initial analysis of RANP WF:

(1) Study RAN aspects related to data transfer over UP

No RAN impacts.

(2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection

Based on the analysis in section 2.2.1.2 and 2.2.2.2, we think NG-RAN is involved and it has control of data collection configuration.

(3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion)

No NG-RAN involvement.

(4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain

No 3GPP impacts.

(5) Others

The discussions of Step 1 are up to other WGs.

Summary

For Option 1a, the following table shows a summary of analysis.

|  | UE-triggered data collection | NW-triggered data collection |
| --- | --- | --- |
| (1) Study RAN aspects related to data transfer over UP | No RAN2 impacts | No RAN2 impacts |
| (2) Discuss level of NG-RAN involvement in the control and configuration of UE side data collection | NG-RAN is involved and it has control of data collection configuration | NG-RAN is involved and it has control of data collection configuration |
| (3) Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) | No NG-RAN involvement | No NG-RAN involvement |
| (4) Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain | No 3GPP impacts. | No 3GPP impacts. |

We have the following proposal:

Proposal 9: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 1a, NG-RAN is involved and it has control of data collection configuration.

The scalability aspects of CP

For CP options, there are Option 2-CP (via NAS signalling) and Option 3-CP (via RRC signalling).

Take Option 3-CP for example, firstly, we think all data contents should be standardized and RAN1 should decide on the contents. Secondly, the main concern is about how to transfer large training data from UE to NW via CP options. Our views are as below:

- As RAN1 mentioned, the transmission latency is relaxed

- Option 3-CP is similar to NW-sided data collection. For NW-sided data collection, RAN2 has some progress, and generally the UE can collect data based on network configuration, and then NW can use UEInformationRequest/Response to request the UE to transfer the data. In addition, the UE may send some assistance information to NW to help NW decision. In general, we think NW-sided data collection method is flexible enough, and it balances the impact on UE side and data transmission efficiency

For Option 2-CP, there is similar analysis as Option 3-CP, and thus Option 2-CP can balance the impact on UE side and data transmission efficiency.

Proposal 10: No scalability issues are observed for Option 2-CP and Option 3-CP.

Discussion on FFSes from RAN2#128 meeting

In the last RAN2 meeting, the following agreements have been agreed for data collection:

Agreement: 5. For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. 6. The network can provide the data collection configuration (at any point in time), with or without UE request. 7. The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. 8. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

On the UE request for data collection in Agreement 5 above, there is a FFS on contents of the UE request. The request sent by UE may contains preference information for network to determine when to send signal to stop UE data collection. Because data collection is performed by the UE, the UE knows how much data is sufficient for the training or suitable for UE memory. For example, the request may contain the data volume or duration of data collection. Network can use this preference information from the UE to decide on when to stop data collection and/or to de-configure the data collection configuration provided by network.

Proposal 11: The request sent by UE may contains preference information for network to determine when to send signal to stop UE data collection. (e.g., information may be the data volume or duration of data collection).

An UE request data collection when it is requested by OTT service. This request from the OTT service can come at any time (e.g. when the UE is out-of-5G coverage (e.g. via WiFi), when UE is in a cell not supporting data collection etc.). This OTT request can be pending at the UE until the UE enters 5G coverage and the cell supports data collection.

If the UE is in RRC_Connected mode and the UE AS layer receives the OTT request of data collection, the UE can directly initiate uplink RRC message for UE request. If the UE is in RRC_Idle or RRC_Inactive and the UE AS layer receives the OTT request of data collection, the UE may wait for state transition due to another traffic request, or the UE may initiate a state transition just because of this data collection request. In our understanding, the former one has minimal impacts to 3GPP specifications, while the later one may have some impacts, e.g. whether a new establishmentCause/resumeCause is needed or not.

Proposal 12: If the UE is in RRC_Idle or RRC_Inactive and the UE AS layer receives the OTT request of data collection, it is proposed RAN2 to discuss whether the UE is allowed to do state transition just because of the data collection request.

For other contents in the request, it relies on the configuration defined by RAN1. For example, if the UE wants to collect L1-RSRP, the UE may need to send the CSI configuration identification, e.g., CSI config ID to the NW. Thus, RAN2 should wait for RAN1’s progress on the data collection configuration.

Proposal 13: For other contents in the UE request for data collection, RAN2 should wait for more progress on the data collection configuration in RAN1.

In the last meeting, RAN2 agreed that the network can configure whether UE is allowed to initiate requests for data collection. The gNB may select the use cases that the UE can request. For example, gNB can select the UE to request data collection configuration for BM-Case 1 but not BM-Case 2. We think that such selection is useful as the network can use it to control the UE request (FFS on whether such selection is sent in dedicated UE signaling or in SIB).

Observation 2: It is useful for the network to select certain use cases where the UE can request for data collection.

Proposal 14: Network can configure whether the UE is allowed to initiate requests for data collection for certain use cases (e.g. BM-Case 1 and/or BM-Case 2 etc.)

- Conclusion

Based on our analysis in section 2, we have the following observations and proposals:

For Option 2 (via CN)

Proposal 1: For RAN aspects related to data transfer over UP and aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain for option 2, as a baseline, data transfer request between UE and CN can be transparent to RAN, i.e. rely on direct signalling between the UE and CN. Whether there is any impact on RAN depends on detailed solution which should be discussed by SA2.

Proposal 2: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 2, NG-RAN is involved and it has control of data collection configuration. For NW-triggered data collection, either NG-RAN or CN can be responsible for choosing proper UEs for data collection.

Proposal 3: For NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) for option 2, when CN NF needs some information for NG-RAN (e.g. congestion status in Uu), NG-RAN may be involved, but existing procedures can be used for that purpose. visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage.

Proposal 4: For option 2, the interaction between CN NF and OTT server can be left to SA2.

For Option 3 (via OAM)

Observation 1: For Option 3, RAN2 can focus on CP option for now, and UP option can wait for SA5 progress.

Proposal 5: For RAN aspects related to data transfer over UP and aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain for option 3, Option 3-UP is to wait for SA5 progress, and Option 3-CP can be based on the current MDT mechanism.

Proposal 6: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 3, NG-RAN is involved and it has control of data collection configuration.

Proposal 7: For NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion) for option 3, there is NG-RAN involvement. Visibility Opt B) (Partial visibility) and Opt C (No standardized visibility) are not supported as they may lead to big challenges, e.g. user privacy leakage.

Proposal 8: For option 3, the interaction between OAM and OTT server can be left to SA5.

For Option 1a

Proposal 9: For level of NG-RAN involvement in the control and configuration of UE side data collection for option 1a, NG-RAN is involved and it has control of data collection configuration.

For scalability aspects of CP

Proposal 10: No scalability issues are observed for Option 2-CP and Option 3-CP.

For discussion of FFSes from RAN2#128

Observation 2: It is useful for the network to select certain use cases where the UE can request for data collection.

Proposal 11: The request sent by UE may contains preference information for network to determine when to send signal to stop UE data collection. (e.g., information may be the data volume or duration of data collection).

Proposal 12: If the UE is in RRC_Idle or RRC_Inactive and the UE AS layer receives the OTT request of data collection, it is proposed RAN2 to discuss whether the UE is allowed to do state transition just because of the data collection request.

Proposal 13: For other contents in the UE request for data collection, RAN2 should wait for more progress on the data collection configuration in RAN1.

Proposal 14: Network can configure whether the UE is allowed to initiate requests for data collection for certain use cases (e.g. BM-Case 1 and/or BM-Case 2 etc.)

- References

R2-2407807, CR on RAN2 inputs to TR 38.843

R2-2411152, Reply LS to SA2 on AIML data collection, Source: RAN2, To: SA2, Cc: TSG RAN, RAN1, RAN3, TSG SA, SA5, SA3

RP-243307, Way forward on AIML UE sided data collection, RAN2 Chair (InterDigital)

Draft_RAN2_128_Meeting_Report_v1

R2-2409532	Reply LS on AIML Data Collection (S5-246299; contact Nokia)	SA5	LS in	Rel-18	FS_AIML_MGT_Ph2	To:RAN2	Cc:RAN, RAN1, RAN3, SA, SA2, SA3