3GPP TSG-RAN WG2 Meeting #129                                                             R2-2501121

Athens, Greece, Feb. 17th – 21st, 2025

Source:	TCL

Title:	Discussion on NW side data collection for positioning

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

Data collection is a very important function in the AI/ML-based LCM, which is used to provide the input data to the different LCM aspects (i.e., model inference, monitoring, and training). In the recent RAN2#128 meeting, there were full discussions on logging conditions/events and starting/stopping of logging for NW side data collection. Some conclusions were drawn as follows [1].

Agreements - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

In this contribution, we provide our views on NW side data collection for the positioning use case, focusing on the first priority Case 3a and Case 3b.

- Discussion

2.1 NW side data collection for Case 3a

The training data can be autonomously collected by gNB or OAM for Case 3a. As for gNB-centric data collection, SRS is configured reusing the legacy RRC message for measurement data collection. The label data (intermediate positioning measurement) can be calculated based on the UE location information, which can be contained in the RRM measurement report associated with the corresponding SRS configuration for data collection to match the measurement data and label data. The SRS configuration can be contained in the RRM measurement report configuration or associated to the RRM measurement report configuration via a measurement ID.

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

As for OAM-centric data collection, the Immediate MDT with RAN measurement configuration can be extended. The channel measurement (e.g., DP, PDP, CIR) by RAN should be additionally supported for Immediate MDT so that OAM can collect the measurement data. The content of channel measurement waits for RAN1 discussion. The measurement collection can be triggered by event or at the end of measurement collection period, and the trigger events can be further discussed. Besides, the UE location information provision is supported in the legacy Immediate MDT so there should be no extension for the label data collection.

Proposal 2: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a. The content of channel measurement waits for RAN1 discussion.

2.2 NW side data collection for Case 3b

The label data can be generated by PRU, non-PRU UE and LMF for Case 3b, which is captured as follows [2].

Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed.

If the label data is generated by PRU and LMF, LMF can easily match the measurement data and label data. However, the non-PRU UE may generate the label data via PRS. In this case, the UE may move within the time gap between the PRS reception and SRS transmission, which leads to the mismatch between the measurement data and label data. Hence, the data association should be considered to guarantee the reliability of model training. Intuitively, LMF can request the label data from UE via LPP signaling and then UE sends the label data to LMF to check validity or sends the label data after self-validation along with the assistance information. The assistance information for data association waits for RAN1 discussion, which is captured as follows [2].

Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

Proposal 3: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- Conclusion

We have the following proposals:

Proposal 1: The RRM measurement report containing UE location information can be associated with the corresponding SRS configuration for label data collection for Case 3a.

Proposal 2: The Immediate MDT with NW-side channel measurements, e.g., DP, PDP, CIR, can be introduced to support OAM-centric training data collection for Case 3a. The content of channel measurement waits for RAN1 discussion.

Proposal 3: The data association procedure should be considered for Case 3b and the assistance information for data association waits for RAN1 discussion.

- References

- RAN2 #128 Chair Notes

- RAN1 #117 Chair Notes