3GPP TSG RAN2 Meeting #129                                                                R2-2501132

17th Feb. – 21st Feb. 2025, Athens, Greece

Agenda item:	8.4.2

Source:	Samsung

Title:	Procedure and Configuration of LP-WUS in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1]. And during the previous meeting #127bis & 128, with respect to procedure and configuration of LP-WUS in RRC idle/inactive mode agenda, RAN2 agreed as follows:

#127bis LP-WUS entry/exit condition If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. Subgrouping For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. RAN2 inform this conclusion to SA2/CT1/RAN3. This LS is approved in R2-2409226 #128 Subgrouping FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

And RAN1 reached the following agreements regarding procedures and configurations for idle/inactive modes during meeting #119[2] :

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1 Agreement At least for the 1:1 LO to PO mapping, - The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4. Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the findings from the LP-WUS/WUR study conducted during the Study Item (SI) phase in Release 18, it has been proposed that LP-WUS/WUR should be implemented as a power saving mechanism specifically designed for RRC_IDLE and RRC_INACTIVE mode devices. Furthermore, the concept of an "ultra-deep sleep state" has emerged as a potential power saving strategy for both RRC_IDLE and RRC_INACTIVE modes. By leveraging LP-WUS/WUR, a user equipment's (UE) Main Receiver (MR) can effectively transit into an ultra-deep sleep state, thereby achieving significant power savings. While in this ultra-deep sleep state, the UE can use its LP-WUS/WUR functionality to continuously monitor network signaling activities, waking up the MR when necessary.

- Entry/Exit condition

According to the previous RAN2 discussion, now we have the entry/exit condition(s) of LP-WUS monitoring configured in SIB (e.g., SIB1).

Since with the agreed entry condition and with the NR measurement already ruled out from the exit condition, now the remained discussion is with respect to exit condition of LP-WUS monitoring, which is the serving cell measurement result based on LR below a configured threshold.

So far the RAN2 agreements related with entry/exit condition of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode are as follows:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements.

Summarizing the above agreements, we have the below agreed conditions:

Observation 1. Conditions of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

- Separate entry thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

- Separate exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Now the remaining debate revolves around incorporating extra entry/exit criteria, is specifically "low-mobility." Given the scenarios where LP-WUS monitoring occurs while UEs operate in RRC idle/inactive modes, rapid movement within a cell might cause inaccurate behavior due to reliance solely on measurement-based thresholds.

For instance, consider high-speed moving devices like those on vehicles or trains. These UEs aren't ideal candidates for LP-WUS monitoring because they'll quickly transition back to regular MR paging due to their high-speed. Consequently, frequent toggling between these states leads to ping-pong and even increased energy usage of the device.

To address this issue, we propose to integrate a "low mobility" criterion as an optional feature that networks can configure according to specific requirements. This addition aims to enhance efficiency and reduce unnecessary power consumption during LP-WUS operations.

Proposal 1: The entry/exit conditions of LP-WUS monitoring may optionally include low mobility criterion based on serving cell quality from MR (FFS LR)

- Subgrouping

To further enhance the energy saving gain, RAN2 agreed to take PEI subgrouping method as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping.

For UEs in RRC idle/inactive state, LP-WUS can either replace PEI (LP-WUS → PO) or coexist with PEI (LP-WUS → PEI → PO). Thus, in the above two different cases, subgrouping of LP-WUS can limit the monitoring of PF/PO, or PEI occasions, respectively.

- Scenario 1) When LP-WUS replaces PEI and subgroups are applied on LP-WUS, it reduces the number of PF/PO that UEs need to monitor.

- Scenario 2) When both LP-WUS and PEI exist and subgroups are applied on LP-WUS and PEI, it reduces the number of PEI occasions/ PF/PO that UEs need to monitor.

For Scenario 1, where UE monitors PO upon receiving LPWUS signal, UEs monitoring a PO can be divided into several LPWUS subgroups based on UE_ID.  LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +   
                  (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

For Scenario 2, where UE first monitors PEI upon receiving LPWUS signal and then monitors PO (if indicated by PEI).  LP WUS subgroup ID based on UE_ID can be determined independently from paging subgroup ID used for PEI as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +  
(LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Some companies have proposed to consider PEI subgrouping while determining the LPWUS subgroup ID. In our view this cannot reduce false alarms or triggers to wakeup MR. It may reduce PDCCH monitoring in PF/PO after receiving PEI. However, when LP WUS is configured, power savings from reducing PDCCH monitoring in PF/PO after receiving PEI is not significant compared to power saving from MR deep sleep/LP WUS monitoring.

Therefore, we support to reuse the formula for UE_ID based PEI subgrouping, for LP-WUS.

Proposal 2. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

- Separate Band Issue

During the recent RAN plenary meeting, Rel-19 LP-WUS separate band issues have been discussed [3][4] and made the following way forward:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

And the considered solutions for intra-frequency case are as follows[4]:

| Scenarios | Overload on LR band | Limited Usage of LP-WUS | Expected specification efforts |
| --- | --- | --- | --- |
| Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) | Yes | No | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline. - Frequency priority configured for LP-WUS capable UE in SIB |
| Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) | No | No | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in SIB or dedicated signalling. - UE needs to perform cell reselection following the legacy frequency priority after waking up. |
| Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) | Highly dependent on the awareness of load in LR band by the gNB | Dependent on LR band load | - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in dedicated signalling. |

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3:  MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

We propose to narrow down our options from three to two by removing Solution 2, which significantly impacts specifications across RAN1-RAN4 due to increased latency and higher chances of increased initial access latency.

In order to enable UEs to receive paging messages on one band while performing Random Access procedures on another, they must undergo re-selection processes. Given that the LR band serves as the outcome of initial cell selection prioritization, this re-selection criterion must be changed after paging message receipt. Consequently, these redundant re-selections inevitably prolong initial access times.

Proposal 3: Regarding the resolution of separate-band concerns, RAN2 will not consider Solution 2 involving UE behaviors associated with receiving paging messages and executing RACH operations on distinct frequency bands.

- 3. Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. Conditions of LP-WUS monitoring when UE is in RRC Idle/ Inactive mode

Entry condition(s)

- If the serving cell quality from MR (or, from both MR and LR) is above threshold(s)

- The metrics for serving cell quality includes (LP-)RSRP and optional (LP-)RSRQ

- Separate entry thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Exit condition(s)

- If the serving cell quality from LR is below a threshold

- The metrics for serving cell quality includes LP-RSRP and optional LP-RSRQ

- Separate exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 1: The entry/exit conditions of LP-WUS monitoring may optionally include low mobility criterion based on serving cell quality from MR (FFS LR)

Proposal 2. LP WUS subgroup ID based on UE_ID is determined as follows:

LP WUS subgroup ID = (floor (UE_ID/(N*Ns)) mod LPWUSsubgroupsNumForUEID) +               (LPWUSsubgroupsNumPerPO - LPWUSsubgroupsNumForUEID)

- LPWUSsubgroupsNumForUEID: number of LPWUS subgroups for UE_ID based LPWUS subgrouping in a PO, which is broadcasted in system information

- LPWUSsubgroupsNumPerPO: number of LPWUS subgroups per PO, which is broadcasted in system information

Proposal 3: Regarding the resolution of separate-band concerns, RAN2 will not consider Solution 2 involving UE behaviors associated with receiving paging messages and executing RACH operations on distinct frequency bands.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] Draft_Minutes_report_RAN1#119_v020, Draft Report of 3GPP TSG RAN WG1 #119 v0.2.0, RAN1

[3] RP-243266 Way forward on Rel-19 LP-WUS separate band issues  RAN2 chair (InterDigital)

[4] RP-242656 On LR and MR operating in different bands in release 19 Vodafone, Vivo, Huawei, HiSilicon