3GPP TSG-RAN WG2 Meeting #129	R2-2501143

Athens, Greece, Feb 17 – 21, 2025

Agenda item:	8.7.4.2

Source:	Samsung

Title:	DSR enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, RAN2 made the following agreements for DSR enhancements:

Agreements on DSR enhancements - Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this paper, we further discuss the essential aspects on the enhanced DSR.

Discussion

Triggering Threshold and Reporting Threshold

In Rel-18 DSR, the triggering threshold is used in both DSR triggering and reporting. In Rel-19 enhanced DSR, since we have defined the concept of reporting threshold for DSR reporting, one thing that has not been discussed explicitly so far is whether the triggering threshold is also used for reporting in the case of enhanced DSR. That is, whether the triggering threshold is also used as reporting threshold, participating in remaining time portion division for enhanced DSR.

Proposal 1: RAN2 to discuss whether triggering threshold is used as reporting threshold for enhanced DSR reporting.

MAC CE Format

Compared to the Rel-18 DSR, the most important extension for enhanced DSR is to introduce the extension bit/field, since there could be varying number of field set, i.e., [BT, E, remaining time, buffer size], per LCG, due to the possibility of absence of the field set of a certain remaining time portion.

As illustrated in Fig. 1, the reserved bit in legacy DSR MAC CE is replaced by Extension (E) bit, which indicates whether further delay information of the remaining time portion follows for the same LCG or not. Note that we do not need to define how to individually indicate which remaining time portion the presented delay information is associated to, since the reported remaining time value implicitly indicates the one it belongs to.

Fig. 1. Proposed enhanced DSR MAC CE format.

Proposal 2. RAN2 to consider the MAC CE format for enhanced DSR, where Extension (E) bit indicates whether further delay information of remaining time portion is present for the same LCG or not.

Besides, the order of the field placement of the enhanced DSR MAC CE is worth discussing, in particular, from two aspects, i.e., 1) the field set order within the same LCG, and 2) the field set order among different LCGs.

For example, given that the set of fields, i.e., [BT, E, Remaining Time, Buffer Size], for a remaining time portion is reported in two consecutive octets (as illustrated in Fig. 1), the multiple such field sets belonging to the same LCG, can be placed consecutively in ascending order of the corresponding remaining time value, such that the most urgent delay information is placed firstly within the same LCG.

For the order of the field sets among different LCGs, we can follow the same approach as that of the legacy DSR, i.e., in ascending order of LCGi.

Proposal 3. RAN2 to consider the MAC CE format for enhanced DSR, where the field set placement is based on 1) ascending order of the value of the remaining time field within LCG, and 2) ascending order of the LCGi among LCGs.

DSR Format Selection

It is clear that if there is at least one LCG with more than one remaining time portion to be reported, the enhanced DSR MAC CE should be selected.

The case for further discussion is whether to use legacy DSR or enhanced DSR MAC CE, for the case that all the LCGs with delay information to be reported, have only one remaining time portion to be reported, respectively.

We think whether to use the legacy DSR or enhanced DSR for the above case should be studied after the detailed MAC CE format for the enhanced DSR is determined, since before that it is unclear whether extra signalling overhead is encountered when using enhanced DSR, compared to the legacy DSR.

Proposal 4. RAN2 to postpone the discussion on DSR format selection between legacy DSR and enhanced DSR, until the MAC CE format of enhanced DSR is determined.

Non-Delay-Critical Data Reporting

In the last meeting, we have agreed that

- The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

However, there are some ambiguities requiring further clarifications related to the agreement above.

First of all, it is better to confirm which non-delay critical data the agreement indicates.

Specifically, given that we have already allowed that the reporting threshold can be larger than the triggering threshold (since we agreed it is up to NW implementation to configure them), the non-delay critical data belonging to the remaining time portion that is lower-bounded by the triggering threshold and upper-bounded by the reporting threshold, is already supposed to be reported for the corresponding remaining time portion. So, the non-delay critical data that the agreement indicates should imply the one with remaining time larger than the largest remaining time threshold among triggering and reporting thresholds. Also, the data belonging to low importance PDU set that has not been reported as delay-critical data when PSI-based SDU discard is activated, should also be included in the scope of the non-delay critical data that the agreement indicates.

Proposal 5. RAN2 to confirm that the non-delay critical data ahead of delay critical data that is agreed to be included in the buffer size calculation for enhanced DSR, includes

- the data with remaining time larger than the largest remaining time threshold among reporting and triggering thresholds, and

- the data belonging to low importance PDU set when PSI-based SDU discard is activated.

Also, how to report the non-delay-critical data ahead of delay-critical data should be discussed.

For example, if there are multiple remaining time portions that are reported for the associated LCG, we should decide which remaining time portion the size of the non-delay critical data should be added to. Among the remaining time portions with delay-critical data, considerable options are, adding it to 1) the remaining time portion with shortest remaining time threshold, 2) the remaining time portion with largest remaining time threshold, 3) the remaining time portion of the nearest delay-critical data after it, from resource allocation/buffering point of view, or 4) a dedicated remaining time portion for non-delay critical data per LCG.

Proposal 6. RAN2 to discuss, among the remaining time portions with delay-critical data, which remaining time portion, the size of the non-delay critical data should be added to, considering the options:

- Option 6-1: the remaining time portion with shortest remaining time threshold/value

- Option 6-2: the remaining time portion with largest remaining time threshold/value

- Option 6-3: the remaining time portion of the nearest delay-critical data in the buffer

- Option 6-4: a dedicated remaining time portion for non-delay critical data per LCG

Last, the meaning of ‘ahead of’ in the agreement should be further clarified.

Considering the motivation of such operation is to let NW allocate proper UL resource to cover both the delay critical data and non-delay critical data ahead of it, here, ‘ahead of’ should be considered from the perspective of UL resource allocation. However, the non-delay critical data can be ‘ahead of’ delay critical data within the same LCH or between different LCHs when allocating UL resource. So, RAN2 should discuss whether the ‘ahead of’ is applied within the same LCH only or inclusively across different LCHs.

Proposal 7. RAN2 to discuss whether the condition of the ‘non-delay critical data ahead of delay critical’ is applicable within the same LCH only or also between different LCHs inclusively.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 to discuss whether triggering threshold is used as reporting threshold for enhanced DSR reporting.

Proposal 2. RAN2 to consider the MAC CE format for enhanced DSR, where Extension (E) bit indicates whether further delay information of remaining time portion is present for the same LCG or not.

Proposal 3. RAN2 to consider the MAC CE format for enhanced DSR, where the field set placement is based on 1) ascending order of the value of the remaining time field within LCG, and 2) ascending order of the LCGi among LCGs.

Proposal 4. RAN2 to postpone the discussion on DSR format selection between legacy DSR and enhanced DSR, until the MAC CE format of enhanced DSR is determined.

Proposal 5. RAN2 to confirm that the non-delay critical data ahead of delay critical data that is agreed to be included in the buffer size calculation for enhanced DSR, includes

- the data with remaining time larger than the largest remaining time threshold among reporting and triggering thresholds, and

- the data belonging to low importance PDU set when PSI-based SDU discard is activated.

Proposal 6. RAN2 to discuss, among the remaining time portions with delay-critical data, which remaining time portion, the size of the non-delay critical data should be added to, considering the options:

- Option 6-1: the remaining time portion with shortest remaining time threshold/value

- Option 6-2: the remaining time portion with largest remaining time threshold/value

- Option 6-3: the remaining time portion of the nearest delay-critical data in the buffer

- Option 6-4: a dedicated remaining time portion for non-delay critical data per LCG

Proposal 7. RAN2 to discuss whether the condition of the ‘non-delay critical data ahead of delay critical’ is applicable within the same LCH only, or also between different LCHs inclusively.