3GPP TSG-RAN WG2 Meeting #129	R2-2501223

Athens, Greece, Feb. 17th – 21st, 2025

Source:        ZTE Corporation

Title:           Consideration on the UEIBM

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

In the last meeting, the UEIBM topic was touched, but the companies’ views are divergent, in this chapter we share our views on this topic based on the latest RAN1 and RAN2 progress.

- Discussion

In this chapter, we first discuss the UEIBM impact on MAC layer, then give our preliminary thinking on the RRC parameters structure, at last we share our understanding on how to handle LTM and UEIBM.

2.1 UEIBM Impact on MAC Layer

For the UEIBM, RAN1 has discussed the Events definition (e.g. Event 1, 2 and 7), event trigger condition, UEIBR (UE initiated Beam Report) procedure (e.g. mode A and mode B) and also other related topics. On the event trigger condition, a window would be introduced for the instance statistics. However, whether this mechanism is captured in RAN1 or RAN2 specification is still FFS though in the last meeting, RAN1 made some progress on the first PUCCH channel configuration as bellow.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

For the UEIBM reporting, if it is determined to be captured in the RAN2 spec, many detailed issues need to be further discussed, e.g. UEIBM Procedure description, timer management, interaction with the physical layer. Considering the time schedule, RAN2 can start discussion on the UEIBM impact on MAC layer procedure at the next meeting if there is still no conclusion from RAN1 in this meeting.

Proposal 1: RAN2 can start discussion on the UEIBM impact on the MAC layer procedure at the next meeting if there is still no conclusion from RAN1 in this meeting.

2.2 RRC Impact of the UE Initiated Beam Management (UEIBM)

The RRC impact is mainly from the RRC Parameters, according to the RAN1’s LS[1], some UEIBM related RRC parameters are listed as below:

Table 1: UEIBM RRC Parameters

| RAN2 Parent IE | Parameter name in the spec | Description | Value range |
| --- | --- | --- | --- |
| [CSI-ReportConfig] | reportTransmissionMode-r19 | This parameter is used to inform one report transmission mode… | {ModeA, ModeB} |
| CSI-ReportConfig | eventThreshold-r19 | This parameter is used to inform a threshold value for trigger event detection regarding Event-2. | FFS |
| CSI-ReportConfig | eventDetectionTimeWindowLength-r19 | This parameter is used to inform the time window for triggering event determination regarding Event-2 | FFS |
| CSI-ReportConfig | eventInstanceCount-r19 | This parameter is used to inform the number of event 2 instances for at least one same new beam within a configured time window that the UE can initiate UEIBM report | FFS |
| CSI-ReportConfig | enabledCurrentBeamReport-r19 | This parameter is to enable or disable whether current beam is always reported. | Up to RAN2 |
| CSI-ReportConfig | nrofReportedRS-UEIBR-r19 | This parameter is used to inform the number of reported RS(s) in a report format for UE-initated/event-driven beam reporting. | FFS |
| CSI-ReportConfig | newBeamResourceSetEvent2-r19 | This parameter is used to confiure the new beam resource set for Event 2 (Opt1) | FFS |
| CSI-ReportConfig | eventType-r19 | This parameter is used to inform a event type for UE-initated/event-driven beam reporting.- Event-1/2/7 | {Event1, Event2, Event7} |
| CSI-ReportConfig | valueOfQ | This parameter is used to inform the value of Q for Event7 | FFS |
| CSI-ReportConfig | configuredResourceForSecondChannelOfModeB | This parameter is used to inform Type-1 CG PUSCH resource for second channel in mode-B | configuredGrantConfigIndex |
| CSI-ReportConfig | firstPUCCHResourceConfig-UEIBR-r19 | This parameter is used to configure the periodic PUCCH resource for first PUCCH for both mode-A and mode-B. | periodicityAndOffset, PUCCH-ResourceID |
| CSI-ReportConfig | eventThresholdEvent1-r19 | This parameter is used to inform a threshold value for trigger event detection regarding Event-1. | configuredGrantConfigIndex, UL bwp-Id, serving cell index |

Based on these listed parameters, RAN2 need to discuss the below 2 high level questions:

Issue 1: Should the new IE for the UEIBM configuration be added under CSI-ReportConfig or under CSI-MeasConfig

According to the RAN1’s parameter list, it seems that the new IE for the UEIBM configuration should be added under the CSI-ReportConfig. However, the UEIBM configuration is quite different from the current elements in the CSI-ReportConfig (as shown in the Fig1). Furthermore, in the current CSI-ReportConfig, many parameters are mandatory present, which is even not used in the UEIBM configuration.

Fig 1: Framework of the Legacy CSI-ReportConfig

Thus, similar to the current LTM-CSI-ReportConfig, it’s better to add UEIBM configuration under the CSI-MeasConfig.

Fig 2: UEIBM Report Config

Proposal 2: Add UEIBM configuration under the CSI-MeasConfig.

Issue 2: How to associate the new beams with the UEIBM report for each BWP

Regarding the new beams, RAN1 has made many agreements for the Event 2, and the Event 7 may follow the same logic.

R1#118Agreement =>Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration =>If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. =>FFS: The RS in the RS resource set can be updated by MAC-CE

For the legacy CSI resource configuration, as shown in the Fig1, a CSI-ResourceConfigID was indicated for each CSI-ReportConfig. In each CSI-ResourceConfig, csi-RS-ResourceSetList and bwp-id are included. For the UEIBM, the new beams would be configured for each Event (e.g. Event 2/7) on each BWP, thus we also need to consider how to associate the new beams with the UEIBM report for each BWP. Generally, there are 2 options:

- Option 1: Reuse the existing CSI-ResourceConfig structure, then the new beams can be indicated by a csi-ResouceConfigId;

- Option 2: Indicate the new beams by a bwp-Id and new beams resource set Index immediately

Fig 3: Framework for the New Beams Configuration

The option 1 is quite simple, but the drawback is that some parameters are not useful for the UEIBM, e.g. resourceType. The option 2 can save the signaling overhead, but requires RAN2 to design new signaling structure.

Proposal 3: RAN 2 to discuss the below two options on how to associate the new beams with the UEIBM report for each BWP:

- Option 1: Reuse the existing CSI-ResourceConfig structure, then the new beams can be indicated by a csi-ResouceConfigId;

- Option 2: Indicate the new beams by a bwp-Id and new beams resource set Index immediately for each UEIBM Report Config.

Based on the proposal 2 and 3, we’d like to further discuss the general structure of the UEIBM configuration. First, the parameters in Table 1 can be divided into 2 types:

- Type 1: eventType-r19 and Event specific parameters:

- Event 2: eventThreshold-r19, enabledCurrentBeamReport-r19, nrofReportedRS-UEIBR-r19, newBeamResourceSetEvent2-r19, eventDetectionTimeWindowLength-r19eventInstanceCount-r19

- Event 7: valueOfQ, and may also include the parameters currently defined for the Event 2

- Event 1: eventThresholdEvent1-r19

- Type 2: Procedure specific parameters: reportTransmissionMode-r19, configuredResourceForSecondChannelOfModeB, firstPUCCHResourceConfig-UEIBR-r19

For the Type 1 parameters, the structure as in the Fig4 can be taken as baseline. In this structure whether the choice or the sequence structure should be used for the UEIBM-ReportEventType depends on whether multi-event would be supported in one report config. For the Event 7, the similar parameters to Event 2 maybe added based on RAN1’s further progress.

Fig 4: Framework of the UEIBM-CSI-ReportConfig

Proposal 4: Take Fig 4 as the start point for the UEIBM-CSI-ReportConfig design.

For the Type 2 parameters, it’s not clear to RAN2 whether these procedure related parameters should be configured per cell level or per UEIBM report config level. Thus, an LS can be sent to the RAN1 for confirmation.

Proposal 5: For the Type 2 parameters (e.g. reportTransmissionMode-r19, configuredResourceForSecondChannelOfModeB, firstPUCCHResourceConfig-UEIBR-r19), an LS can be sent to the RAN1 to confirm whether these parameters should be configured per serving cell level instead of per UEIBM report config level.

2.3 How to handle LTM and UEIBM

In the last meeting, another issue is about how to handle the LTM and UEIBM. Because of the similarity of these two topics, if the same framework (same MAC procedure and/or RRC configuration structure) is used, it can reduce the specification complexity. However, according to the current progress of each topic, at least it’s premature to harmonize these two topics with one framework. At this stage, the two topics can be discussed independently, and when pictures of both 2 topics are clear, we can further discuss which parts can be unified.

Proposal 6: The LTM and UEIBM should be discussed independently at this stage, it can be further discussed which parts can be unified when the pictures for both topics are clear.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 can start discussion on the UEIBM impact on the MAC layer procedure at the next meeting if there is still no conclusion from RAN1 in this meeting.

Proposal 2: Add UEIBM configuration under the CSI-MeasConfig.

Proposal 3: RAN 2 to discuss the below two options on how to associate the new beams with the UEIBM report for each BWP:

- Option 1: Reuse the existing CSI-ResourceConfig structure, then the new beams can be indicated by a csi-ResouceConfigId;

- Option 2: Indicate the new beams by a bwp-Id and new beams resource set Index immediately for each UEIBM Report Config.

Proposal 4: Take Fig 4 as the start point for the UEIBM-CSI-ReportConfig design.

Proposal 5: For the Type 2 parameters (e.g. reportTransmissionMode-r19, configuredResourceForSecondChannelOfModeB, firstPUCCHResourceConfig-UEIBR-r19), an LS can be sent to the RAN1 to confirm whether these parameters should be configured per serving cell level instead of per UEIBM report config level.

Proposal 6: The LTM and UEIBM should be discussed independently at this stage, it can be further discussed which parts can be unified when the pictures for both topics are clear.

- Reference

- R1-2410758  LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 RAN1