3GPP TSG-RAN WG2 #129	R2-2501319

Athens, Greece, 17th – 21st Feb. 2025

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In RAN#106, the WID for “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was updated [1], however, the UL capacity objective and justification is the same as before.

The main objective related to support of capacity enhancements for uplink have two sub-objectives:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

And the justification for these objectives is:

Need for Uplink capacity enhancement: NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified. Therefore, in order to unlock the additional UL capacity potential, there is a need to identify methods to de-couple the UL from the DL as much as possible. Apart from enhancements above, reducing the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction could be beneficial, therefore should be studied and if beneficial, specified.

In this contribution we look further into this objective.

# 2	Main characteristics and Terminology

The main characteristics of UL capacity enhancements will likely be:

- UEs in RRC CONNECTED, IDLE mode (or IDLE/suspended mode) with a fresh GNSS measurement may use this new functionality, for example, based on information received in broadcasting, UE implementation, and UE situation at the moment (for example UE buffer size)

- A first message UE -> eNB is sent on a contention-based resource, this message is likely to be similar to msg3 in EDT

- A reply message eNB -> UE is sent to one UE  on the regular (N)PDSCH (possibly replies to many UEs can be combined for saving PDCCH resources), this message is likely to be similar to MsgB for 2-step RA in NR

  - The UEs can expect a reply message earliest UE-eNB RTT after the end of the message transmission UE -> eNB

- UEs that do not get a reply message within a reasonable time will (after random backoff?) need to retransmit

- UEs that fail too many times most likely shall stop using the contention based channel

- There must be a mechanism to control the load on the contention-based channel, for example some kind of broadcasted back off

During RAN2#127bis, there was a statement captured in the chair notes:

We use the term CB Msg3 EDT for now

However, we believe this term is too long. Therefore, we propose to give it a name that is shorter and easier to write – this will make it easier to refer to the functionality in the spec.

As the first message is similar to MsgA in 2-step RA in NR (but without the preamble part) and especially the second message (the reply from eNB) will be similar to MsgB in 2-step RA in NR. We suggest using similar names but with prefix CB-:

RAN2 to discuss the terms to be used to describe the EDT improvements in Rel-19, for example the new functionality can be called “contention-based early data transmission” (CB EDT).

RAN2 to discuss the terms to be used for the first message from UE to eNB in the EDT improvements in Rel-19, it may for example be called “contention-based message A” or CB MsgA.

RAN2 to discuss the terms to be used for the reply to CB MsgA from eNB to UE in the EDT improvements in Rel-19, it may for example be called “contention-based message B” or CB MsgB.

This terminology is shorter than “CB msg3 EDT” and will make it simple and clear to distinguish the new functionality from legacy when we write the specification later.

# 3	Contention based EDT

## 3.1	Example of one DSA and CB MsgB window

To explore the possibilities for the DSA window and the CB MsgB, we start with one possible configuration for a DSA window and a CB MsgB window.

We assume a 3.75 kHz SCS GEO scenario with 544 ms RTT and with good coverage, that is, no repetitions and each transmission use one RU in uplink respectively one subframe in downlink. Each RU is then 32 ms long.

The DSA window is 12 subcarriers, and 8 RUs in time, in total 96 transmission opportunities, but not all of them can be selected at any time for one UE (to transmit CB MsgA on two different subcarriers at the same time may be costly to implement for a UE as power amplifier linearity and peak power will increase, see “A.2	Restriction on selectable DSA resources”).

The DSA window is then 8*32 ms = 256 ms long.

For the timing of the CB MsgB, we assume the same restrictions as for legacy scheduling (for more info see A.3	Legacy NB-IoT resources). That is, a minimum of 3 ms, for eNB processing and switching time of UE from UL tx to DL rx, between last NPUSCH subframe and the first NPDCCH subframe, a minimum gap of four subframes between the last NPDCCH subframe and the first NPDSCH subframe, and a minimum gap of 12 subframes between the last NPDSCH subframe and the first NPUSCH Format 2 subframe (the HARQ ACK signalling). The NPUSCH Format 2 can also be replaced by another NPDCCH monitoring occasion.

A UEs can expect a CB MsgB message earliest UE-eNB RTT (the TA plus Kmac) after the end of a CB MsgA, but here we assume the UE transmit two replicas in the DSA window and then wait until the end of the DSA window before switching to NPDCCH monitoring for CB MsgB.

To summarize all information about the Figure 1:

- The x-axis is proportional in time

- 3.75 kHz SCS

- One RU is 32 ms on a single subcarrier

- UL transmissions are illustrated at the UE

- DL transmissions are illustrated at the eNB

- UL subframes not available for UL transmission are ignored

- DL subframes not available for DL transmission are ignored

- The HARQ ACK/NACK uses 8 ms

  - The NPUSCH resources for HARQ ACK/NACK are illustrated as a window of 8*8 ms on subcarrier 0 to 11 while the actual resources for HARQ ACK/NACK are on subcarriers 38 to 45, further we can only signal the ACK/NACK to be either 13 or 21 subframes after the last NPDSCH subframe for 3.75 kHz SCS, thus a real window can only be 2*8 ms

- DSA window uses 12 subcarriers and 8 RUs in time

  - DSA window is 12*8 ms = 256 ms long

- The DSA window Periodicity is just arbitrarily selected to be 256+544+21+64 ms = 885 ms

  - it may be longer (for example if DL subframes are excluded, or if UL subframes are excluded)

  - it may be shorter (for example if we make the ACK/NACK window shorter)

- CB MsgA uses one RU without repetitions and there is two replicas (in blue colour)

- CB MsgB has same timings as in legacy scheduling of DL

  - 3 ms gap for eNB and UE processing between last NPUSCH subframe and the first NPDCCH subframe

  - 4 ms gap for eNB and UE processing between last NPDCCH subframe and the first NPDSCH subframe (plus the scheduling delay k0 = 0 in DCI)

  - 12 ms gap for UE processing after the last NPDSCH subframe and the first NPUSCH Format 2 subframe (the HARQ ACK signalling)

- the UEs transmit two CB MsgA replicas in the DSA window and then wait until the DSA window end plus UE-eNB RTT before switching to NPDCCH monitoring

- Note that the effect Koffset is not shown (that is, figure assumes a perfect Koffset = TA), if Koffset is included likely HARQ A/N is a little delayed (especially for UEs that have lower RTT than the maximum RTT in the cell)

- Note that Kmac is assumed to be 0

Figure 1: Example of DSA window, CB MsgB and HARQ A/N, no repetitions

In the next figure (Figure 2) we have the same assumptions as in Figure 1, but we have assumed the maximum number of repetitions are used for each transmission (128 for UL and 2048 for DL) and we ignore the 40 ms silent period every 256 ms of UL transmission.

To summarize all information about the Figure 2:

- Same assumptions as Figure 1 except:

  - 128 repetitions for each UL transmission

  - 2048 repetitions for each DL transmission

  - We ignore that the UE needs 40 ms silent periods every 256 ms of UL transmission

Figure 2: Example of DSA window, CB MsgB and HARQ A/N, maximum repetitions

The transparent payload maximum propagation RTT (see 38.821 table 4.2-2) is about 542 ms in GEO, about 42 ms in 1200 km LEO, and about 26 ms in 600 km LEO.

The eNB do not know the UEs individual RTT, it needs to assume a UE is at the maximum RTT and all UEs will need to use the cell specific Koffset for the CB EDT functionality.

The minimum size of the CB MsgB window, as seen from the UE, is the propagation RTT + 21 + 8 ms (with one NPDSCH subframe, without repetitions) and RTT + 4115 + 1024 ms (with one NPDSCH subframe, with maximum repetitions).

Each CB MsgB NPDCCH search space can contain several occasions to schedule a CB MsgB.

However, one method to address multiple UEs in the same CB MsgB window if multiple RNTIs are used (in some way the UEs within a DSA window choose different RNTIs to listen for in the CB MsgB window) is to have a NPDCCH search space and send multiple DCIs for multiple CB MsgB (and use k0 to separate the corresponding NPDSCH messages in time). This method has one big drawback, as the eNB do not know when the UE sent other replicas and the UE do not know which replica that will be successful – the UEs must  search for multiple RNTIs, but eNB may reply to any of the RNTIs thus the UE must decode all RNTIs that the replicas indicate and therefore must decode every CB MsgB message and look for the UE IDs to find if it was addressed in that CB MsgB or not. Therefore, the CB MsgBs cannot overlap in time in one search space for NPDCCH, and every CB MsgB that is scheduled may need to be decoded by the UEs to know whether the correct UE ID is in it.

The UEs likely must decode every CB MsgB even if each CB MsgA use a different RNTIs because UE do not know which RNTI the eNB will reply to, when multiple RNTIs are used.

If the UE is addressed in the DCI of a CB MsgB (the DCI is scrambled with the RNTI the UE is listening for) but if the CB MsgB content do not match the UE ID of the UE, then the UE can start monitoring NPDCCH after the 12 ms gap after the last NPDSCH subframe because it will never need to send a HARQ A/N if it was not addressed and any successful UEs do not need to monitor NPDCCH for another CB MsgB (successful UEs can send the HARQ ACK/NACK at the same time as other UEs start monitoring NPDCCH for another CB MsgB).

To make sure many UEs can be addressed in a CB MsgB window and to enable eNB flexibility in scheduling, we can extend the CB MsgB window with multiple CB MsgB transmissions opportunities from the minimum 21 + 8 ms to 21 + (21 – 3) + 8 ms (with one NPDSCH subframe, without repetitions) and from the minimum 4115 + 1024 ms to 4115 + (4115 – 3) + 1024 ms (with one NPDSCH subframe, with maximum repetitions). Note that the three subframes needed before the first NPDCCH occasion is not needed in front of the following NPDCCH subframes because the eNB has already had time to prepare the transmission after the NPUSCH transmissions, and the UEs have already switched to DL monitoring in the first NPDCCH occasion.

One CB MsgB window can allow for multiple CB MsgB transmissions consecutive in time.

## 3.2	End of DSA window and start of CB MsgB window

Here we discuss where the CB MsgB window(s) can be in relation to the DSA window.

The CB MsgB window at the UEs can be placed in different ways, for example A) UE-eNB RTT after each CB MsgA transmission (that is every CB MsgA opportunity there is a potential CB MsgB), B) UE-eNB RTT after a group of CB MsgA opportunities (that is, we place some CB MsgB windows in between the CB MsgA opportunities inside the DSA window), C) UE-eNB RTT after the end of the DSA window (that is only one CB MsgB window after the DSA window).

Both option A and B will require the DSA window to extend longer in time to reach the same collision probability – with the advantage that some UEs will successfully end their CB EDT procedure early (some energy gain and some delay gain for that UE). It is quite difficult to investigate whether the possible lower delay of ending early can compensate that the whole DSA window is longer which will mean higher delay for UEs that fail their early replica attempts.

Option A and B has the drawback that a UE will not be free to choose any CB MsgA transmission opportunities within a DSA window. This increase collision probability (see A.2	Restriction on selectable DSA resources), or we must allocate even more resources to the DSA window. Further, A and B has the risk that the eNB has few CB MsgA to answer in any one CB MsgB occasion.

A UE individual CB MsgB window, after each UEs CB MsgA replica transmission or after a few CB MsgA opportunities, risk less opportunities for the eNB to answer multiple CB MsgA in one CB MsgB and will make the DSA window longer in time.

Option C has the disadvantages that the UEs cannot end the replica transmissions early and may have higher delay (adding CB MsgB opportunities within the DSA window will extend DSA window and add delay for some UEs – not clear what the compound effect is). However, there is maximum opportunity for eNB to answer multiple CB MsgA in the CB MsgB window.

Having only one CB MsgB window that start after the end of the DSA window ensures plenty of CB MsgA to answer in one CB MsgB and gives an even load on the CB MsgA opportunities.

Considering the ease of option C and the drawbacks of A and B, we propose:

There is one CB MsgB window per DSA window and the CB MsgB window start immediately after the DSA window, taking UE-eNB RTT into account.

In legacy, the UE is allowed 3 ms processing time after NPUSCH transmission to switch from tx to rx (in NTN that is not needed as the RTT is added to the time before a reply can be expected), these 3 ms also allow the eNB some processing time for preparing a new grant/assignment based on the received NPUSCH.

In last meeting, RAN2 made this agreement:

7.	For SA case (single replica), after the end of all repetition of CB Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

For legacy, this is captured in 16.6 of 36.213 (see A.3.3	NPDCCH resources). For both DSA and SA, we can capture the NPDCCH monitoring start in one proposal:

If a DSA window NPUSCH transmission end in uplink subframe n or a SA NPUSCH transmission end in uplink subframe n, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3. The UE shall start NPDCCH monitoring in the first NPDCCH subframe, after uplink subframe n+Kmac+3, that do not overlap uplink subframe n+Kmac+3.

If the UEs are allowed to start another CB EDT procedure while one CB EDT procedure is ongoing, we get messy error cases to handle. To ease the detection of duplicates in the eNB and avoid complicated error cases if there is successful CB MsgA in two consecutive DSA windows from the same UE, we propose:

The UE shall at most have one ongoing CB EDT procedure at any time.

## 3.3	DSA window size

Sending on multiple subcarriers at the same time will require higher UE peak power and a more linear power amplifier (see A.2	Restriction on selectable DSA resources), it is therefore beneficial to spread the DSA window in time instead of in frequency.

For easy and flexible configuration, we believe that the NW shall be able to configure several CB MsgA transmission opportunities consecutive in time and in frequency, together with a DSA window periodicity.

The CB EDT configuration has an indication of uplink subcarrier spacing.

The CB EDT configuration has an indication of start and end subcarrier, or start and number of subcarriers, per CE level.

The CB EDT configuration has a MCS index per CE level, indicating TBS, modulation, and number of RUs for CB MsgA.

The CB EDT configuration has the number of CB MsgA transmission opportunities in time per CE level, that makes up one DSA window.

The CD EDT configuration has, the number of repetitions of CB MsgA per CE level.

The DSA window has a start time relative the H-SFN per CE level.

The DSA window has a DSA window periodicity per CE level.

RAN2 to discuss how to handle H-SFN wrap around if periodicity do not evenly divide the H-SFN period.

With the proposals above, we have the DSA window specified in frequency (the subcarriers) and in time ([nrof CB MsgA opportunities] * [nr of repetitions] * [nr of RUs] * [time for one RU] plus possible excluded UL subframes and possible 40 ms silent gaps).

## 3.4	Mobile terminated EDT

The mobile terminated EDT (MT-EDT) is described in 36.300 section 7.3c.

The main characteristic of MT-EDT is that the msg4 (in random access) can contain downlink data for the UE, and that the termination of the procedure has this: “The procedure ends with the reception of the HARQ feedback (ARQ) acknowledging the successful DL transmission.”.

To support MT-EDT on the CB EDT channels with as small changes as possible compared to legacy, we need 1) msg4 can contain MAC SDUs, and 2) each msg4 can be acknowledged by HARQ ACK.

MT-EDT is a part of EDT that would extend the use cases for the new CB EDT feature and MT-EDT will benefit from the increased uplink capacity and decreased delay of CB EDT. Therefore, we propose:

Mobile terminated EDT is supported.

The CB MsgB can contain MAC SDUs carrying downlink data for each CB MsgA that is acknowledged.

For each UE in a CB MsgB, the successful reception of a CB MsgB can be individually acknowledged with a HARQ ACK transmission.

## 3.5	HARQ ACK for successful CB MsgB

Random access msg4 (and other scheduled data on NPDSCH) is acknowledged on resources that are indicated using 4 bits in the DCI that schedules the msg4.

To enable each UE to acknowledge the successful reception of a CB MsgB we need to allocate one HARQ ACK resources per UE that is addressed in one CB MsgB. Simplest and least change compared to legacy is to reuse the same HARQ ACK/NACK resource allocation as in the DCI.

The CB MsgB has a 4 bit HARQ ACK resource allocation, reusing the existing HARQ ACK/NACK allocation signalling in the DCI, per UE that is replied to.

This means for 3.75 kHz SCS, the ACK/NACKs can only use the same eight subcarriers (38 to 45) and the two k0 (13 and 21) as legacy. For 15 kHz SCS, the ACK/NACKs can only use the same four subcarriers (0 to 3) and the four k0 (13, 15, 17 and 18) as legacy.

To enable to not send HARQ feedback, we note the method for dynamic HARQ feedback control, from 36.212 section 6.4.3.2:

-	HARQ-ACK resource – 4 bits as defined in clause 16.4.2 of [3]. If downlinkHARQ-FeedbackDisabledDCI-NB is configured, or if downlinkHARQ-FeedbackDisabledBitmap-NB and downlinkHARQ-FeedbackDisabledDCI-NB are configured, and the value is ‘15’, it functions as a HARQ feedback disabled indicator.

In the HARQ-ACK resources for CB MsgB, the value ‘15’ indicate HARQ feedback disabled.

## 3.6	Number of UEs that can be answered in one CB MsgB

For 3.75 kHz SCS, if there is no need for repetitions, the legacy HARQ ACK allocation can support up to sixteen UEs simultaneously (eight subcarriers and two different k0, as the NPUSCH Format 2 is 8 ms, see section A.3.1	NPUSCH resources below) but if repetitions are needed only maximum eight different UEs can be supported with HARQ ACK resources simultaneously.

For 15 kHz SCS, if there is no need for repetitions, the legacy HARQ ACK allocation can support up to twelve UEs simultaneously (four subcarriers and three different k0 that are non-overlapping, as the NPUSCH Format 2 is 2 ms, see section A.3.1	NPUSCH resources below) but if more than two repetitions are needed only maximum four different UEs can be supported simultaneously.

For 3.75 kHz SCS, the legacy HARQ ACK resource allocation can support up to eight UEs simultaneously when repetitions are needed.

For 15 kHz SCS, the legacy HARQ ACK resource allocation can support up to four UEs simultaneously when more than two repetitions are needed.

The multiplexing of many UEs in one CB MsgB will increase the needed TBS, and thus may increase the number of subframes and/or the number of repetitions needed.

Considering the multiplexing of many UEs in CB MsgB and the drawback of large TBS and the legacy allocation of HARQ ACK resources, we think a reasonable trade-off is to multiplex up to four UEs in one CB MsgB. However, this may depend on many factors and the exact number can be left to the eNB implementation.

The number of CB MsgA replies in one CB MsgB can be left to eNB implementation.

## 3.7	CB MsgB window and the RNTIs for MsgA and MsgB

Considering the low load at the agreed packet loss ratio of 10%, it is reasonable to assume not too many CB MsgA needs to be replied to in one CB MsgB window. Further, if the eNB cannot reply to all successful CB MsgA in a CB MsgB window, the unsuccessful UEs will retransmit later.

To enable saving NPDCCH resources (the WID objective) by answering multiple CB MsgA in one CB MsgB, the RNTIs to be used for identifying whether a CB MsgB may contain a reply for a certain CB MsgA or not will affect whether the eNB can answer multiple CB MsgA in one CB MsgB.

If the number of UEs to reply to is higher than the capability of one CB MsgB, we can have multiple NPDCCH occasions in one CB MsgB window.

For maximum possibility for eNB to answer multiple CB MsgA in one CB MsgB is if all UEs uses the same RNTI for CB MsgB (and the same RNTI can be used for all CB MsgA), which will be possible for the low loads considered.

There is one RNTI for CB MsgA and CB MsgB transmissions.

This is simple for the UEs as they do not need to monitor multiple RNTIs. If the expected load on one CB MsgB is high, we can have multiple CB MsgB opportunities within one CB MsgB window.

The CB EDT functionality is supposed to work for UEs in RRC CONNECTED, IDLE or IDLE/suspended mode and eNB cannot know what UE that is accessing, all UEs shall use the cell specific Koffset.

The UE shall use the cell specific Koffset for the CB EDT feature, regardless of if it has a UE specific Koffset or not.

As UEs in IDLE shall be able to use this functionality, it is reasonable to assume that the CB MsgB must be scheduled in the same way as a RAR is scheduled, that is only using NPDCCH format 1 (using the two NCCEs for one DCI, for better coverage and easier NPDCCH decoding for the UEs):

The scheduling of CB MsgB uses NPDCCH format 1.

As the nr of repetitions for NPDSCH is included in the DCI for CB MsgB and all DL subframes are not available for NPDCCH or NPDSCH, each CB MsgB transmission opportunity inside a CB MsgB window can vary in total number of subframes (when excluded ones are counted).

However, the location of potential NPDCCH occasions inside a CB MsgB window needs to be clear to the UEs. If the DSA window and the start of the CB MsgB window are not always at the same location in the resource grid relative any excluded subframes, different CB MsgB windows will have different NPDCCH start subframes relative the end of the DSA window.

To have multiple CB MsgB opportunities in a CB MsgB window, we can have a first NPDCCH occasion in a CB MsgB window as the first possible non-excluded DL subframe after what is specified in Proposal 5 and then specify the number of repetitions for NPDCCH and guess on the needed subframes and repetitions for NPDSCH, and then assume one NPDCCH occasion after each period until the end of the CB MsgB window. For example, second NPDCCH monitoring subframe is the first NPDCCH subframe 21 ms after first NPDCCH monitoring subframe, not counting excludes subframes, and third NPDCCH subframe starts 21 – 3 ms after second NPDCCH occasion etc. we may also add some extra search space subframes for the eNB to have flexibility in the scheduling.

To enable the NW to adjust the CB MsgB capacity according to design target and expected load, we propose:

The CD EDT configuration has an NPDCCH repetition number per CE level for the CB MsgB window.

The CD EDT configuration has an NPDCCH occasion periodicity per CE level for CB MsgB window.

The CD EDT configuration has a CB MsgB window size per CE level.

The UEs shall monitor NPDCCH occasions inside the CB MsgB window unless it has successfully decoded a CB MsgB containing a matching Contention Resolution Identity.

## 3.8	Design of CB MsgB

The second WI sub-objective of this topic is the efficient delivery (reduced overhead) of Msg4 (CB MsgB). In RAN2#126 meeting, [2] showed how the overhead reduction cannot come from optimizations in the RRCEarlyDataComplete ASN.1 structure. Thus, RAN2 should focus on different alternatives such as the transmission of a shared CB MsgB .

If multiple UEs can be acknowledged in one CB MsgB, the delay decreases and we will use much less (N)PDCCH resources.

In NR 2-step random access, the 2-step MsgB can consist of a Backoff Indicator and one or multiple successRAR and/or fallbackRAR sub-PDUs. Each successRAR consists of UE Contention Resolution Identity, and some more fields. Hence, a MsgB can be used to respond to multiple UEs which have transmitted MsgA, i.e., this is a shared MsgB.

NR’s 2-step random access support multiplexing of multiple UE Contention Resolution Identities in MsgB.

In NR 2-step RA we have (from NR TS 38.321):

Figure 6.1.5a-4: Example of a MSGB MAC PDU with MAC SDU(s)

Figure 6.1.5a-5: Example of a MSGB MAC PDU without MAC SDU(s)

In NR 2-step RA, the NR successRAR looks like this (from NR MAC)

Figure 6.2.3a-2: successRAR

A similar design can be applied to CB MsgB, i.e., CB MsgB can consist of a Backoff Indicator (to get fast control of the load on the CB MsgA resources) and one or multiple contention resolution identities.

The contention resolution Identity can reuse the “UE Contention Resolution Identity MAC Control Element” that is derived from the CCCH (that is from RRCConnectionResumeRequest or RRCEarlyDataRequest depending on if UP-EDT or CP-EDT is configured for CBDT or a new RRC message if that is decied).

This avoids the transmission of separate CB MsgB for each individual UE whose CB MsgA transmission is decoded successfully.

Introduce multiplexing of several UE Contention Resolution Identities in CB MsgB using NR MSGB as baseline.

Introduce a Backoff Indicator in CB MsgB.

In legacy CP-EDT, msg3 will have a UE identity in the RRC message:

RRCEarlyDataRequest-r15-IEs ::=     SEQUENCE {

s-TMSI-r15                                S-TMSI,

…

For UP-EDT

RRCConnectionResumeRequest-r13-IEs ::=          SEQUENCE {

resumeIdentity-r13                                                CHOICE {

resumeID-r13                                                      ResumeIdentity-r13,

Then in CB MsgB, the UE can repeat part of the RRC message in a UE Contention Resolution Identity MAC Control Element used for the contention resolution (from LTE MAC):

6.1.3.4		UE Contention Resolution Identity MAC Control Element

The UE Contention Resolution Identity MAC control element is identified by MAC PDU subheader with LCID as specified in table 6.2.1-1. This control element has a fixed 48-bit size and consists of a single field defined as follows (figure 6.1.3.4-1)

-    UE Contention Resolution Identity: If this MAC control element is included in response to an uplink CCCH transmission, then this field contains the uplink CCCH SDU if the uplink CCCH SDU is 48 bits long. If the CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the uplink CCCH SDU. If this MAC control element is included in response to an uplink DCCH transmission (i.e. the MAC entity is configured with rach-Skip or rach-SkipSCG), then the MAC entity shall ignore the contents of this field.

Figure 6.1.3.4-1: UE Contention Resolution Identity MAC control element

The CB MsgB contention resolution uses legacy UE Contention Resolution Identity MAC Control Element for the contention resolution.

## 3.9	Multi carrier DSA

It is easy to support multicarrier by including an indication in system info on what carrier the CB EDT is configured.

The CB EDT configuration indicates which carrier is used for the CB EDT.

RAN2 to discuss whether one carrier per CE level can be configured, or if all CE levels are supported on the same carrier.

## 3.10	Contention control

When CB EDT is applied, the UE shall start another round of attempts for CB MsgA transmission (after waiting for a random back-off) only if all the replicas in the current round have been transmitted and none of them is acknowledged by the NW.

If no successful contention resolution was received in the CB MsgB window after a CB MsgA replica was transmitted, the UE shall draw a random number from a back off (indicated in a CB MsgB, if received, or from back off indicator in system information) and wait the number of DSA windows and in the DSA window after the wait, do the CB MsgA replica selection and transmission.

## 3.10	Contention mechanism: collision detection and backoff

CB MsgA transmission may fail due to collision or bad reception quality at the network side. In NR 2-step random access, when the transmission of MsgA fails, the UE may have another MsgA transmission attempt with a random backoff and an increased MsgA transmission power (i.e., power ramping). The UE would switch to 4-step random access if the number of MsgA transmissions has reached MsgA-TransMax, provided that preambleTransMax is greater than MsgA-TransMax. A similar mechanism can be introduced for CB EDT to handle CB MsgA transmission failures.

After a failure in CB MsgA transmission (no matching UE ID received in the CB MsgB), the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power. FFS parameters and details of the mechanism.

After a number of failed CB MsgA transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

## 3.11	Contention control

When CB EDT is applied, the UE shall start another round of attempts for CB MsgA transmission (after waiting for a random back-off) only if all the replicas in the current round have been transmitted and none of them is acknowledged by the NW.

If no successful contention resolution was received in the CB MsgB window after a CB MsgA replica was transmitted, the UE shall draw a random number from a back off (indicated in a CB MsgB, if received, or from back off indicator in system information) and wait the number of DSA windows and in the DSA window after the wait, do the CB MsgA replica selection and transmission.

## 3.12	OCCs

If only some UEs can handle OCCs it may not be useful for CB EDT functionality to have OCC transmissions enabled as a that will require separate frequency and time resources for those UEs (as transmission time is double for OCC2 compared to no OCC used). Further, OCCs only help if UEs has a CTO (as discussed in RAN1) that is similar and also that there are small power imbalances between them, as well as single tone transmissions (see LS [4]) – it is unlikely that UEs in IDLE/SUSPENDED will have similar CTO and that there are no power imbalance and that only single tone is supported for DSA.

Further, using OCCs with CRDSA will require the UE to use the same OCC used in all the replicas to enable cancelling it on other replica occasions. This may decrease the gain of OCC as it is a restriction to selectable DSA resources.

OCCs are not used for the CB EDT functionality.

## 3.13	Some eMTC considerations

All issues considered above needs to be investigated also for eMTC. One agreement from last meeting needs an update as eMTC can support full duplex UEs:

10.	Assuming that there will be scenarios where it’s possible to receive a CB Msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB Msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB Msg4)

A full duplex eMTC UE may receive a successful contention resolution CB MsgB while it is transmitting the repetitions of a replica, and therefore can stop the repetition transmissions.

An eMTC UE may stop transmitting CB MsgA replicas or CB MsgA repetitions of a replica if the UE receive a CB MsgB with a matching contention resolution identity.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The UEs likely must decode every CB MsgB even if each CB MsgA use a different RNTIs because UE do not know which RNTI the eNB will reply to, when multiple RNTIs are used.

Observation 2	One CB MsgB window can allow for multiple CB MsgB transmissions consecutive in time.

Observation 3	A UE individual CB MsgB window, after each UEs CB MsgA replica transmission or after a few CB MsgA opportunities, risk less opportunities for the eNB to answer multiple CB MsgA in one CB MsgB and will make the DSA window longer in time.

Observation 4	Having only one CB MsgB window that start after the end of the DSA window ensures plenty of CB MsgA to answer in one CB MsgB and gives an even load on the CB MsgA opportunities.

Observation 5	For 3.75 kHz SCS, the legacy HARQ ACK resource allocation can support up to eight UEs simultaneously when repetitions are needed.

Observation 6	For 15 kHz SCS, the legacy HARQ ACK resource allocation can support up to four UEs simultaneously when more than two repetitions are needed.

Observation 7	NR’s 2-step random access support multiplexing of multiple UE Contention Resolution Identities in MsgB.

Observation 8	In the 5G QoS model, there is an acceptable packet delay budget that 98% of the packets need to fulfill and a there is an acceptable packet loss rate.

Observation 9	There is a standardized 5QI 10 that targets NTNs. It has an RAN PDB of 1080 ms and a PER of 10^-6.

Observation 10	For 5QI 10, each transmission can have a failure rate of up to 14% without violating the QOS requirements in GEO scenarios.

Observation 11	Restrictions that limit the UEs ability to freely select transmission opportunities within a DSA window will increase the probability of collision.

Observation 12	When configuring resources for the DSA window, it is beneficial to allocate more resources in the time domain and less in frequency domain than vice versa.

Observation 13	For 3.75 kHz SCS, HARQ ACK/NACK transmission without repetitions use 8 ms on one subcarrier.

Observation 14	For 15 kHz SCS, HARQ ACK/NACK transmission without repetitions use 2 ms on one subcarrier.

Observation 15	For 3.75 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 32 ms on one subcarrier.

Observation 16	For 15 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 8 ms for one subcarrier, 4 ms for three subcarriers, 2 ms for six subcarriers and 1 ms for twelve subcarriers.

Observation 17	Increasing number of RUs can increase the UL transport block size.

Observation 18	The number of subframes for one NPUSCH transmission increase if more RUs or more repetitions are used.

Observation 19	Some of the contiguous subframes cannot be used for NPUSCH transmissions.

Observation 20	The NB-IoT UEs cannot transmit NPUSCH continuously for more than 256 ms before inserting a 40 ms silent period of postponed NPUSCH transmission subframes.

Observation 21	The NPDSCH transmissions uses 15 kHz SCS and 12 subcarriers.

Observation 22	Not all DL subframes are available for NPDSCH transmission, for example subframes with NPBCH, NPSS, NSSS, SIB1-NB and configured gaps are excluded.

Observation 23	Increasing number of subframes can increase the DL transport block size.

Observation 24	The number of subframes for one NPDSCH transmission increase if more subframes or more repetitions are used.

Observation 25	Some of the contiguous subframes cannot be used for NPDSCH transmissions.

Observation 26	The resource for HARQ ACK/NACK is indicated with 4 bits in the DCI.

Observation 27	For 3.75 kHz SCS, HARQ ACK resources can be on one of 8 different subcarriers and use one of 2 different scheduling delays.

Observation 28	For 15 kHz SCS, HARQ ACK resources can be on one of 4 different subcarriers and use one of 4 different scheduling delays.

Observation 29	There is a gap of at least 12 subframes between the the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK.

Observation 30	NPDCCH uses 6 or 12 subcarriers in one subframe (without repetitions) can carry two respectively one DCI.

Observation 31	There is a guard period of at least 3 subframes after an NPUSCH transmission (Format 1 or 2) and first possible NPDCCH reception.

Observation 32	There is a guard period of at least 12 subframes after an NPDSCH reception and the first possible NPDCCH reception.

Observation 33	Some of the contiguous subframes cannot be used for NPDCCH transmissions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the terms to be used to describe the EDT improvements in Rel-19, for example the new functionality can be called “contention-based early data transmission” (CB EDT).

Proposal 2	RAN2 to discuss the terms to be used for the first message from UE to eNB in the EDT improvements in Rel-19, it may for example be called “contention-based message A” or CB MsgA.

Proposal 3	RAN2 to discuss the terms to be used for the reply to CB MsgA from eNB to UE in the EDT improvements in Rel-19, it may for example be called “contention-based message B” or CB MsgB.

Proposal 4	There is one CB MsgB window per DSA window and the CB MsgB window start immediately after the DSA window, taking UE-eNB RTT into account.

Proposal 5	If a DSA window NPUSCH transmission end in uplink subframe n or a SA NPUSCH transmission end in uplink subframe n, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3. The UE shall start NPDCCH monitoring in the first NPDCCH subframe, after uplink subframe n+Kmac+3, that do not overlap uplink subframe n+Kmac+3.

Proposal 6	The UE shall at most have one ongoing CB EDT procedure at any time.

Proposal 7	The CB EDT configuration has an indication of uplink subcarrier spacing.

Proposal 8	The CB EDT configuration has an indication of start and end subcarrier, or start and number of subcarriers, per CE level.

Proposal 9	The CB EDT configuration has a MCS index per CE level, indicating TBS, modulation, and number of RUs for CB MsgA.

Proposal 10	The CB EDT configuration has the number of CB MsgA transmission opportunities in time per CE level, that makes up one DSA window.

Proposal 11	The CD EDT configuration has, the number of repetitions of CB MsgA per CE level.

Proposal 12	The DSA window has a start time relative the H-SFN per CE level.

Proposal 13	The DSA window has a DSA window periodicity per CE level.

Proposal 14	RAN2 to discuss how to handle H-SFN wrap around if periodicity do not evenly divide the H-SFN period.

Proposal 15	Mobile terminated EDT is supported.

Proposal 16	The CB MsgB can contain MAC SDUs carrying downlink data for each CB MsgA that is acknowledged.

Proposal 17	For each UE in a CB MsgB, the successful reception of a CB MsgB can be individually acknowledged with a HARQ ACK transmission.

Proposal 18	The CB MsgB has a 4 bit HARQ ACK resource allocation, reusing the existing HARQ ACK/NACK allocation signalling in the DCI, per UE that is replied to.

Proposal 19	In the HARQ-ACK resources for CB MsgB, the value ‘15’ indicate HARQ feedback disabled.

Proposal 20	The number of CB MsgA replies in one CB MsgB can be left to eNB implementation.

Proposal 21	There is one RNTI for CB MsgA and CB MsgB transmissions.

Proposal 22	The UE shall use the cell specific Koffset for the CB EDT feature, regardless of if it has a UE specific Koffset or not.

Proposal 23	The scheduling of CB MsgB uses NPDCCH format 1.

Proposal 24	The CD EDT configuration has an NPDCCH repetition number per CE level for the CB MsgB window.

Proposal 25	The CD EDT configuration has an NPDCCH occasion periodicity per CE level for CB MsgB window.

Proposal 26	The CD EDT configuration has a CB MsgB window size per CE level.

Proposal 27	The UEs shall monitor NPDCCH occasions inside the CB MsgB window unless it has successfully decoded a CB MsgB containing a matching Contention Resolution Identity.

Proposal 28	Introduce multiplexing of several UE Contention Resolution Identities in CB MsgB using NR MSGB as baseline.

Proposal 29	Introduce a Backoff Indicator in CB MsgB.

Proposal 30	The CB MsgB contention resolution uses legacy UE Contention Resolution Identity MAC Control Element for the contention resolution.

Proposal 31	The CB EDT configuration indicates which carrier is used for the CB EDT.

Proposal 32	RAN2 to discuss whether one carrier per CE level can be configured, or if all CE levels are supported on the same carrier.

Proposal 33	If no successful contention resolution was received in the CB MsgB window after a CB MsgA replica was transmitted, the UE shall draw a random number from a back off (indicated in a CB MsgB, if received, or from back off indicator in system information) ad wait the number of DSA windows and in the DSA window after the wait, do the CB MsgA replica selection and transmission.

Proposal 34	After a failure in CB MsgA transmission (no matching UE ID received in the CB MsgB), the UE may attempt another transmission after waiting for a random back-off and increasing the transmission power. FFS parameters and details of the mechanism.

Proposal 35	After a number of failed CB MsgA transmission attempts, the UE falls back to 4-step random access EDT. FFS on the maximum number of transmission attempts.

Proposal 36	If no successful contention resolution was received in the CB MsgB window after a CB MsgA replica was transmitted, the UE shall draw a random number from a back off (indicated in a CB MsgB, if received, or from back off indicator in system information) ad wait the number of DSA windows and in the DSA window after the wait, do the CB MsgA replica selection and transmission.

Proposal 37	OCCs are not used for the CB EDT functionality.

Proposal 38	An eMTC UE may stop transmitting CB MsgA replicas or CB MsgA repetitions of a replica if the UE receive a CB MsgB with a matching contention resolution identity.

# 4	References

RP-243278, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc., RAN#106, Dec. 2024

R2-2405201, “Efficient delivery of RRCEarlyDataComplete message”, TOYOTA Info Technology Center, RAN2#126, May, 2024.

R2-2405202, “Discussion on EDT enhancements”, ESA, RAN2#126 Fukuoka, May 2024.

R1-2410895, “Reply LS on OCC for CB Msg3 NPUSCH”, From: RAN1, To: RAN2, November 2024

# Appendix A	Background

Here we list some important background information of issues discussed.

## A.1	The 5G QoS model

The 5G QoS model is described in 23.501 section 5.7.

“Standardized or pre-configured 5G QoS characteristics, are indicated through the 5QI value, and are not signalled on any interface, unless certain 5G QoS characteristics are modified as specified in clauses 5.7.3.3, 5.7.3.4, 5.7.3.6, and 5.7.3.7.”

Some of the most important characteristics are the Packet Delay Budget (PDB) and the Packet Error Rate (PER).

Among the standardized 5QIs there is one for NTNs.

### A.1.1	Standardized 5QI for NTNs

The 5QI 10 is for NTN, and the standardized parameters is copied below:

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Non-GBR (NOTE 1) | 90 | 1100ms (NOTE 10,NOTE 13, NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 10: In RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 17: The worst case one way propagation delay for GEO satellite is expected to be ~270ms,~21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically two way propagation delay e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-10 can accommodate the worst case PDB for GEO satellite type. |  |  |  |  |  |  |  |  |

In NOTE 1 above, it is clear delayed packets shall not be included in the PER, further for PDB we have (23.501):

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER.

In the table we see that, for 5QI 10, the PDB is 1100 ms which of 1080 ms is for RAN and 20 ms for CN. Further, an acceptable PER is 10^-6.

This means from an QoS point of view, it is acceptable if 2% or less of the packets experience a delay above 1080 ms in RAN.

In GEO, a delay of 1080 ms is one first transmission and two retransmissions if the UE has UL resources every TTI for a first transmission and there is no scheduling delay, or one first transmission and one retransmission if the UE need to send SR and wait for a grant before the first transmission.

Even if an operator have other QoS requirements than the 5QI 10, there must be an acceptable PDB and acceptable number of packet arriving late, and an acceptable level of packets lost.

In the 5G QoS model, there is an acceptable packet delay budget that 98% of the packets need to fulfill and a there is an acceptable packet loss rate.

There is a standardized 5QI 10 that targets NTNs. It has an RAN PDB of 1080 ms and a PER of 10^-6.

If we assume that the PDB allow for two attempts for each packet, then we must have a fail rate per transmission at 10^-3 or better to achieve a PER of 10^-6 or better if all packets with longer delay are lost. However, we can accept a fail rate per transmission that is higher than 10^-3 if less than 2% of the packets are delayed longer than the PDB (that is, those packets are only delayed and not lost).

Assuming a packet failure for each transmission we can calculate the probability of success after 1 transmission, and probability of success after 2 transmissions and probability that more than two transmissions are needed for success, see table below.

|  | Pn, probability of success after n transmissions |  | P[n>2], probability of more than two transmissions needed |
| --- | --- | --- | --- |
| Probability of packet failure | P1 (n = 1) | P2 (n = 2) | P[n > 2] = 1 – P1 – P2 |
| 0.001 | 0.999 | 0.000999 | 0.000001 |
| 0.01 | 0.99 | 0.0099 | 0.0001 |
| 0.03 | 0.97 | 0.0291 | 0.0009 |
| 0.05 | 0.95 | 0.0475 | 0.0025 |
| 0.1 | 0.9 | 0.09 | 0.01 |
| 0.14 | 0.86 | 0.1204 | 0.0196 |
| 0.1414 | 0.8586 | 0.12140604 | 0.019994 |

If, for example, probability of failure for each transmission attempt is 3%, then the probability that more than two attempts are needed is 9 in ten thousand (0.0009) packet attempts. And for a packet failure probability of 14% the probability to need more than 2 transmission is 1.96%.

Thus, the 5G QoS model (at least 98% of packets within the PDB), when the PDB allows for two transmissions – we can have a probability of failure of each individual transmission attempt of more than 14%.

For 5QI 10, each transmission can have a failure rate of up to 14% without violating the QOS requirements in GEO scenarios.

## A.2	Restriction on selectable DSA resources

There may be reasons for the UEs not being able to freely select transmission opportunities inside a DSA window.

For example:

- UEs not able to transmit on more than one subcarrier (more than one RU) in one TTI

  - Due to UE filter and emission mask issues (more expensive power amplifier), or UE peak power requirements

- UEs not able to transmit on different subcarriers in consecutive TTIs

  - UE may need some time to retune the oscillator

- Selecting two consecutive TTIs would violate the requirement on 40 ms silent period every 256 ms of transmission

Any such limits will spread out the load evenly on the transmission opportunities inand will therefore increase the collision probability.

As a simple example, consider a DSA window with four transmission opportunities where each UE sends two replicas, and compare two cases:

A) there are two subcarriers with two consecutive TTIs. UEs needs to randomly select one of the subcarriers in the first TTI and one of the subcarriers in the second TTI.

B) there is one subcarrier with four consecutive TTIs. The UEs can freely select two of the four TTIs.

We can calculate the probability of collision given a number of UEs, N, that are transmitting in the DSA window:

| Case A | Pn, probability of success for n UEs |  |  |  |  | M, average number of successful UEs | Pc, collision probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N, number of UEs | P0 (n = 0) | P1 (n = 1) | P2 (n = 2) | P3 (n = 3) | P4 (n = 4) | M = 0*P0+1*P1+2*P2+3*P3 | 1 – M/N |
| 2 | 0.25 | 0 | 0.75 | 0 | 0 | 1.5 | 0.25 |
| 3 | 0.0625 | 0.625 | 0.3125 | 0 | 0 | 1.25 | 0.5833 |
| 4 | 0.25 | 0.5625 | 0.1875 | 0 | 0 | 0.9375 | 0.7656 |

| Case B | Pn, probability of success of n UEs |  |  |  |  | Average number of successful UEs, M | Collision probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of UEs, N | n = 0 | n = 1 | n = 2 | n = 3 | n = 4 | M = 0*P0+1*P1+2*P2+3*P3 | 1 – M/N |
| 2 | 0.1667 | 0 | 0,8333 | 0 | 0 | 1.6667 | 0.1667 |
| 3 | 0.1389 | 0.4167 | 0.3333 | 0.1111 | 0 | 1.4167 | 0.5278 |
| 4 | 0.2407 | 0.5370 | 0.2222 | 0 | 0 | 0.9815 | 0.7546 |

Restrictions that limit the UEs ability to freely select transmission opportunities within a DSA window will increase the probability of collision.

This fact will affect the best way to configure resources for the DSA window, it is beneficial to allocate more time resources than frequency resources for the DSA window.

For example, comparing a DSA window with totally 24 possible replica resources and 12 ms DSA window periodicity allocated as a) 6 subcarriers times 4 ms, and b) 2 subcarriers for 12 ms.

If a UE cannot select two resources that are on different subcarrier at the same time, then for a) after selecting a first replica resource, there are 5 excluded resources for selecting the second replica resource while for b) after selecting a first replica resources there are only 1 excluded resource when selecting the second replica resource.

Figure A.1: Example 2 Excluded resources (red) after selecting a first replica (blue).

When configuring resources for the DSA window, it is beneficial to allocate more resources in the time domain and less in frequency domain than vice versa.

## A.3	Legacy NB-IoT resources

Here we go through the most important properties of legacy NB-IoT that will affect the CB EDT functionality.

### A.3.1	NPUSCH resources

The UE can only transmit in subframes that are NB-IoT UL subframes (see 36.213 section 16.5).

There are two uplink subcarrier spacings (SCS) 3.75 kHz (subcarrier 0 to 47 in one PRB) and 15 kHz (subcarrier 0 to 11 in one PRB).

There are two NPUSCH formats, Format 1 for data transmissions and Format 2 for HARQ ACK/NACK transmissions. See 36.211:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Table 10.1.2.1-1: NB-IoT parameters.

| Subcarrier spacing |  |  |
| --- | --- | --- |

The Format 2 is used for HARQ ACK/NACK transmissions. Format 2 is single tone for both 3.75 kHz and 15 kHz SCS and uses 8 ms for 3.75 kHz SCS and 2 ms for 15 kHz SCS without repetitions.

For 3.75 kHz SCS, HARQ ACK/NACK transmission without repetitions use 8 ms on one subcarrier.

For 15 kHz SCS, HARQ ACK/NACK transmission without repetitions use 2 ms on one subcarrier.

For Format 1 3.75 kHz SCS, we can only have a single tone (1 subcarrier) transmissions and these uses 32 ms without repetitions for one RU.

For Format 1 15 kHz SCS, we can have single tone or multi tone (multi subcarriers) transmissions without repetitions for one RU:

|  | RU duration |
| --- | --- |
| 1 | 8 ms |
| 3 | 4 ms |
| 6 | 2 ms |
| 12 | 1 ms |

For 3.75 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 32 ms on one subcarrier.

For 15 kHz SCS, NPUSCH Format 1 transmission on one RU without repetitions use 8 ms for one subcarrier, 4 ms for three subcarriers, 2 ms for six subcarriers and 1 ms for twelve subcarriers.

Each uplink transmission can use one or more RUs where each RUs follow contiguous in time. Higher nrof RUs enables higher transport block sizes. Further, each transmission can have repetitions. From 36.213:

Table 16.5.1.1-2: Number of resource units () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Increasing number of RUs can increase the UL transport block size.

The number of subframes for one NPUSCH transmission increase if more RUs or more repetitions are used.

To further complicate things when repetitions are used, there are also the uplink transmission gaps. From 36.211:

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

Thus, after 256 ms of NPUSCH transmission the UE must be silent for 40 ms – this will affect all NPUSCH transmission that are longer than 256 ms.

For example, a 1 RU, 3.75 kHz SCS uses 32 ms for one transmission and if 128 repetitions are used (the maximum) one transmission will use 32*128 ms = 16*256 ms for the transmissions but every 256 ms there is a 40 ms gap, thus the total transmission length is 16*256 + 15*40 ms.

Some of the contiguous subframes cannot be used for NPUSCH transmissions.

The NB-IoT UEs cannot transmit NPUSCH continuously for more than 256 ms before inserting a 40 ms silent period of postponed NPUSCH transmission subframes.

### A.3.2	NPDSCH resources and ACK/NACK resources

For downlink there is only 15 kHz SCS and each NPDSCH transmissions uses all the 12 subcarriers.

The NPDSCH transmissions uses 15 kHz SCS and 12 subcarriers.

The UE can only transmit in subframes that are NB-IoT DL subframes (see 36.213 section 16.4), some of the excluded subframes are NPBCH (1 ms every 10 ms), NPSS (1 ms every 10 ms), NSSS (1 ms every 20 ms), and SIB1-NB (scheduled in MIB on the NPBCH). Further gaps may be configured (36.211 10.2.3.4).

Not all DL subframes are available for NPDSCH transmission, for example subframes with NPBCH, NPSS, NSSS, SIB1-NB and configured gaps are excluded.

Each downlink transmission use one or more subframes contiguous in time. Higher nrof subframes enables higher transport block sizes. Further, each transmission can have a number of repetitions. From 36.213:

Table 16.4.1.3-1: Number of subframes () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

Table 16.4.1.5.1-1: Transport block size (TBS) table.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Increasing number of subframes can increase the DL transport block size.

The number of subframes for one NPDSCH transmission increase if more subframes or more repetitions are used.

There is a hard coded gap of four subframes between last NPDCCH subframe and the first (possible) NPDSCH subframe. To further complicate things, some of the subframes are not used for NPDSCH and are excluded. From 36.213

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N consecutive NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages or scheduling gap (if any) or processing gap (if any) where, n0<n1<…,nN-1 ,

The hard coded gap can be increased by k0 signalled in the DCI, from 36.213:

Table 16.4.1-1: for DCI format N1.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 4 | 16 |
| 2 | 8 | 32 |
| 3 | 12 | 64 |
| 4 | 16 | 128 |
| 5 | 32 | 256 |
| 6 | 64 | 512 |
| 7 | 128 | 1024 |

Table 16.4.1-1a: for DCI format N1 with DCI CRC scrambled by G-RNTI.

| 0 | 0 |
| --- | --- |
| 1 | 4 |
| 2 | 8 |
| 3 | 12 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

Some of the contiguous subframes cannot be used for NPDSCH transmissions.

#### A.3.2.1	UE procedure for reporting ACK/NACK

The legacy NPDSCH transmissions are acknowledged on resources that are indicated using 4 bits in the DCI that scheduled the NPDSCH.

The resource for HARQ ACK/NACK is indicated with 4 bits in the DCI.

From 36.213 (where k0 – 1 is the nrof subframes between last NPDSCH subframe and the first NPUSCH Format 2 RU carrying HARQ ACK/NACK for a UE):

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	 DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12 subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

…

Table 16.4.2-1: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 38 | 13 |
| 1 | 39 | 13 |
| 2 | 40 | 13 |
| 3 | 41 | 13 |
| 4 | 42 | 13 |
| 5 | 43 | 13 |
| 6 | 44 | 13 |
| 7 | 45 | 13 |
| 8 | 38 | 21 |
| 9 | 39 | 21 |
| 10 | 40 | 21 |
| 11 | 41 | 21 |
| 12 | 42 | 21 |
| 13 | 43 | 21 |
| 14 | 44 | 21 |
| 15 | 45 | 21 |

Table 16.4.2-2: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 0 | 13 |
| 1 | 1 | 13 |
| 2 | 2 | 13 |
| 3 | 3 | 13 |
| 4 | 0 | 15 |
| 5 | 1 | 15 |
| 6 | 2 | 15 |
| 7 | 3 | 15 |
| 8 | 0 | 17 |
| 9 | 1 | 17 |
| 10 | 2 | 17 |
| 11 | 3 | 17 |
| 12 | 0 | 18 |
| 13 | 1 | 18 |
| 14 | 2 | 18 |
| 15 | 3 | 18 |

There is a gap of at least 12 subframes between the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK. Further, looking into the possible allocations we note that:

For 3.75 kHz SCS, HARQ ACK resources can be on one of 8 different subcarriers and use one of 2 different scheduling delays.

For 15 kHz SCS, HARQ ACK resources can be on one of 4 different subcarriers and use one of 4 different scheduling delays.

There is a gap of at least 12 subframes between the the last NPDSCH subframe and the first NPUSCH subframe carrying the HARQ ACK/NACK.

### A.3.3	NPDCCH resources

NB-IoT have possibility of having one or two NPDCCH in one subframe, 36.211:

One or two NPDCCHs can be transmitted in a subframe.

Table 10.2.5.1-1: Supported NPDCCH formats

| NPDCCH format | Number of NCCEs |
| --- | --- |
| - 0 | - 1 |
| - 1 | - 2 |

Further NPDCCH can also be configured with a number of repetitions, 36.211:

With Format 1, each NCCE uses 6 subcarriers, and Format 0 uses all the 12 DL subcarriers.

NPDCCH uses 6 or 12 subcarriers in one subframe (without repetitions) can carry two respectively one DCI.

If coverage is challenging, we may assume only NPDCCH format 1 with one DCI can be transmitted in each subframe when no repetitions are used.

There are limits to the timing of when the NPDCCH can earliest occur after NPUSCH transmission (which can be Format 1 with data or Format 2 with ACK/NACK), 36.213 section 16.6:

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-  and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

- if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3.

otherwise,

- If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3.

There are also limits to the timing of when the NPDCCH can earliest occur after NPDSCH transmission that do not require NPUSCH Format 2 for ACK/NACK, 36.213 section 16.6:

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

There is a guard period of at least 3 subframes after an NPUSCH transmission (Format 1 or 2) and first possible NPDCCH reception.

There is a guard period of at least 12 subframes after an NPDSCH reception and the first possible NPDCCH reception.

Further, all excluded subframes for NPDSCH are also excluded for NPDCCH.

Some of the contiguous subframes cannot be used for NPDCCH transmissions.

# Appendix B	RAN2 agreements

## B.1	RAN2#125bis

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

## B.2	RAN2#126

1 RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

## B.3	RAN2#127

Agreements:

1.	RAN2 will continue working on a CB Msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

## B.4	RAN2#127bis

In evaluation of possible methods to improve UL capacity for CB Msg3 ETD-like transmission we focus on a working point where the PLR is lower than 10% (this does not mean that we are mandating any network to work at any working point)

We use the term CB Msg3 EDT for now

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB Msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB Msg3 assuming that OCC2 might also apply to CB Msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB Msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB Msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB Msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB Msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB Msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB Msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

## B.4	RAN2#128

Agreements:

1.	Only system Information is used to provide cell-specific CB Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB Msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB Msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB Msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB Msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB Msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB Msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)