3GPP TSG-RAN WG2 Meeting #129	R2-2501363

Athens, Greece, Feb. 17th – 21st, 2025

Title:	LS on segmented UE capabilities

Response to:

Release:	Rel-16

Work Item:	RACS-RAN-Core, TEI17

Source:	RAN2

To:	CT4

Cc:

Contact person:	Lian Araujo

lian.araujo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 specified in TS 38.331 and TS 36.331 features where the UE can send segmented Radio Access UE capabilities in RRC level in two ways:

- Upon network request, the UE may send segmented UE capabilities, up to 16 segments;

- Upon network request, the UE may send segmented UE capabilities, up to a limit of segments provided by the network (the limit ranges from 2 to 16);

Considering that one segment can be as large as the maximum PDCP SDU size (8188 bytes in LTE, 9000 bytes in NR), RAN2 understands that the UE capability size (e.g. with 8 or more segments) may be larger than possible to transfer in N26 interface, which would have a limit around 64 kB. RAN2 thinks that in case the network can limit the amount of segments provided (the second bullet listed above), the network may limit the number of segments to 7 to avoid exceeding the N26 interface size limit. However, this solution requires that the feature on network-controlled number of segments is supported by both UE and network.

RAN2 kindly asks CT4 to consider the above scenario and assess the need to address it in CT4.

# 2	Actions

To CT4 group

ACTION: 	RAN2 respectfully asks CT4 to take the above into account and provide feedback if any.

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #129bis             07 - 11 April 2025	                             Wuhan, CN

TSG-RAN2 Meeting #130	19 - 23 May 2025	Malta, MT